3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2109179

# Elbonia, November 15 – 22, 2021	(revision of S2-2108500r06)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0029 | rev | 4 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on cases that SMF rejects UE join request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1.It is specified in clause 7.2.1.3 that “If no MB-SMF is assigned for the MBS session ID, the SMF may select an MB-SMF and request it configure the multicast session or the SMF may reject the join request.” It is clarified that in that case SMF select the MB-SMF based on local configuration. 2. AMF’s MB-SMF discovery scenario is missing when the shared tunnel for shared delivery is established. NOTE: Changes on top of S2-2108007 are highlighted with colours. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - If no MB-SMF can be assigned to the MBS session ID (according to the MB-SMF profile registered in configuration procedure or preconfigured by the operator), the NRF does not provide MB-SMF profile to - When the shared tunnel for shared delivery is established, AMF may discover the serving MB-SMF based on MBS session ID and optionally Area Session ID which are received from NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scenario that “If no MB-SMF is assigned for the MBS session ID” for preconfigured MBS session is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.146: "Multimedia Broadcast/Multicast Service (MBMS); Stage 1".

[3]	3GPP TS 22.246: "Multimedia Broadcast/Multicast Service (MBMS) user services; Stage 1".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[6]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[7]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[8]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[9]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[10]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[11]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[12]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[13]	3GPP TS 26.346: "MBMS: Protocols and Codecs".

[14]	3GPP TR 23.757: "Study on architectural enhancements for 5G multicast-broadcast services".

[15]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[16]	3GPP TS 38.401: "NG-RAN; Architecture description".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[18]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[x]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

# * * * * Second change * * * *

### 7.1.2	MB-SMF discovery and selection for multicast/broadcast session

To facilitate the MB-SMF discovery/selection for one multicast/broadcast MBS session, the following mechanism is used:

-	The MB-SMF registers its capability related to multicast/broadcast session management (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s), TMGI range, service area) as part of its profile to the NRF by invoking Nnrf_NFManagement_NFRegister. In addition, when an multicast session is configured, and the MBS session ID is not yet included in the MB-SMF profile, the MB-SMF updates its profile towards the NRF with the MB Session ID (i.e. TMGI or source specific IP multicast address).

NOTE:	The operator can preconfigure MB-SMF for specific source IP multicast address range or TMGI range.

-	When the UE joins the multicast MBS session via PDU session modification procedures, the SMF serving the PDU session invokes the Nnrf_NFDiscovery_Request Request including the MBS session ID for multicast provided by the UE and optionally other information, i.e. the S-NSSAI and the associated NSI ID (if available), DNN, etc., to query the NRF for MB-SMF information. Based on the MBS session ID and other information for query, the NRF decides whether an MB-SMF serving the MBS session exists (i.e. the NRF decides whether the requested MBS Session ID is in the profile of an MB-SMF, as defined in clause 7.1.1.2 or 7.1.1.3 or preconfigured by the operator). If so, the NRF provides in Nnrf_NFDiscovery_Request Response the information of the MB-SMF currently serving the MBS session. The SMF serving the PDU session selects the MB-SMF currently serving the multicast MBS session, based on the MB-SMF information provided by the NRF. For local MBS services, the SMF takes MB-SMF service area, UE location into account when selecting the MB-SMF. If no MB-SMF serving the multicast session exists, the NRF does not provide MB-SMF profiles in the response message to the SMF as defined in TS 29.510 [x].

-	When the shared tunnel for shared delivery is established as described in clause 7.2.1.4, the AMF may invoke the Nnrf_NFDiscovery_Request Request including MBS Session ID and Area Session ID (if received from the NG-RAN), to query the NRF for the MB-SMF information. In this case, based on the MBS session ID and Area Session ID, the NRF provides in Nnrf_NFDiscovery_Request Response the MB-SMF information currently serving the MBS session and the Area Session ID.

-	When the multicast MBS session context is deleted from the MB-SMF, e.g. due to MBS session release, the MB-SMF updates its profile towards the NRF, i.e., removing the MBS session ID which is no longer served by the MB-SMF.

-	During MBS session information provisioning procedures defined in clause 7.1.1.2, unless the MB-SMF information is available by other means, e.g. locally configured in the NEF/MBSF/AF, the NEF/MBSF/AF queries the NRF with information of the multicast/broadcast session (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s)), and selects the MB-SMF(s) based on the MB-SMF information provided by the NRF. For local MBS services, the NEF/MBSF/AF takes MB-SMF service area into account when selecting the MB-SMF(s).

# * * * * End of changes * * * *