3GPP TSG-WG SA2 Meeting #148E 	S2-2109308

# Electronic, November 15 – 19, 2021										(revision of S2-2108917r01)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3235 | rev | 4 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling of paging reception for 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Sony, Ericsson, Apple, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MUSIM |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This is a TS 23.501 counterpart to TS 23.502 CR2920R3 (SP-211131). Adding the feature to enable paging reception when the UE detects Paging Occasion (PO) collisions. Rev3: Contrary to the other MUSIM features, the Paging Timing Collision Control is based on the existing Mobility Registration Update procedure and is not indicated by the UE to the network as a distinct feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Upon detection of Paging Occasion (PO) collisions, the MUSIM UE uses the Mobility Registration procedure to trigger a new 5G-GUTI assignment. Rev3: Added text to clarify that the Paging Timing Collision Control being based on the existing Mobility Registration Update procedure, the UE does not indicate support for this feature to the network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing MUSIM functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.38.1; 5.38.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** FIRST CHANGE *****

### 5.38.1	General

A network and a UE may support one or more of the following features for Multi-USIM UE operation:

-	Connection Release as described in clause 5.38.2;

-	Paging Cause Indication for Voice Service, as described in clause 5.38.3;

-	Reject Paging Request, as described in clause 5.38.4;

-	Paging Restriction, as described in clause 5.38.5.

-	Paging Timing Collision Control, as described in clause 5.38.x.

In the Registration procedure (as specified in clause 4.2.2.2.2), when a Multi-USIM UE has more than one USIM active, supports and intends to use one or more Multi-USIM specific features, it indicates to the AMF the corresponding Multi-USIM feature(s) are supported (except for the Paging Timing Collision Control feature). Based on the received indication of the supported Multi-USIM features from the UE, the AMF shall indicate to the UE the support of the Multi-USIM features based on the Multi-USIM features supported by network and any preference policy by the network, if available. When a UE returns to having only one USIM active from a Multi-USIM UE that previously indicated to the network it supported Multi-USIM feature(s), the UE shall indicate all the Multi-USIM features are not supported to the network for that USIM. The AMF shall only indicate the support of Paging Restriction feature together with the support of either Connection Release feature or Reject Paging Request feature.

The Multi-USIM UE includes the support of individual features for Connection Release, Paging Cause Indication for Voice Service, Reject Paging Request and Paging Restriction as specified in clause 5.4.4a.

NOTE:	The Paging Timing Collision Control feature being based on the Mobility Registration Update, and it doesn’t require capability exchange between the UE and network.

A Multi-USIM UE shall use a separate PEI for each USIM when it registers to the network.

***** NEXT CHANGE *****

### 5.38.x	Paging Timing Collision Control

To avoid possible paging occasion collision and to enhance the likelihood that paging is received successfully for different USIMs, a Multi-USIM UE may need a new 5G-GUTI to modify the timing of the Paging Occasions (POs) for a USIM when the USIM’s registration is not emergency registration. When a Multi-USIM UE needs a 5G-GUTI assignment, it performs a Mobility Registration Update without any specific indication (i.e. it is using a normal Registration procedure). This triggers the AMF to allocate a new 5G-GUTI and provide it to the Multi-USIM UE in the Registration Accept message.

NOTE:	It is recommended to avoid excessive signalling load from UE due to this procedure.

***** END OF CHANGES *****