3GPP TSG-WG SA2 Meeting #148E e-meeting 	S2-210xxxx

Elbonia, 15-22 November 2021

Title:	[Draft] Reply LS on EPS requirements for ID_UAS

Response to:	C2-217131/S2-2109369

Release:	Release 17

Work Item:	ID_UAS

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Stefano Faccin

Tel. Number:

E-mail Address:	sfaccin AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 would like to thank CT1 for their LS.

Regarding CT1 questions:

CT1 Question 1): Whether UUAA-SM and C2 authorization need to be supported in deployments where MMEs/S-GWs are of previous releases (prior to release-17) with the limitations mentioned above.

SA2 answer:  no, there is no need to support this prior to release 17.

CT1 Question 2): If the answer to the Question 1 is No, whether the UAV UE due to the UE mobility may end-up being served by a target MME, or target S-GW or both, of a previous release (prior to release-17) with the limitations mentioned above, and if so, how to handle this mobility situation for ID_UAS.

SA2 answer:  Yes, the UAV UE due to the UE mobility may end-up being served by a target MME, or target S-GW or both, of a previous release (prior to release-17) with the limitations mentioned above A network may also make use of DÉCOR feature to ensure that the UE is always served by a proper MME/S-GW that supports ePCO.  If DÉCOR does not apply and a UAV, previously authenticated and authorized, becomes served by an MME or S-GW of a previous release (prior to release-17) with the limitations mentioned by CT1 LS, then any potential future UUAA procedure for re-authentication or re-authorization cannot be performed, and the USS may end up revoking authorization for the UAV.

SA2 sees no need for any enhancements to Whether any protocol changes are needed to support scenarios where a UAV is served by an MME that does not support ePCO is a stage 3 decision. As an example, a network may make use of DÉCOR feature to ensure that the UE is always served by a proper MME/S-GW that supports ePCO.

2. Actions:

To CT1:

ACTION: 	SA2 kindly ask CT1 to keep the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #149E		14-25 February 2022	Elbonia

TSG-SA2 Meeting #150		4-8 April 2022	TBD