SA WG2 Meeting #148E e-meeting	S2-2108645

# 15-22 November 2021, Elbonia	(was S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3412 | rev | - | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for SSC modes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2021-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | LS from CT6 (C6-210258) asks SA2 modify 3GPP TS 23.501 to explicitly state which SSC modes have to be supported i.e. which ones are optional or mandatory. From URSP point of view based on the existing stage 3 in TS 24.526, all of them are currently optional for the UE to support since the UE skips an RSD which includes an SSC mode that the UE does not support. The following is stated: ii) the selected route selection descriptor includes a PDU session type or an SSC mode which is not supported by the UE, the UE shall proceed to step 4); (…) 4) if there is any route selection descriptor which has not yet been evaluated, the UE shall proceed to step 1). If all route selection descriptors for the matching non-default URSP rule have been evaluated and there is one or more non-default matching URSP rule which has not yet been evaluated, the UE shall proceed to step a). If all non-default matching URSP rules have been evaluated, the UE shall inform the upper layers of the failure. Given though the PCF/UDR are not aware in advance of the UE’s capabilities in terms of SSC mode support, if none of the SSC modes are mandatory this may lead to interoperablility issues, if for example the SSCMSP does not include an RSD with at least one SSC mode the UE supports. SSC mode 1 is the one used in real life implementations so far and requires that the UPF acting as PDU Session Anchor is maintained regardless of the access technology. SSC mode 1 is the equivalent of what is used in legacy systems (EPS,GPRS) with IP address preservation so every “mainstream” implementation is expected to support it. SSC modes 2 and 3 refer to some special deployment cases where the PDU session is released and re-established and therefore requires cooperation from UE and NW to support and also possibly the Application and upper layer stack in the UE to provide certain functionality. SSC mode 3 does not work with ethernet PDU sessions. It is therefore proposed that is clarified in TS 23.501 that SSC mode 1 is mandatory for the UE to support. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies that SSC mode 1 is mandatory for the UE to support |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability issue if the UE cannot support any of the SSC modes the network supports |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.9.2.1, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

##### 5.6.9.2.1	SSC Mode 1

For a PDU Session of SSC mode 1, the UPF acting as PDU Session Anchor at the establishment of the PDU Session is maintained regardless of the access technology (e.g. Access Type and cells) a UE is successively using to access the network.

In the case of a PDU Session of IPv4 or IPv6 or IPv4v6 type, IP continuity is supported regardless of UE mobility events.

In this Release of the specification, when IPv6 multihoming or UL CL applies to a PDU Session of in SSC mode 1, and the network allocates (based on local policies) additional PDU Session Anchors to such a PDU Session, these additional PDU Session Anchors may be released or allocated, and the UE does not expect that the additional IPv6 prefix is maintained during the lifetime of PDU Session.

SSC mode 1 may apply to any PDU Session type and to any access type and is mandatory for the UE to support.