- SA WG2 Meeting #S2-148E	S2-2108662r01

- 15 - 22 November, 2021, Electronic meeting

- Source:	vivo Mobile Communications Ltd, China Telecom, China Unicom, CATT, Spreadtrum Communications

- Title:	New SID on UE Aggregation for Industry with Multi-connectivity

- Document for:	Approval

Agenda Item:	9.1.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

- Title: 	Study on UE Aggregation for Industry with Multi-connectivity

- Acronym: FS_UE_Aggregation

- Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-18

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

- 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  | X |

- 2	Classification of the Work Item and linked work items

- 2.1	Primary classification

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

- 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 740005 | 5G System - Phase 1 | Architecture of 5GS (23.501) and policy control for 5GS (23.503) |

- 3	Justification

One scenario that has been discussed in RWS-210172 (vivo), RWS-210355 (CMCC), RWS-210451 (Huawei), RWS-210479 (ZTE), RWS-210422 (interdigital), RWS-210192 (FGI), and RWS-210199 (Rakuten) is using multiple UEs for uplink aggregation of one traffic flow transmission,.

Another scenario that has been discussed in RWS-210172 (vivo), RWS-210060 (Spreadtrum), RWS-210345 (CMCC), RWS-210346 (CMCC), RWS-210454 (Huawei), RWS-210229 (LG), RWS-210422 (interdigital), and RWS-210479 (ZTE) is that, in order to archive reliability with more resource efficiency, some UEs act as backup of other UEs, who are transmitting traffic, without radio resource consumption, and switch the traffic path very quickly when one or more UEs that are transmitting traffic fail. Similar as handling of failure case for uplink aggregation of one traffic flow transmission, this kind of reliability requires ultra-fast switch when failure occurs, which is not able to be archived by using application-level solutions.

This study focuses on the CN based solution if it is sufficient to meet the requirement.

New uses cases and deployment scenarios were identified and should be studied where it  is desired to distribute the traffic across other types of access paths from a single device that supports two UEs:

-	Two 3GPP access paths in two different PLMNs, e.g., one path using 3GPP NTN access via PLMN 1 and another path using 3GPP TN access via PLMN 2;

-	Two 3GPP access paths in the same PLMN, e.g., one path using LTE/EPC and another path using NR/5GC.

## 4	Objective

The objectives of this SA2 study is to investigate how the 5GC can be enhanced to support UE aggregation for uplink aggregation and fast switching:

- (WT#1) Support of management and authorization for UE aggregation.

- (WT#2) Support of policy enforcement for binding multiple UEs, e.g., load balance, standby, etc.;

- (WT#3) When redundancy for an application session is supported with two UE(s), support of status report for fast switching to UE backup when connection with a UE fails;

-	(WT#4) Study how a device connection can support more types of 3GPP access paths, as defined below.

4.21	Study how a device connection can support two 3GPP access paths in different PLMNs provided by two different UEs. The following cases will be considered:

- The device connection has one 3GPP access path via PLMN-2 and another 3GPP access path via PLMN-1/SNPN, and the device registers with PLMN-2 and with PLMN-1/SNPN using different 3GPP credentials.

- The device connection has one 3GPP access path via PLMN-2 and another 3GPP access path via PLMN-1/SNPN, and the UE registers with PLMN-2 and with PLMN-1/SNPN using the same 3GPP credentials.

How the device selects the different networks to register with, will not be considered.

4.2	Study how a device connection can support two 3GPP access paths in the same PLMN. The following case will be considered:

a)	The device connection has one 3GPP access path in a PLMN using LTE/EPC and another 3GPP access path in the same PLMN using NR/5GC.

The study will be based on the existing interworking architecture, so (i) the UE is simultaneously registered with EPC and 5GC in the PLMN using the existing dual-registration functionality, as per TS 23.501 without the need to support N26, (ii) the PDU sessions are terminated in a common PGW-U/UPF, and (iii) a common PGW-C/SMF is used.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 1 | 0.5 | No | Self-contained |
| WT#2 | 1 | 0.25 | No | Self-contained |
| WT#3 | 0.5 | 0.25 | No | Self-contained |
| WT#4 | 5 | 4.5 | No | Self-contained |
| WT#4.1 | 4 | 4 | No | Self-contained |
| WT#4.2 | 1 | 0.5 | No | Self-contained |

Total TU estimates for the study phase: 9.5;

Total TU estimates for the normative phase: 4.5;

Total TU estimates: 14.

- 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | "23.XXX" | Study on architecture enhancements for UE Aggregation | SA#96 (Jun22) | SA#97 (Sept 22) | zhenhua.xie@vivo.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

## 6	Work item Rapporteur(s)

## Zhenhua Xie, vivo Mobile Communications Ltd, zhenhua.xie@vivo.com

- 7	Work item leadership

SA2

- 8	Aspects that involve other WGs

SA3 for the Security aspects, SA5 for the Charging aspects, RAN WG2 for the RAN related issues.

- 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

vivo Mobile Communications Ltd

China Telecom

China Unicom

CATT

Spreadtrum Communications