3GPP TSG-WG SA2 Meeting #147E e-meeting 	S2-2107088

Elbonia, 18 – 22 October 2021

Title:	[Draft] LS Reply on rejected NSSAI for the maximum number of UE reached when TAIs belonging to different PLMNs

Response to:	S2-2107020 / LS on rejected NSSAI for the maximum number of UE reached when TAIs belonging to different PLMNs

Release:	Release 17

Work Item:	eNS_Ph2

Source:	SA2

To:	CT1

Cc:	XX

Contact Person:

Name:	Stefano Faccin

Tel. Number:

E-mail Address:	sfaccin AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 would like to thank CT1 for the questions on the applicability of the rejected NSSAI due to the maximum number of UEs.

The applicability of the quota for a specific serving PLMN is specific to the serving PLMN, and does not apply to other PLMNs, unless a set of PLMNs share the CN (like it is common when the RA includes TAIs of different PLMN that also are marked as equivalent PLMNs). Therefore,

1) if the UE receives a cause code indicating the rejection of a S-NSSAI due to NSAC, and the RA includes TAIs of various PLMNs and these are marked as equivalent PLMNs, then the UE can assume this rejection is valid in the RA and the TAIs of all these PLMNs, so by definition this should imply the limit applies to all these PLMNs in the equivalent PLMN list.

2) if the UE receives a cause code indicating the rejection of a S-NSSAI due to NSAC, and the RA includes TAIs only of the R-PLMN, and there are equivalent PLMNs in the registration accept, then the UE can assume this rejection is valid in the RA but not outside the RA, ie. the limitation is not necessarily applying to the equivalent PLMNs in the list.

2. Actions:

To CT1.

ACTION: 	SA2 kindly ask CT1 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #148E		15 – 19 November 2021	Elbonia

TSG-SA2 Meeting #149E		TBD		Elbonia