SA WG2 Meeting #148e	S2-2108311r01

# Nov 15th – Nov 22th, 2021; Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0035 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on enabling EAS IP Replacement procedure by AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-11-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | EAS Relocation may be triggered by an AF request (e.g. due to the load balance between EAS instances in the EHE) or by the network (e.g. due to the UE mobility). And two triggers have different mechanisms in the EAS IP replacement procedure, the procedure triggered by AF is that AF selects target EAS according to AF internal decision, e.g., load balance, and the procedure triggered by network is that AF selects target EAS according to target DNAI provided by SMF. However the enabling EAS IP Replacement procedure by AF in the clause 6.3.3.1.1 mixes two triggers together. To make the procedure clearer, the Step4 and Step5 of Figure 6.3.3.1.1-1 Enabling EAS IP replacement procedure by AF should be changed to two options. Some description of Step4 and Step5 is corrected to be consistence with Figure 6.3.3.1.1-1 corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changing the Step4 and Step5 of Figure 6.3.3.1.1-1, i.e. Step4 ‘AF Influence procedure (SMF reconfigures UL CL and Local PSA) or UE Mobility ' is changed to Step4a ‘AF triggering procedure (SMF reconfigures UL CL and Local PSA) ' and Step5 ‘Early/Late Notification with enhancement described in clause 6.3.3.2.2’ is changed to Step4b ‘Due to UE Mobolity, Early/Late Notification with enhancement triggered’. - Some description is corrected to be consistence with Figure 6.3.3.1.1-1 corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unclear description of the procedure can be confusing in clause 6.3.3.1.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.1.1 |  |  |  |  |  |  |  |  |
| ; |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

##### 6.3.3.1.1	Enabling EAS IP Replacement Procedure by AF

Figure 6.3.3.1.1-1: Enabling EAS IP replacement procedure by AF

NOTE 1:	This procedure covers the scenarios that the UE moves from non-EC to EC or the AF decides to enable the EAS IP replacement in the middle of a session.

1.	UE requests to establish a PDU Session.

2.	UE is preconfigured with the Source EAS IP address or discovers the IP address of the application server for the service subject to Edge Computing and the Source EAS IP address is returned to the UE via EAS Discovery procedure as described in clause 6.2.

3.	UE communicates with the Source EAS.

4a.	EAS Relocation may be triggered by an AF request (e.g. due to the load balance between EAS instances in the EHE). When AF detects that the EAS is capable of runtime context mirroring and an optimal EAS is found, then AF decides to influence the traffic routing in 5GC. The EAS IP replacement information (i.e. source EAS IP address and port number, target EAS IP address and port number) is sent to the SMF within the AF Influence information and the SMF reconfigures the UL CL UPF for local traffic routing and Local PSA with EAS IP replacement information.

UL CL is configured by SMF to forward UL packet to Local PSA if the destination IP address is the Source EAS IP address.

Local PSA is configured by SMF to enforce the "Outer Header Creation" and "Outer Header Removal" as described in step 5. FARs "Outer Header Creation" and "Outer Header Removal" are reused for such an instruction from SMF to UPF.

Detailed enhancement to the AF Influence procedure is described in clause 6.3.3.2.

If a new Local PSA is selected by SMF, the SMF may configure the new Local PSA to buffer the uplink traffic per clause 6.3.5 and enforce the "Outer Header Creation" and "Outer Header Removal" as described in step 6.

If AF is not notified by 5GC that the 5GC supports EAS IP replacement mechanism, the AF does not include the target EAS identifier and does not initiate the EAS relocation.

4b.	EAS Relocation may be also triggered by 5GC (e.g. due to UE Mobility). When Early/Late Notification procedure with enhancement described in clause 6.3.3.2 is triggered, the SMF notifies AF about the target DNAI and may provide the capability of supporting EAS IP replacement in 5GC. Based on the target DNAI, the AF selects a proper target EAS, then the AF triggers to mirror the runtime context between Source EAS and Target EAS. Once the Target EAS is ready, AF responds to SMF about the EAS IP replacement information. During the addition or change of UL CL and Local PSA as described in clause 4.3.5.4, 4.3.5.6 or 4.3.5.7 of TS 23.502 [3], SMF may (re)configure Local PSA for EAS IP address replacement between Source EAS and Target EAS.

5	Local PSA starts to perform "Outer Header Creation" and "Outer Header Removal" FARs as instructed by SMF, which results in EAS IP address replacement:

-	For UL traffic, the destination IP address and port number are replaced with the Target EAS IP address and port number;

-	For DL traffic, the source IP address and port number are replaced back with the Source EAS IP address and port number.

NOTE 2:	In this solution, the PSA UPF need not to understand the logic of EAS IP replacement.

Then all subsequent uplink traffic of this EC service for this UE is forwarded to the target EAS.

NOTE 3:	AF decides when and how to stop the Source EAS from serving the UE based on its local configuration.

END OF CHANGE