3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108331r04

# Elbonia, November 15 – 19, 2021	(revision of S2-2107112)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3239 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Session Continuity for EPC 5GC IWK, and inter-AMF Mobility. |
| --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In interworking between 5GC and EPC when a UE initiates in EPC a home routed PDN connection, and uses a network slice subject to interworking and NSAC, the SMF + PGW-C is allocated in the HPLMN. When the UE moves to 5GS and registers in 5GS, given that the UE is already counted when it attached in EPC, the AMF must ensure that the UE registers successfully regardles of congestion (maximum quota reached for NSAC) during the interworking of the network slice in 5GS. Hence there is a need to ensure that even if the quota in 5GC has been reached the AMF registers the roaming UE successfully in order to maintain session continuity for home routed PDN connections. The above applies while the UE is roaming or at home. In addition to the above in case of inter-AMF mobility in 5GC, there is a need to ensure session continuity when there is inter-mobility between AMFs in 5G for network slices that are requested/allowed in the old AMF and supported, requested and allowed in the new AMF and that are subject to NSAC when the new AMF is in a new service area served by a different NSACF (within different PLMNs or within the same PLMN) than the old AMF. In this case the NSACF used by the new AMF ignores congestion (i.e. does not reject the request if maximum quota reached) for these network slices ONLY. This will not apply to newly requested network slices in the new AMF not requested or supported in the old AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UDM is configured with additional information for subscribed S-NSSAIs per VPLMN that indicates if the subscribed S-NSSAI is subject to both EPC interworking and NSAC in EPC. This enables the AMF to indicate that such an S-NSSAI is to be guaranteed session continuity at handover hence the NSACF ignores if the maximum number of UE has been reached and return success for suhc an S-NSSAI. For a S-NSSAI handed over from 5G and suject to NSAC in EPC, the SMF+ PGW-C includes a session continuity indication in the request to the NSACF they interact with so the NSACF ignores if the maximum number of UEs has been reached. Same prinicple applies during inter-AMF mobility, when the new AMF includes a session contiuity inidcation for the list of registsred and allowed S-NSSAI in the old AMF allowing the NSACF to ignore if the maximum number of UEs has been reached. This approach ensure accurate accounging in the access domain where the UE is located. This CRis related to the 23.502 CR on the same topic |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 5.15.11.5	Support of Network Slice Admission Control and Interworking with EPC

If EPS counting is required for a network slice, the Network Slice Admission Control for maximum number of UEs and/or for maximum number of PDU Sessions per network slice is performed at the time of PDN connection establishment in case of EPC interworking. To support the NSAC for maximum number of UEs and/or for maximum number of PDU Sessions per network slice in EPC, the SMF+PGW-C is configured with the information indicating which network slice is subject to NSAC. During PDN connection establishment in EPC, the SMF+PGW-C selects an S-NSSAI associated with the PDN connection as described in clause 5.15.7.1. If the selected S-NSSAI by the SMF+PGW-C is subject to the NSAC, the SMF+PGW-C triggers interaction with NSACF to check the availability of the network slice by invoking separate NSAC procedures for number of UE and number of PDU Session (as described in clause 4.11.5.9 of TS 23.502 [3]), before the SMF+PGW-C provides the selected S-NSSAI to the UE. If the network slice is available, the SMF+PGW-C continues to proceed with the PDN connection establishment procedure.

The NSACF performs the following for checking network slice availability prior to returning a response to the SMF+PGW-C:

-	For NSAC for number of UEs, if the UE identity is already included in the list of UE IDs registered with a network slice, or the UE identity is not included in the list of UE IDs registered with a network slice and the current number of UE registration did not reach the maximum number, the NSACF responds to the SMF+PGW-C with the information that the network slice is available. The NSACF includes the UE identity in the list of UE IDs if not already on the list and increases the current number of UE registration. Otherwise, the NSACF returns a response indicating that the maximum number with the network slice has been reached.

-	For NSAC for number of PDU Sessions, if the current number of PDU sessions is below the maximum number, the NSACF responds to the SMF+PGW-C with the information that the network slice is available. The NSACF increases the current number of PDU sessions. Otherwise, the NSACF returns the response indicating that the maximum number with the network slice has been reached.

If the maximum number of UEs and/or the maximum number of PDU sessions has already been reached, unless operator policy implements a different action, the SMF+PGW-C rejects the PDN connection.

NOTE 1:	As an implementation option, if the APN is mapped to more than one S-NSSAI and the first selected S-NSSAI is not available (e.g. either current number of UE registration reached maximum or current number of PDU sessions reached maximum), then based on the operator policy the PGW-C+SMF can try another mapped S-NSSAI for the PDN connection establishment procedure.

If the establishment of a new PDN Connections is with a different SMF+PGW-C from the SMF+PGW-C used for already existing PDN connection associated with the same S-NSSAI, each SMF+PGW-C will send a request for update (e.g. increase or decrease) to the NSACF. The NSACF may maintain a registration entry per SMF+PGW-C for the same UE ID.

The SMF+PGW-C provides the Access Type to the NSACF when triggering a request to increase or decrease the number of UEs and/or the number of PDU Sessions for an S-NSSAI.

NOTE 2:	The SMF+PGW-C determines the Access Type based on the RAT type parameter in the PMIP or GTP message received from the ePDG; or alternatively it can internally determine the Access Type based on the source node (e.g. SGW) sending the request for the PDN Connection establishment.

When the UE with ongoing PDN connection(s) moves from EPC to 5GC, the SMF+PGW-C triggers a request to decrease the number of the UE registration in NSACF and the AMF triggers a request to increase the number of the UE registration in NSACF when the UE is registered in the new AMF. If there are more than one PDN connections associated with the S-NSSAI, the NSACF may receive multiple requests for the same S-NSSAI from different SMF+PGW-Cs. When the UE with ongoing PDU session(s) moves from 5GC to EPC, the SMF+PGW-C triggers a request to increase the number of the UE registration in NSACF and the old AMF triggers a request to decrease the number of the UE registration in NSACF when the UE is deregistered in old AMF. If there are more than one PDU sessions associated with the S-NSSAI, the NSACF may receive multiple requests for the same S-NSSAI from different SMF+PGW-Cs. The NSACF maintains a list of UE IDs based on the requests from SMF+PGW-C(s) and AMF, and adjusts the current number of registrations accordingly.

When EPS counting is performed for a network slice, and the UE with ongoing PDN connection(s) moves from EPC to 5GC, session continuity is guaranteed from NSAC standpoint, as the admission was granted at the time of PDN connection establishment, i.e. the number of PDU session is not counted again in 5GC. Similarly, when the UE with ongoing PDU session(s) moves from 5GC to EPC, session continuity is guaranteed from NSAC standpoint as the admission of the PDN Connection(s) to the network slice was already granted at the time of PDU Session establishment in 5GC.

To support session continuity in case of handover between EPC and 5GC, AMFs verifies if the subscription associated with the S-NSSAI indicates that the S-NSSAI is subject to EPC interworking and NSAC hence session continuity is to be guaranteed. This enables AMFs to instruct an NSACF to ensure session continuity for the S-NSSAI. In this case the NSACF does not reject the handover from EPC to 5GC while roaming or at home even if the maximum number of UE registrations  with the network slice has been reached. This ensures successful UE registration in 5GC in this case. When a UE hands over from 5GC to EPC, the SMF+ PGW-C verifies if UE is coming from 5GS and NSAC has to be performed on the S-NSSAI for number of UEs registered with an S-NSSAI. If this is the case, the SMF+ PGW-C includes in the message to the NSACF an indication for the S-NSSAI that session continuity is to be guaranteed. It is up to implementation to ensure that the maximum number of registered UEs does not go over maximum S-NSSAI Registered UE limit while ensuring SC in either of the above handover cases.

NOTE X:	Different NSACFs are used in case of roaming between 5GC and EPC with home routed PDN connections.

If the PDN connection associated with S-NSSAI is released in EPC, the SMF+PGW-C triggers a request (i.e. decrease) to NSACF for maximum number of UEs and/or maximum number of PDU sessions per network slice control. The NSACF decreases the current number of registrations and removes the UE identity from the list of UE IDs if the PDN connection(s) associated with the S-NSSAI are all released in EPC.

NOTE 3:	Network Slice Admission Control in EPC is not performed for the attachment without PDN connectivity.

If EPS counting is not required for a network slice, the Network Slice Admission Control for maximum number of UEs and/or for maximum number of PDU Sessions per network slice is performed when the UE moves from EPC to 5GC, i.e. when the UE performs mobility Registration procedure from EPC to 5GC (Network Slice Admission Control for maximum number of UEs per network slice) and/or when the PDN connections are handed over from EPC to 5GC (Network Slice Admission Control for maximum number of PDU Sessions per network slice). The SMF+PGW-C is configured with the information indicating the network slice is subject to NSAC only in 5GS. The PDN connection interworking procedure is performed as described in clause 5.15.7.1. Mobility from EPC to 5GC does not guarantee all active PDU Session(s) can be transferred to the 5GC in certain circumstances when either the current number of UE registration or the current number of PDU sessions would exceed the maximum number when the UE moves from EPC to 5GC.

NOTE 4:	Given that session continuity is not guaranteed when EPS counting is not required, it is recommended for services which require the session continuity to support EPS counting.

# * * * * End of changes * * * *