3GPP TSG SA WG2 Meeting SA2#148E 		S2-2108368

November 15 – 22, 2021, Electronic, Elbonia

Title:	[DRAFT] Reply LS on mandatory SSC modes supported by UE

Response to:	LS to SA2 on mandatory SSC modes supported by UE (C6-210258 / S2-2108269)

Release:	Rel-15 / Rel-16

Work Item:	5GS_Ph1_UEConTest

Source:	Ericsson [to be: SA2]

To:	CT6

Cc:	CT1

Contact person: 	Stefan Rommer

stefan.rommer@ericsson.com

Attachment: 	none

# 1	Overall description

SA2 would like to thank CT6 for the LS on mandatory SSC modes supported by UE.

The current SA2 5GS specifications (TS 23.501, TS 23.502 & TS 23.503) in releases up to rel.17 do not specify whether any of SSC modes (1, 2 or 3) are mandatory or optional for the UE to support and use. In TS 23.228 clause Y.1.0 the following is stated:

To support IMS, the UE shall establish a PDU session with PDU session type set to IP for the corresponding DNN (see TS 23.501 [93], clause 5.6.1), and acquire an IP address according to TS 23.501 [93], clause 5.8.2.2. If IMS Multimedia Telephony Service as specified in TS 22.173 [53] is to be used within the PDU session, SSC mode 1 shall be set for the PDU session. In this release of the specification, it is assumed that single PDU session with multiple PDU session anchors defined in TS 23.501 [93] clause 5.6.4 is not applicable for PDU sessions dedicated to IMS.

From URSP rules perspective, the following is SA2's reply for Rel-15 and Rel-16:

Q1)	Is it correct, if we handle SSC modes 2 and 3 as optional?

SA2 reply: From URSP rules perspective, the support of SCC mode 2 and 3 is optional in the UE. The support of SSC Mode 1 is mandatory.

Q2)	Is it possible to modify 3GPP TS 23.501 to explicitly state which SSC modes have to be supported?

SA2 reply: SA2 is still discussing this topic.

Q3)	If the support of SSC mode 2 and/or 3 is not optional but conditional, can you please specify the conditions?

SA2 reply: Please see reply to Q1.

# 2	Actions

To CT6

ACTION: 	SA2 kindly requests CT6 to take the above information into account

# 3	Dates of next TSG SA WG2 meetings

SA2 WG Meeting #149 E		February 14 – 25, 2022		Elbonia

SA2 WG Meeting #150		April 4 – 8, 2022 		TBD