3GPP TSG-SA2 Meeting # 148E (e-meeting)	S2-2108445

# Nov 15 – 22, 2021											 	 	                      (revision of S2-210XXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3371 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the UE ID of SNPN UE used for accessing emergency services in PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A SNPN subscribed UE in limited states can access to a PLMN for emergency services. A PLMN may authenticate a UE during emergency registration or not. During the emergency registration, in case of authentication failure, the network may still accept the UE access or not, depend on operators’ policy. A PLMN doesn’t support NID and doesn’t support NSI based SUCI with NID. A SNPN’s MCC and MNC may not be unique Scenario1: the MCC and MNC of the subscribed SNPN is unique - NSI based SUPI shall not be used since the serving PLMN doesn’t support the NID not use it to select AUSF. So the authentication cannot be performed. If authentication is required by the PLMN, the SNPN subscribed UE may be rejected by the PLMN. - IMSI based SUPI shall be used in order to support the possible authentication by the serving PLMN. Scenario2: the MCC and MNC of the subscribed SNPN is not unique - Issue1: The AMF of serving PLMN cannot index the AUSF of subscribed SNPN based on the PLMN ID of the IMSI, so the authentication cannot be performed when IMSI is provided. - Issue2: UE A and UE B with the same IMSI may access a PLMN and the PLMN AMF cannot distinguish UE A and UE B. In order to fix the above issues, the following are proposed: - the IMSI based SUPI shall be included if and the MCC and MNC of the SNPN is unique. - the NSI based SUPI shall not be included. - the PEI shall be included when the MCC and MNC of the SNPN is not unique. - whether the MCC+MNC of the SNPN is unique or not is pre-configured in the subscription/credential. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the UE ID of SNPN UE used for accessing emergency services in PLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PLMN cannot differentiate or authenticate UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

5.30.2.4.1	General

An SNPN-enabled UE supports the SNPN access mode. When the UE is set to operate in SNPN access mode the UE only selects and registers with SNPNs over Uu as described in clause 5.30.2.4.

Emergency services are supported in SNPN access mode. If the UE is in limited service state, the UE shall attempt to camp on an acceptable cell of any available SNPN supporting emergency calls (irrespective of SNPN ID or GIN) or on any available PLMN supporting emergency calls (irrespective of PLMN ID).

When a UE uses SNPN subscription to perform emergency registration to a PLMN and if SUCI is required to provide as described in clause 4.2.2.2.2 of TS 23.502 [3], the SUCI is the IMSI based SUCI and NSI based SUCI is not provided.

If a UE is not set to operate in SNPN access mode, even if it is SNPN-enabled, the UE does not select and register with SNPNs. A UE not set to operate in SNPN access mode performs PLMN selection procedures as defined in clause 4.4 of TS 23.122 [17]. For a UE capable of simultaneously connecting to an SNPN and a PLMN, the setting for operation in SNPN access mode is applied only to the Uu interface for connection to the SNPN. Clause D.4 provides more details.

An SNPN-enabled UE that supports access to an SNPN using credentials from a Credentials Holder and that is equipped with a PLMN subscription needs to first enter SNPN access mode to be able to select SNPNs. Once the UE has entered SNPN access mode, SNPN selection is performed as described in clause 5.30.2.4. Once an SNPN has been selected the UE attempts registration in the SNPN using the PLMN credentials.

NOTE:	Details of activation and deactivation of SNPN access mode are up to UE implementation.

When a UE is set to operate in SNPN access mode the UE does not perform normal PLMN selection procedures as defined in clause 4.4 of TS 23.122 [17].

UEs operating in SNPN access mode read the information described in clause 5.30.2.2 from the broadcast system information and take them into account during network selection.

****************** END CHANGES ***************