3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108476r01

# Elbonia, November 15 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3246 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.502: TSCTSF Discovery and Selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Similar Reason for change as for the corresponding 23.501 CR titled “TSCTSF Discovery and Selection” If more than one TSCTSF intances are deployed for a given DNN, S-NSSAI in the network and the TSCTSF is discovered via NRF, then the selected TSCTSF shall store the assocation of (TSCTSF NF ID, DNN, S-NSSAI) in the UDR. Nevertheless, this is only applicable when AF request is received before PDU Session establishment. It doesn’t work when AF request is received after PDU Session establishment or even when there is no AF request for the PDU Session. Furthermore, it states that such assocation is deleted after all the PDU Session(s) for the given DNN/S-NSSAI are released, however, TSCTSF has no concept of “PDU Session” and TSCTSF is still in use when there is active AF request targeting to the given DNN/S-NSSAI for time sync. or TSC QoS service. Notably, this assocation is used to select the same TSCTSF for a given DNN and S-NSSAI, it should be independent of the time sync. application data, i.e. even when there is no time sync. data stored in the UDR, this association should also work. Crux of the problem is that the same UDR cannot be discovered for a given DNN/S-NSSAI thus it is not always possible to ensure we have selected the right UDR to retrieve TSCTSF NF ID. As a matter of fact, only one of the TSCTSF instances can be used for the given DNN, S-NSSAI even multipleTSCTSF intances for a given DNN, S-NSSAI are deployed. Additionly, support of multipleTSCTSF intances for a given DNN, S-NSSAI brings extra network complexilities and impacts, so it is hard to see the benefits in practice. In order to make TSCTSF discovery and selection workable, it is proposed to simply specify that only one TSCTSF instance for a given DNN, S-NSSAI. This avoids the impacts to UDR/BSF services and the extra complexilities. In order to make TSCTSF discovery and selection workable, it is proposed to consider the following options - Option 1: instead of UDR, use BSF service for the assocation of (TSCTSF NF ID, DNN, S-NSSAI) handling, i.e. PCF, NEF can retrieve BSF to obtain the assocation, and TSCTSF can (de)-register BSF for the assocation. - Option 2: simply specify that only one TSCTSF instance for a given DNN, S-NSSAI. This avoids the impacts to UDR/BSF services and the extra complexilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - (Option 2) Remove the UDR based TSCTSF instance discovery and selection for AF session with required QoS procedure and Time Synchronization service procedure - Updates for TSCTSF instance discovery for SMF initiated SM Policy Association Modification procdure, i.e., remove the repeated text and refer to clause 6.3.24 of TS 23.501 [2] - udpate the Npcf_Policy Authorization service, Nnef_AFsessionWithQoS and Ntsctsf_QoSandTSCAssistance |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Key logic regarding TSCTSF discovery and selection are not clear and misuse of UDR for this. This also brings extra network complexilities and impacts with little benefit. Additionaly, unresolved open issue kept. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.5.1, 5.2.5.1, 5.2.5.3.1, 5.2.5.3.3, 5.2.6.9.2, 5.2.6.9.5, 5.2.27.3.2, 5.2.27.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.16.5.1	SMF initiated SM Policy Association Modification

The SMF may initiate the SM Policy Association Modification procedure if a Policy Control Request Trigger is met.

NOTE 1:	When SMF instance is changed within the same SMF set the callback URI can be updated via this procedure.

Figure 4.16.5.1-1: SMF initiated SM Policy Association Modification

For local breakout roaming, the interaction with HPLMN (e.g. step 2) is not used. In local breakout roaming, the V-PCF interacts with the UDR of the VPLMN.

1.	When a Policy Control Request Trigger condition is met the SMF requests to update (Npcf_SMPolicyControl_Update) the SM Policy Association and provides information on the conditions that have been met as specified in clause 5.2.5.4.5.

If the SMF is notified by NRF that the stored PCF instance is not reachable, it should query the NRF for PCF instances within the PCF set and select another instance (see clause 6.3.1.0 of TS 23.501 [2]).

The QoS constraints from the VPLMN are provided by the H-SMF to the H-PCF in the home routed roaming scenario as defined in clause 4.3.2.2.2.

2.	When an AF has subscribed to an event that is met due to the report from the SMF, the PCF reports the event to the AF by invoking the Npcf_PolicyAuthorization_Notify service operation.

If the SMF has reported that new 5GS Bridge information has been detected and no subscription for "5GS bridge information Notification" event exists for this PDU session yet:

-		If integration with TSN applies (see clause 5.28 of TS 23.501 [2]), then the PCF informs a pre-configured TSN AF using the Npcf_PolicyAuthorization_Notify (5GS Bridge ID, the port number of the DS-TT port, and MAC address of the DS-TT Ethernet port for the PDU Session, and  UE-DS-TT Residence Time (if available)) service operation for the event of "5GS bridge information Notification" as described in clause 6.1.3.18 of TS 23.503 [20].

-	Otherwise, i.e. if the integration with TSN does not apply, the PCF may inform the pre-configured TSCTSF or a TSCTSF discovered or selected via NRF (as described in clause 6.3.24 of TS 23.501 [2]) using the Npcf_PolicyAuthorization_Notify (User Plane Node ID, the port number of the DS-TT port, and MAC address of the DS-TT Ethernet port or IP address for the PDU Session) service operation for the event of "5GS bridge information Notification" as described in clause 6.1.3.18 of TS 23.503 [20].

NOTE 2:	For a given DNN and S-NSSAI, it is assumed that the network only needs to deploy one TSCTSF instance or TSCTSF Set.

NOTE 3:	The PCF is assumed to be pre-configured to know when it needs to inform the TSCTSF (e.g. for UE-DS-TT Residence time for QoS delay estimation and/or for time sync services).

When the TSN AF or TSCTSF receives the Npcf_PolicyAuthorization_Notify message and no AF session exists for this PDU Session, the TSN AF or TSCTSF shall use the Npcf_PolicyAuthorization service described in clause 5.2.5.3 to request creation of a new AF session specific to the received MAC address of the DS-TT Ethernet port or IP address of the PDU Session. The TSN AF or TSCTSF shall then use the Npcf_PolicyAuthorization service to subscribe for notifications for 5GS Bridge information Notification event over the newly established AF session. The TSN AF or TSCTSF may provide a Port or User-Plane Management Information Container for the PDU Session and related port number in the Npcf_PolicyAuthorization creation request.

If the SMF has reported UE-DS-TT Residence Time or PMIC with port number or UMIC, then the PCF also provides these information elements to the TSN AF or TSCTSF.

When integration with TSN applies (see clause 5.28 of TS 23.501 [2]), the TSN AF calculates the bridge delay for each port pair, using the UE-DS-TT Residence Time of the DS-TT Ethernet port(s) for the 5GS bridge indicated by the 5GS user-plane Node ID.

3.	If the PCF determines a change to policy counter status reporting is required, it may alter the subscribed list of policy counters using the Initial, Intermediate or Final Spending Limit Report Retrieval procedures as defined in clause 4.16.8.

4.	The PCF makes a policy decision as described in TS 23.503 [20]. The PCF may determine that updated or new policy information needs to be sent to the SMF.

If the SMF reported accumulated usage for the PDU session in step 1 the PCF deducts the value from the remaining allowed usage for the subscriber, DNN, and S-NSSAI in the UDR by invoking Nudr_DM_Update (SUPI, DNN, S-NSSAI, Policy Data, Remaining allowed Usage data, updated data) service operation.

If the SMF reported accumulated usage for a MK(s) in step 1 the PCF deducts the value from the remaining allowed usage for the MK in the UDR by invoking Nudr_DM_Update (SUPI, DNN, S-NSSAI, Policy Data, Remaining allowed Usage data, updated data (including MK(s))) service operation.

When new PCF instance is selected in step 1, the new PCF should invoke Nbsf_Management_Update service operation to update the binding information in BSF.

In the non-roaming case, the PCF may subscribe to Analytics from NWDAF as defined in clause 6.1.1.3 of TS 23.503 [20].

In the home-routed roaming scenario, the H-PCF ensures that the QoS constraints provided by the VPLMN are taken into account as described in TS 23.503 [20].

NOTE 4:	For local breakout roaming, PDU Session policy control subscription information and Remaining allowed usage subscription information for monitoring control as defined in clause 6.2.1.3 of TS 23.503 [20] are not available in V-UDR and V-PCF uses locally configured information according to the roaming agreement with the HPLMN operator.

5.	The PCF answers with a Npcf_SMPolicyControl_Update response with updated policy information about the PDU Session determined in step 4.

# * * * * Next change * * * *

#### 5.2.5.1	General

The following table illustrates the PCF Services.

Table 5.2.5.1-1: NF services provided by PCF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Npcf_AMPolicyControl | Create | Request/Response | AMF |
|  | Update | Request/Response | AMF |
|  | UpdateNotify | Subscribe/Notify | AMF |
|  | Delete | Request/Response | AMF |
| Npcf_Policy Authorization | Create | Request/Response | AF, NEF, TSCTSF |
| (NOTE) | Update | Request/Response | AF, NEF, TSCTSF |
|  | Delete | Request/Response | AF, NEF, TSCTSF |
|  | Notify | Subscribe/Notify | AF, NEF, NWDAF, PCF, TSCTSF |
|  | Subscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
|  | Unsubscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
| Npcf_SMPolicyControl | Create | Request/Response | SMF |
|  | UpdateNotify | Subscribe/Notify | SMF |
|  | Update | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
| Npcf_BDTPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| Npcf_UEPolicyControl | Create | Request/Response | AMF, V-PCF |
|  | Update | Request/Response | AMF, V-PCF |
|  | UpdateNotify | Subscribe/Notify | AMF, V-PCF |
|  | Delete | Request/Response | AMF, V-PCF |
| Npcf_EventExposure | Subscribe | Subscribe/Notify | NEF, NWDAF |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| Npcf_AMPolicyAuthorization | Create | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
|  | Notify | Subscribe/Notify | AF, NEF |
|  | Subscribe |  | AF, NEF |
|  | Unsubscribe |  | AF, NEF |
| NOTE: In the Npcf_PolicyAuthorization operations, PCF is a consumer when the PCF for the UE and the PCF for the PDU session are different. |  |  |  |

# * * * * Next change * * * *

#### 5.2.5.3	Npcf_PolicyAuthorization Service

##### 5.2.5.3.1	General

Service description: This service is to authorise an AF request and to create policies as requested by the authorized AF for the PDU Session to which the AF session is bound. Additionally, this service allows an AF or TSCTSF to exchange port management information with DS-TT and NW-TT. This service allows the NF consumer to subscribe/unsubscribe the notification of events, which are defined in clause 6.1.3.18 of TS 23.503 [20].

# * * * * Next change * * * *

##### 5.2.5.3.3	Npcf_PolicyAuthorization_Update service operation

Service operation name: Npcf_PolicyAuthorization_Update

Description: Provides updated information to the PCF.

Inputs, Required: Identification of the application session context.

Inputs, Optional: Media type, Media format, bandwidth requirements, sponsored data connectivity if applicable, flow description, AF Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition, spatial validity condition, Information for EAS IP Replacement in 5GC, EAS deployment information and AF indication for simultaneous connectivity over source and target PSA at edge relocation as described in clause 5.6.7 of TS 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 of TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 of TS 23.503 [20], Port Management Information Container and related port number, User plane node Management Information Container, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.3.23 of TS 23.503 [20], individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order), Alternative QoS Related parameter set(s), QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], MPS for Data Transport Service indicator as described in clause 6.1.3.11 of TS 23.503 [20].

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in clause 6.1.3.16 of TS 23.503 [20]).

Outputs, Optional: The service information that can be accepted by the PCF.

Provides updated application level information and communicates with Npcf_SMPolicyControl service to determine and install the policy according to the information provided by the NF Consumer. Updates an application context in the PCF.

# * * * * Next change * * * *

##### 5.2.6.9.2	Nnef_AFsessionWithQoS_Create service operation

Service operation name: Nnef_AFsessionWithQoS Create

Description: The consumer requests the network to provide a specific QoS for an AF session.

Inputs, Required: AF Identifier, UE address (i.e. IP address or MAC address), Flow description(s) or External Application Identifier, QoS Reference.

Inputs, Optional: time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20] DNN if available, S-NSSAI if available, Alternative QoS Related parameter sets.

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

# * * * * Next change * * * *

##### 5.2.6.9.5	Nnef_AFsessionWithQoS_Update service operation

Service operation name: Nnef_AFsessionWithQoS Update

Description: The consumer requests the network to update the Service Requirement(s) and/or additional Alternative Service Requirement(s) for an AF session.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: Flow description, QoS reference, time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], Alternative QoS Related parameter sets.

Outputs, Required: Result.

Output (optional): None.

# * * * * Next change * * * *

##### 5.2.27.3.2	Ntsctsf_QoSandTSCAssistance_Create operation

Service operation name: Ntsctsf_QoSandTSCAssistance_Create

Description: The consumer requests the network to provide a specific QoS for an AF session.

Inputs, Required: AF Identifier, UE address, Flow description(s) or External Application Identifier, QoS Reference.

Inputs, Optional: time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters  in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting as described in clause 6.1.3.21 of TS 23.503 [20], individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20] , DNN if available, S-NSSAI if available, Alternative QoS Related parameter sets as described in clause 6.1.3.22 of TS 23.503 [20].

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

# * * * * Next change * * * *

##### 5.2.27.3.3	Ntsctsf_QoSandTSCAssistance_Update operation

Service operation name: Ntsctsf_QoSandTSCAssistance_Update

Description: The consumer requests the network to update the QoS and/or additional Alternative QoS for an AF session.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: Flow description, QoS reference, time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting as described in clause 6.1.3.21 of TS 23.503 [20], individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], Alternative QoS Related parameter sets.

Outputs, Required: Result.

Output (optional): None.

# * * * * End of changes * * * *