3GPP TSG-SA2 Meeting #148E	S2-2108479

# 15 – 22 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0675 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fixes for AF session with required QoS update procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-11-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Provide updates to 23.503 for TSC QoS Changes are required to align TS23.503 AF Session with Required QoS with TS 23.502 clause 4.15.6.6 and 4.15.6.6a (Setting Up and Updating AF Session with required QoS). Changes to 23.502 permit inclusion of Traffic pattern and QoS parameters in AF requests for QoS, including for Alt-QoS. TS23.503 needs to be aligned with the changes in TS23.502 and current alternative QoS specificatoin in 23.501. Specifically: - Requested GFBR is specified as an individaul QoS Parameter in 23.502, clause 4.15.6.6 but it is missing in 23.503 clause 6.1.3.22 - Alternative QoS Related parameters for Alternative Service Requirements are specifed in 23.502 but are not defined in 23.503. These are defined as the parameters required in 23.501 clause 5.7.1.2a for the Alternative QoS Profile. Two options are allowed for the AF Request: 1 – Only a QoS Reference is provided, which is the pre-Rel-17 status-quo 2 –Only Individual QoS parameters are provided with no QoS Reference Note the PCF formulates a PCC rule taking into account 1 - QoS parameters and 2 - Alternative QoS related parameters requested by the AF. The SMF (PCC rules) and NG-RAN (QoS Profile) are not affected by these changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 23.503 clause 6.1.3.22 is updated for consistency with 23.501 and 23.502 and to allow an AF to specify individual QoS Related Parameters |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS23.501, TS23.502 and TS23.503 are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.1.3.22	AF session with required QoS

The AF may request that a data session to a UE is set up with a specific QoS (e.g. low latency or jitter) and priority handling. The AF can request the network to provide QoS for the AF session based on the service requirements with the help of a QoS reference parameter which refers to pre-defined QoS information. In place of the QoS reference, the AF may provide individual QoS parameters associated to the Flow Description.

a)	When the AF provides only a QoS reference to determine the QoS parameters but no QoS parameters:

-	When the PCF authorizes the service information from the AF and generates a PCC rule, it derives the QoS parameters of the PCC rule based on the service information and the indicated QoS reference parameter.

NOTE 1:	A SLA has to be in place between the operator and the ASP defining the possible QoS levels and their charging rates. For each of the possible pre-defined QoS information sets, the PCF needs to be configured with the corresponding QoS parameters and their values as well as the appropriate Charging key (or receive this information from the UDR).

-	The AF may change the QoS by providing a different QoS reference parameter while the AF session is ongoing. If this happens, the PCF shall update the related QoS parameter sets in the PCC rule accordingly.

b)	When the AF provides individual QoS parameters and no QoS Reference:

-	The AF provides the following QoS Parameters associated with the Flow Description. One of the following:

a)	In the case that the AF takes the role of TSN AF which is a 5GC NF, TSC Assistance Container: describes the traffic characteristics (burst arrival time, periodicity, survival time and Flow direction) needed for TSCAI determination (as described in clause 5.27 of TS 23.501 [2]); or

b)	In the case that the AF belongs to a third party: burst arrival time, flow direction, periodicity, survival time and time domain.

-	The AF provides QoS information, i.e. priority, maximum Burst Size, Requested 5GS delay and Maximum Flow Bitrate and Guaranteed Flow Bitrate.

-	If the AF request for QoS is sent via the TSCTSF, the request contains a Requested 5GS Delay and the TSCTSF has received the UE-DS-TT Residence Tine from the PCF, the TSCTSF provide a Requested PDB.

-	When the PCF authorizes the service information from the AF, it derives the QoS parameters of the PCC rule based on the service information and the individual QoS information received from the AF and TSCTSF. The PCF can select a standardized, pre-configured or existing dynamically assigned 5QI that matches the QoS parameters. Otherwise, the PCF generates a new dynamically assigned 5QI based on the QoS parameters.

The PCF performs Session binding using the DS-TT port MAC address or UE IP address.

The PCF generates a PCC Rule with service data flow filter (including IP Packet Filter set as in clause 5.7.6.2 of TS 23.501 [2]) or Ethernet Packet Filter set as in clause 5.7.6.3 of TS 23.501 [2]) derived from the Flow Descriptions provided by the AF, the mapped 5QI, ARP, GBR and MBR, and the associated TSC Assistance Container as received from the AF or TSCTSF.

For TSC QoS, the PCF derives the 5QI value as defined in clause 5.27.3 of TS 23.501 [2], the PCF derives the MBR using the Maximum Flow Bitrate provided by the AF and sets the GBR equal to the MBR unless the AF provides a Guaranteed Flow Bitrate in which case the MBR and GBR are set separately. The SMF binds the PCC Rule to a QoS Flow as defined in clause 6.1.3.2.4.

If the PCF gets informed about Policy Control Request Triggers relevant for the AF session, the PCF shall inform the AF about it as defined in clause 6.1.3.18.

If an AF session can adjust to different QoS parameter combinations, the AF may provide Alternative Service Requirements containing one or more QoS reference parameters in a prioritized order (which indicates the preference of the QoS requirements with which the service can operate). If so, the AF shall also subscribe to receive notifications from the PCF for successful resource allocation and when the QoS targets can no longer (or can again) be fulfilled as described in clause 6.1.3.18. In place of the QoS Reference, the AF may provide individual Alternative QoS Related parameters. These parameters comprise the parameters used in the Alternative QoS Profile specified in TS23.501 clause 5.7.1.2a..

When the PCF authorizes the service information from the AF and generates a PCC rule, it shall also derive Alternative QoS parameter sets for this PCC rule based on the QoS reference parameters and Alternative QoS Related parameters in the Alternative Service Requirements.

The PCF shall enable QoS Notification Control and include the derived Alternative QoS parameter sets (in the same prioritized order indicated by the AF) in the PCC rule sent to the SMF. When the PCF notifies the AF that QoS targets can no longer be fulfilled, the PCF shall include the QoS reference parameter or the set of individual QoS information corresponding to the Alternative QoS parameter set referenced by the SMF, or an indication that the lowest priority QoS reference or the set of individual QoS information of the Alternative Service Requirements cannot be fulfilled (as described in clause 6.1.3.18).

NOTE 2:	The AF behaviour is out of the scope of this TS but can include adaptation to the change of QoS (e.g. rate adaptation) as well as application layer signalling with the UE.

The AF may change the Alternative Service Requirements while the AF session is ongoing. If this happens, the PCF shall update the Alternative QoS parameter sets in the PCC rule accordingly.

The AF may indicate to the PCF that the UE does not need to be informed about changes related to Alternative QoS Profiles. With this indication received from the AF, the PCF decides whether to disable the notifications to the UE when changes related to the Alternative QoS Profiles occur and sets the Disable UE notifications at changes related to Alternative QoS Profiles parameter in the PCC rule accordingly.

End of CHANGEs