3GPP TSG-SA WG2 Meeting #148-E	S2-2108520

# Elbonia, 15 – 22 November 2021                                                (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3384 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NSAC mechanism for session continuity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A mechanism to support session continuity during the mobility between AMFs, or EPC and 5GC for the case that EPS counting is required is postponed. A mechanism is only to support session continiuity for on-going PDU sessions, and shall not allow to establish a new PDU session if the max number of UE has been reached. This CR proposes to resolve the issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To realize this, it is proposed that - AMF stores the information indicating that the S-NSSAI is exempt from NSAC and corresponding on-going PDU session ID(s). - After successful registration, if the UE requests a new PDU session associated with the S-NSSAI, the AMF rejects the request as the S-NSSAI is allowed only for the existing PDU sessions. - When all existing PDU session(s) associated with the S-NSSAI are released, and the current number of UEs still exceeds the maximum number of UEs, the AMF puts the S-NSSAI in rejected S-NSSAIs, and provides the new Allowed NSSAI and rejected S-NSSAI to the UE via UE Configuration Update procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network Slice Admission Control doesn’t work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st change ***

5.15.11.1	Maximum number of UEs per network slice admission control

The NSACF controls (i.e. increase or decrease) the current number of UEs registered for a network slice so that it does not exceed the maximum number of UEs allowed to register with that network slice. The NSACF also maintains a list of UE IDs registered with a network slice that is subject to NSAC. When the current number of UEs registered with a network slice is to be increased, the NSACF first checks whether the UE Identity is already in the list of UEs registered with that network slice and if not, it checks whether the maximum number of UEs per network slice for that network slice has already been reached.

The AMF triggers a request to NSACF for maximum number of UEs per network slice admission control when the UE's registration status for a network slice subject to NSAC may change, i.e. during the UE Registration procedure in clause 4.2.2.2.2 of TS 23.502 [3], UE Deregistration procedure in clause 4.2.2.3 of TS 23.502 [3], Network Slice-Specific Authentication and Authorisation procedure in clause 4.2.9.2 of TS 23.502 [3], AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure in clause 4.2.9.3 of TS 23.502 [3], and AAA Server triggered Slice-Specific Authorization Revocation in clause 4.2.9.4 of TS 23.502 [3].

The UE may register or deregister for an S-NSSAI via 3GPP access and/or non-3GPP access as described in clause 5.15.5.2.1. The AMF provides the Access Type to the NSACF when triggering a request to increase or decrease the number of UEs. The NSACF takes Access Type into account for increasing and decreasing the number of UEs per network slice. The NSACF stores UE ID with the associated one or more Access Type(s), i.e. the NSACF is able to add or remove a registration for the UE ID for each Access Type.

NOTE 1:	For example, if the NSACF is configured to apply NSAC for 3GPP Access Type only, the NSACF counts registration via 3GPP access type only. If the NSACF is configured to apply NSAC for both Access Types, and the UE newly registers via 3GPP access while the UE is already registered via non-3GPP access (or vice versa), the NSACF updates the UE ID entry with both 3GPP Access Type and non-3GPP Access Type and the NSACF counts the UE twice.

To support session continuity in case of handover between AMFs, or EPC and 5GC if EPS counting is required for a network slice, AMF instructs the NSACF to ensure session continuity for the S-NSSAI, and ignore slice congestion in case of handover, and ensures successful UE registration in 5GC in this case. Based on operator policy, NSACF may accept the request from the AMF. If the maximum number of UEs has been already reached, the NSACF also returns a result parameter indicating that the maximum number of UEs registered with the network slice has been reached. The AMF stores the information indicating that the S-NSSAI is exempt from NSAC and corresponding on-going PDU session ID(s) as UE context. The AMF provides the S-NSSAI in Allowed NSSAI to the UE to gurantee the session continuity for the on-going PDU session(s). After successful registration, if the UE requests a new PDU session associated with the S-NSSAI, the AMF rejects the request as the S-NSSAI is allowed only for the existing PDU sessions. When all existing PDU session(s) associated with the S-NSSAI are released, the AMF triggers Nnsacf_NSAC_NumOfUEsUpdate service operation in clause 5.2.21.2.2 of TS 23.502 [3] to see if the S-NSSAI can be continiuosly provided to the UE as allowed S-NSSAI. If the current number of UEs still exceeds the maximum number of UEs, the AMF puts the S-NSSAI in rejected S-NSSAIs, and provides the new Allowed NSSAI and rejected S-NSSAI to the UE via UE Configuration Update procedure.

NOTE 2:	The details of cause value when the PDU Session Request including the S-NSSAI which is allowed only for the existing PDU sessions is rejected is left to stage 3.

*** End of the changes ***