3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108570r02

Elbonia, November 15 – 22, 2021	(revision of S2-210xxxx)

Title:	[DRAFT] LS on support of DCS variants in UE Onboarding Architecture

Response to:

Release:	Release 17

Work Item:	eNPN

Source:	[Huawei to be] SA2

To:	SA3

Cc:

Contact Person:

Name:	zhuqianghua

Tel. Number:

E-mail Address:	zhuqianghua AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 agreed to support two architecture variants for DCS in case of UE  onboarding to a SNPN:

-	Variant 1: DCS includes AUSF and UDM functionalities

-	Variant 2: DCS includes AAA server functionality

SA2 is discussing two possible options for primary authentication towards DCS when an Onboarding SNPN interworks with a DCS supporting both AAA and AUSF/UDM functionality:

- DCS uses two different Home Network Identifiers (HNI), one for AAA, the other for AUSF/UDM. The Onboarding SNPN selects corresponding AAA or AUSF/UDM based on the HNI provided by the UE in the Onboarding SUCI. Each UE is authenticated during Onboarding via either AAA or AUSF/UDM that corresponds to the HNI provided by the UE in the Onboarding SUCI. The Onboarding SNPN might need to interwork with both AAA and AUSF/UDM, if different UEs using different HNIs.

- DCS uses only one HNI. An Onboarding SNPN can, based on an agreement with the DCS, only use one functionality (either AAA or AUSF/UDM) for primary authentication of all UEs providing this HNI in the Onboarding SUCI. As a consequence, as different Onboarding SNPNs can have different agreements with the DCS, it must be possible that each UE can be authenticated via AAA in a DCS via one Onboarding SNPN, and be authenticated via AUSF/UDM in the same DCS via another Onboarding SNPN .

Additionally, SA2 also agreed that the AUSF in Onboarding SNPN skips the UDM selection and directly performs primary authentication towards DCS supporting AAA server functionality based on local configuration. SA2 asks SA3 to consider this information in their specification accordingly.

2. Actions:

To SA3 group.

ACTION: 	SA2 kindly asks SA3 to take the above information into account and provide any feedback on the feasibility of the two interworking options as described above..

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #149E   	February 14 – 25, 2022 			Elbonia

TSG-SA2 Meeting #150   	April 4 – 8, 2022 				TBD