3GPP TSG-SA2 Meeting #148E	S2-218586

# Electronic, November 15 – 22, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3189 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to AF-triggered dynamically changing AM policies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_DCAMP |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In the case the request from the AF for the service area coverage and high throughput are associated with the (DNN, S-NSSAI) combination(s), there is no need to report the identity of the PCF for each PDU Session related to the (DNN, S-NSSAI). - In the case that the BSF is subscribed to notify the PDU session establishment and termination, the BSF only needs to report when the first SM policy association is established and when the last SM policy association is terminated to the same (DNN, S-NSSAI) combination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose to report the registration of the first SM policy association establishment and deregistration of the last SM policy association if service area coverage and/or the indication of high throughput provided by the AF are associated with the (DNN, S-NSSAI). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary signalling for DCAMP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.14.1, 4.16.14.2, 5.2.13.2.6, 5.2.13.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Management |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------First Change---------------------------------------------

4.16.14.1	General

The procedure for management of AM Policies (access and mobility related policy) depending on the application in use enables modification of the Access and Mobility policies on detection of the start and stop of an application.

The content of this clause applies to non-roaming i.e. to cases where the involved entities (e.g. PCF, SMF and UPF) belong to the home PLMN. The PCF shall not apply a change of AM policies for application traffic detected in PDU Sessions established in Home Routed mode.

If PCF for the UE and PCF for the PDU Session are the same PCF, then steps 2, 4, 5, and 8 in Figure 4.16.14.2-1 are not performed.

If the PCF for the UE and the PCF for the PDU Session are different PCFs, then the PCF for the UE is informed when a SM policy Association is established or released by either:

-	Subscription to the BSF:

-	see steps 2, 3, 5a and 12a in Figure 4.16.14.2.1-1. The BSF reports when a PCF is registered or deregister for the PDU Session to a DNN, S-NSSAI.

-	see steps 2, 3, 5 and 8 in Figure 4.16.14.2.2-1. The BSF reports the registration of a PCF for the PDU session when the first SM Policy association is established and the deregistration of the PCF for the PDU session when the last SM policy association is terminated for a (DNN, S-NSSAI).

-	Request to the PCF for the PDU Session to a DNN, S-NSSAI via AMF and SMF. See steps 1, 5b, 10 and 12b in Figure 4.16.14.2-1. The PCF for the PDU Session reports that the SM Policy Association is established as described in clause 4.16.4 and provides the UE address(es). The SM Policy Association Termination notifies the PCF for the UE as described in clause 4.16.6.

----------------------------------------Next Change---------------------------------------------

4.16.14.2	Procedures for management of AM Policies

4.16.14.2.1	Management of AM Policies at start and stop of application traffic

This procedure applies when the AF provides a service coverage area or the indication of high throughput associated with the Application Identifer(s).

Figure 4.16.14.2.1-1: Management of AM Policies at start and stop of application traffic

1.	The AMF establishes an AM Policy Association for Access and Mobility Policy Control, e.g. RFSP index value, as described in clause 4.16.1.2.

2.	If the AM Policies depends on the application in use, then depending on operator policies in the PCF, the PCF may subscribe to BSF, then step 3 follows, or provides its PCF binding information to the AMF with the indication to be notified about the PCF for a PDU Session for a DNN, S-NSSAI, then step 4 follows.

3.	The PCF for the UE determines that AM Policies (e.g. RFSP index value) depend on the detection of one or more application(s) in use, the DNN, S-NSSAI used to access each Application Id is configured in the PCF or retrieved from the UDR as part of the Application Data Set, then subscribes to the BSF to be notified when a PCF for a PDU Session for this SUPI is registered in the BSF, by invoking Nbsf_Management_Subscribe (SUPI, list of (DNN, S-NSSAI)(s)). Steps 3 and 4 are repeated for each PCF registered for a PDU Session to a (DNN, S-NSSAI) included in the Nbsf_Management.

4.	The SMF establishes a SM Policy Association as described in clause 4.16.4. The allocated UE address/prefix, SUPI, DNN, S-NSSAI and the PCF address is registered in the BSF, as described in clause 6.1.1.2.2 in TS 23.503 [20].

5a.	If the PCF for the UE subscribed to BSF, then the BSF notifies that a PCF for the PDU Session is registered in the BSF, by invoking Nbsf_Management_Notify (DNN, S-NSSAI, UE address(es), PCF address, PCF instance id, PCF Set ID, level of binding). When there are multiple PDU Sessions to the same (DNN, S-NSSAI) the BSF provides multiple notification to the PCF.

5b.	If the PCF for the UE sent the request to notify that a PCF for the PDU Session is available to the AMF in step 2, then the PCF for the PDU Sessions sends Npcf_PolicyAuthorization_Notify (EventID set to SM Policy Association established, UE address, PCF address, PCF instance is, PCF Set ID) to the PCF indicated in the PCF binding information provided by the SMF.

6.	The PCF for the UE subscribes to notifications of event "start/stop of application traffic" as defined in clause 6.1.3.18 in TS 23.503 [20], using Npcf_PolicyAuthorization_Subscribe (UE address, EventId, EventFilter set to "list of Application Identifiers") to the PCF for the PDU Session to the DNN, S-NSSAI. The PCF for the PDU Session then generates PCC Rules including the Application Identifier in the SDF template, if there are multiple Application Identifiers included in the Npcf_PolicyAuthorization, the PCF generates PCC Rules for each Application Identifier. The response includes the NotificationCorrelationId.

7.	The PCF installs PCC Rules and the Policy Control Request Trigger to detect "start/stop of application traffic" in the SMF.

8.	The SMF detects that the Policy Control Request Trigger is met, then reports the start/stop of application traffic to the PCF serving the PDU session.

9.	The PCF for the UE is notified on the start/stop of application traffic by Npcf_PolicyAuthorization_Notify (NotificationCorrelationId, EventId set to "start/stop of application traffic", EventInformation including the ApplicationId). When the reporting of start and stop of a list of Application(s) was requested, the PCF for the PDU Session provides multiple notification to the PCF for the UE.

10.	The PCF checks operator policies then may change AM Policies (e.g. RFSP index value) based on the reporting of start/stop of application traffic.

11.	The SM Policy Association is terminated as described in clause 4.16.6. The allocated UE address/prefix, SUPI, DNN, S-NSSAI and the PCF address are deregistered in the BSF.

12a.	If the PCF for the UE subscribed to BSF, then BSF notifies that the PCF serving a PDU session is deregistered in the BSF, by invoking Nbsf_Management_Notify (Binding Identifier for the PDU Session).

12b.	If the PCF for the UE sent the request to notify that a PCF for the PDU Session is available to the AMF in step 2, then the PCF for the PDU Sessions sends Npcf_PolicyAuthoritation_Notify ((EventID set to SM Policy Association termination, Notification Correlation Id).

NOTE:	The PCF for the UE may subscribe to the notifications of newly registered PCF for the PDU Session and subscribe to the "start/stop of application traffic detection" events for multiple applications with different application identifiers. When PCF receives the notifications for multiple applications, the PCF for the UE can determine which AM policy to apply based on local configuration and operator policy.

4.16.14.2.2	Management of AM Policies at SM policy association establishment and termination with the notification sent by the BSF

Figure 4.16.14.2.2-1: Management of AM Policies at SM policy association establishment and termination with the notification by the BSF

1.	The AMF establishes an AM Policy Association for Access and Mobility Policy Control, e.g. RFSP index value, as described in clause 4.16.1.2.

2.	If the AM Policies depend on the SM policy association establishment and termination for a DNN, S-NSSAI combination, then depending on operator policies in the PCF, the PCF may subscribe to BSF and then step 3 follows, or provides its PCF binding information to the AMF with the indication to be notified about the PCF for a PDU Session for a DNN, S-NSSAI, and then step 4 follows.

3.	The PCF for the UE determines that AM Policies (e.g. RFSP index value) depend on the detection of SM Policy association establishment association with the (DNN, S-NSSAI) combinations configured in the PCF or retrieved from the UDR as part of the Application Data Set, then subscribes to the BSF to be notified when the the PCF is registered for the first SM policy association establishment and the PCF is deregistered for the last SM policy association termination to the same (DNN, S-NSSAI) combination in the BSF, by invoking Nbsf_Management_Subscribe (SUPI, list of (DNN, S-NSSAI)(s), indication of registration/deregistration per (DNN, S-NSSAI)).

4.	The SMF establishes a SM Policy Association as described in clause 4.16.4. The allocated UE address/prefix, SUPI, DNN, S-NSSAI and the PCF address are registered in the BSF, as described in clause 6.1.1.2.2 of the TS 23.503 [20].

5.	If the PCF for the UE subscribed to BSF, then the BSF notifies a PCF registration when the first SM policy association corresponding to the (DNN, S-NSSAI) combination is established, by invoking Nbsf_Management_Notify (DNN, S-NSSAI, notification of registration).

6.	The PCF checks operator policies, and then may change AM Policies (e.g. RFSP index value) based on the reporting of SM policy association establishment.

7.  The SM Policy Association is terminated as described in clause clause 4.16.6, and the allocated UE address/prefix, SUPI, DNN, S-NSSAI and the PCF address are deregistered in the BSF.

8. If the PCF for the UE subscribed to BSF, then BSF notifies of a PCF deregistration when the last SM policy assocation corresponding to the (DNN, S-NSSAI) combination is terminated, by invoking Nbsf_Management_Notify (DNN, S-NSSAI, notification of deregistration).

----------------------------------------Next Change---------------------------------------------

5.2.13.2.6	Nbsf_Management_Susbcribe service operation

Service Operation name: Nbsf_Management Susbcribe

Description: NEF or AF or PCF for a UE can subscribe to be notified of newly registered or deregistered PCF for a PDU Session. In addition, NEF or AF can subscribe to be notified of newly registered or deregistered PCF for a UE.

NOTE 1:	If BSF has already the requested information at the time of the subscription, it will accept the subscription request, and will immediately provide the results in the Outputs parameters.

Inputs, Required: SUPI, DNN(s) [Required, if PCF subscription is for a PDU Session], S-NSSAI(s) [Required, if PCF subscription is for a PDU Session], callback URI.

Inputs, Optional: GPSI, indication of registration/deregistration per (DNN, S-NSSAI).

Indication of registration/deregistration per (DNN, S-NSSAI) indicates to the BSF to report when the first SM policy association is established and when the last SM policy association is terminated to the same (DNN, S-NSSAI) combination.

NOTE 2:	It is up to stage3 to ensure an unambiguous error proof way for the BSF to differentiate between PCF for a PDU Session and PCF for a UE. This may or may not require providing the BSF additional input parameter(s).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Conditional: One or more instance per (DNN, S-NSSAI) of (UE address(es) [If available], PCF address(es) [If available], PCF instance ID [If available], PCF Set ID [If available], level of Binding [If available] (see clause 6.3.1.0 of TS 23.501 [2])) or notification of registration/deregistration per (DNN, S-NSSAI).

NOTE 3:	The parameter UE address(es) is not applicable in the case of PCF for a UE.

----------------------------------------Next Change---------------------------------------------

5.2.13.2.8	Nbsf_Management_Notify service operation

Service Operation name: Nbsf_Management Notify

Description: BSF can notify NEF or AF or PCF for a UE of newly registered PCF for a PDU Session or of deregistered PCF for a PDU Session.

Inputs, Required: Notification Correlation Information, One or more instance per (DNN, S-NSSAI) of (UE address(es) [Required, if PCF notification is for a PDU Session], PCF address(es), PCF instance ID [Conditional, if available] and PCF Set ID [Conditional, if available]), level of Binding [Conditional, if available] (see clause 6.3.1.0 of TS 23.501 [2]) or notification of registration/deregistration per (DNN, S-NSSAI).

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

---------------------------------------- End of Changes----------------------------------------