3GPP TSG-SA2 Meeting # 148E (e-meeting)	S2-2108612

Nov 15 - 22, 2021, Electronic	(revision of S2-2108011)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0022 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Leftover issue for MBS session activation and deactivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Based on [Huawei, HiSilicon, vivo], CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Shared tunnel is established even the MBS session is in inactive state. Now the descripiton may be interpretted as only used when the MBS session is active. - For the MBS session activation how the SMF notify to the AMF is still FFS. There are two option to resolve this issue: - Option 1: SMF gives the UE list and a callback URI to the AMF. Per that callback URI, AMF notify the SMF on which UE is reachable and unreachable. - Option 2: SMF gives a list of (UE, associated PDU session ID) to the AMF. Per that associated PDU session ID, the AMF notify the SMF same as the UE activate the related PDU session. For option 2, it means that the MBS session context in the SMF includes UE list as well as associated PDU session ID for each UE. After that when the UE triggers the service request, the AMF checks whether any PDU session is to be activated by checking the information of PDU session to be activated list from the UE and PDU Session IDs received from the SMF. Option-1 is considered necessary for the UEs that do not respond to paging. For CONNECTED UEs included in Namf_MT_EnableGroupReachability, N1N2Transfer message is stil needed. Considering different scenarios, it is proposed to adopt both options. - How the AMF figure out the paging area is unclear and need be clarified. - Add some clarification on clause 7.2.8 to be more clear that SR is related to the inactive MBS session. Rev02: 1. In 7.2.5.2 step 4b, which is for UEs in CM-CONNECTED state, the PDU Session ID is not included in the service operator. For the case of homogenous RAN, individual fall-back is not needed, CN could use per UE interaction or per node interaction based on operator policy. In the case of per node interaction, the PDU Session ID is not needed. But for the case of non-homogenous RAN, individual fall-back is needed, hence per UE interaction is needed. In this case, the PDU Session ID is needed. For the case of homogenous RAN, if the associated PDU Session is active, RAN will have the UE context with MBS Session information, step 3-10 can be skipped. 2. In step 3, the PDU Session ID and Notification Address cannot be indicated both. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the unsuitalbe restriction that MBS session need be in activte state. - Propose to reuse MT_EnableGroupReachability service to notify SMFs when the UEs are not reachable, and propose to reuse Nsmf_PDUSession_UpdateSMContext to inform SMFs when UEs are reachable after receiving the service request messages from the UEs. - Clarify the way of the AMF figuring out the paging area. - Add the related description that the SR is related to inactive MBS session. Additional changes compared to r01: Rev02: 1. In step 3, make the two parameters optional, and describe only one of the parameter is included. 2. In step 2, specify that, depends on operator policy, steps 3-10 will be skipped. 3. Some editorial modification, including modifying the figure of step 10b that the “steps 9-12” shall be “steps 8-12”. - MBS session activation procedure: clarify the parameters in Nmbsmf_MBSSession_ContextStatusNotify and Namf_Communication_N1N2MessageTransfer (in step 4b); correct the usage of UE reachability Notification Address (in step 3). - Service request procedure: clarify that the procedure also applies to associated PDU session activation when the UE is in CM-CONNECTED state and the user plane of the associated PDU Session is deactivated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is conflict with other part that the shared tunnel can be established even in the inactive state. - It is unclear that how could the AMF notifying the SMF after receiving the service requests from the UEs. - It is unclera whether all the UE registration area need be considered for paging. - It is unclear on whether the SR is related to the active MBS session and whether it is applicable to UE in CM-CONNECTED state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1.4, 7.2.5.2, 7.2.5.3, 7.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 7.2.1.4	Establishment of shared delivery toward RAN node

In the following cases, the shared tunnel for shared delivery is established between the NG-RAN and MB-UPF:

-	The first UE is included in the context of the MBS session in the NG-RAN.

NOTE 1:	When the MBS session is deactivated, if there is at least one UE joining the MBS session is in connected mode in the NG-RAN, the shared delivery is not released.

NOTE 2:	Share delivery establishment procedures are used when MBS supporting NG-RAN node(s) get involved in the MBS session regardless of the state of the MBS session.

-	Handover to the target NG-RAN when the shared delivery tunnel is not established in the target RAN node for this MBS session.

NG-RAN

AMF

MB-SMF

MB-UPF

1. Decide to establish shared delivery for a multicast session

2. N2 Message ()

3. Nmbsmf_MBSSession_ContextUpdate Request

4. N4mb Session Modification

6. Nmbsmf_MBSSession_ContextUpdate Response

7. N2 Message ()

5. Store AMF ID for the MBS session

Figure 7.2.1.4-1: Establishment of shared delivery toward NG-RAN node

1.	A NG-RAN node decides to establish shared delivery for an MBS session when it serves at least one UE within the MBS session. For location dependent services, the NG-RAN node needs to establish shared delivery for the location dependent contents of an MBS session if it serves at least one UE assigned to an MBS session ID and area session ID.

2.	The NG-RAN sends an N2 MBS Session request message (MBS Session ID, N2 SM information (MBS Session ID, [unicast Tunnel Info], [Area Session ID])) towards AMF.

If the NG-RAN node is configured to use unicast transport for the shared delivery, it allocates a GTP tunnel endpoint and provides the unicast Tunnel Info in the request, which includes the GTP tunnel endpoint and NG-RAN node address. For location dependent MBS services, the NG-RAN node also provides the Area Session ID.

3.	The AMF selects the MB-SMF serving the multicast session using the NRF discovery service. It invokes Nmbsmf_MBSSession_ContextUpdate request (MBS Session ID, N2 SM information) to the MB-SMF.

The AMF stores the information for the NG-RAN nodes (e.g. NG-RAN node ID) for the subsequent signaling related to the MBS Session.

4.	If the MB-SMF received unicast Tunnel Info in step 3, it configures the MB-UPF to send multicast data for the multicast session (or location dependent content of the multicast session if an area session ID was received) towards that GTP tunnel endpoint via unicast transport.

5.	The MB-SMF stores the AMF in the multicast session context (or location dependent part of the multicast session context if an area session ID was received) to enable subsequent signalling towards that NG-RAN node.

6.	The MB-SMF sends Nmbsmf_MBSSession_ContextUpdate response (MBS Session ID, N2 SM information ([TMGI], multicast QoS flow information, [transport multicast address], [multicast Tunnel Info])) to the AMF. If the MB-SMF did not receive unicast Tunnel Info in step 3, it provides the transport multicast address (e.g. a LL SSM) and a GTP tunnel endpoint for multicast transport of the shared delivery.

7.	The AMF sends an N2 MBS Session response (MBS Session ID, N2 SM information) to the NG-RAN node. If the NG-RAN node receives the transport multicast address and multicast Tunnel Info of the shared delivery, it uses the transport multicast address to join the multicast transport distribution.

# * * * * 2nd change * * * *

#### 7.2.5.2	MBS session activation procedure

The following can trigger the MBS session activation procedure:

-	AF requests MB-SMF to activate the MBS session;

-	MB-UPF receives the multicast data and notifies MB-SMF.

Figure 7.2.5.2-1: MBS session activation procedure

In this procedure, steps 2 to 10 and steps 11 to 14 can be executed in parallel.

1.	The procedure may be triggered by the following events:

-	When the MB-UPF receives downlink data for a multicast MBS session, based on the instruction from the MB-SMF (as described in clause 7.2.5.3), the MB-UPF sends N4mb Notification (N4 Session ID) to the MB-SMF for indicating the arrival of DL MBS data.

-	The AF sends MBS Activation request (TMGI) to the MB-SMF directly or via NEF.

2.	MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify (MBS Session ID, multicast session activated) to SMF(s).

The SMF sets the related multicast MBS session state as "Active" state and finds out the list of UEs that joined the multicast MBS session identified by the related TMGI. If the SMF determines the user plane of the associated PDU session(s) of the UE(s) with respect to the TMGI are activated already, based on operator policy, steps 3-8a or steps 3-10 will be skipped for those UE(s).

3.	The SMF invokes Namf_MT_EnableGroupReachability Request (List of UEs, [PDU Session ID of the associated PDU Sessions], TMGI, [UE reachability Notification Address]) to AMF(s). When later UE is reachable, the UE reachability Notification Address is used by the AMF to identify and notify the related SMF. Based on operator policy, the SMF includes PDU Session ID of the associated PDU Sessions or UE reachability Notification Address.

After receiving the request, for each UE in the list, the AMF determines CM state of the UE: see steps 4 - 7.

4a.	If there are UEs involved in the multicast MBS Session and in CM-CONNECTED state, the AMF indicates those UEs to the SMF, using Namf_MT_EnableGroupReachability Response (UE list). Otherwise, the response does not include UE list.

4b. For each UE in the UE list included in step 4a, if the QoS profile(s) for associated QoS flow(s) has not yet been provided for the PDU session, the SMF invokes Namf_Communication_N1N2MessageTransfer (N2 SM information (PDU Session ID, MBS Session ID, [QoS profile(s) for associated QoS flow(s)], [mapping information between the unicast QoS flow and multicast QoS flow])) to the AMF for the UE which is identified in step 4a. The mapping information between the unicast QoS flow and multicast QoS flow are included to support the 5GC Individual MBS traffic delivery.

The procedure continues at step 9.

5.	[Conditional] If AMF determines that there are UEs in CM-IDLE state and involved in the multicast MBS Session, the AMF figures out the paging area covering all the registration areas of those UE(s), which need to be paged. The AMF sends a paging request message to the NG-RAN node(s) belonging to this Paging Area with the TMGI as the identifier to be paged if the related NG-RAN node(s) support MBS. If the NG-RAN node(s) does not support MBS, the AMF sends Paging message to the NG-RAN node(s) per UE without using the MBS Session ID as described in step 4b in clause 4.2.3.3 of TS 23.502 [6].

NOTE 1:	The details of the paging are specified by the RAN WGs.

6.	The UE(s) in CM-IDLE state sends Service Request message to the AMF, see clause 4.2.3 of TS 23.502 [6].

7a/7b.	After receiving the Service Request sent by the UE(s),

- Either based on the received PDU Session ID in step 3, the AMF identifies the related SMF and invokes Nsmf_PDUSession_UpdateSMContext request.	The procedure continues at step 9. Or,

- Based on the received UE reachability Notification Address in step 3, the AMF identifies and notifies the related SMF of the UE(s), which are reachable now and its Location Information, by using the Namf_MT_UEReachabilityInfoNotify message. In this case, it can be a separated notification or combined with step 8.

8a.	For UE(s) that do not respond to paging, the AMF informs the SMF of the paging faiure in Namf_MT_UEReachabilityInfoNotify.

8b.	For UE(s) that is indicated as reachable via the Namf_MT_UEReachabilityInfoNotify message, the SMF invokes Namf_Communication_N1N2MessageTransfer (N2 SM information) to the AMF same as described in step 4b.

9.	The AMF sends N2 request message (N2 SM information) to the RAN node.

10a.	 If the shared tunnel has not been established before, the shared tunnel is established at this step, as defined in clause 7.2.1.4. The NG-RAN configures UE with RRC messages if needed.

10b. 	Steps 8 to 12 defined in clause 7.2.1.3 are performed. If 5GC Individual MBS traffic delivery is used, the SMF configures the UPF for individual delivery and if necessary requests the MB-SMF to configure the MB-UPF to send multicast data to the UPF.

11.	If the MB-SMF finds out there are shared tunnel established, steps11-15 are performed. The MB-SMF invokes Namf_MBSCommunication_N2MessageTransfer Request (TMGI, N2 SM Information (Activation, TMGI)) to the AMF for those NG-RAN nodes, which have shared tunnel with MB-UPF. This stepmay be performed in parallel with step 2.

NOTE 2:	The messages in steps 10a, 11, and 12 are MBS-specific and it is possible that the AMF(s) in steps 10a, 11, and 12 are not associate to any UEs involved in the multicast MBS Session.

12.	The AMF sends NGAP activation request message (N2 SM Information) to the NG-RAN nodes.

13.	The NG-RAN nodes responses to AMF by NGAP activation response message. The NG-RAN nodes  establish radio resources to transmit multicast MBS session data to the UE(s). The NG-RAN shall not release the radio connection of a UE that has joined into the multicast session only because no unicast traffic is received for the UE.

14.	AMF to MB-SMF: Namf_MBSCommunication_N2MessageTransfer Response ().

15.	The MB-SMF sends N4mb Session Modification Request to the MB-UPF to forward the receiving packet. The MB-UPF responses to the MB-SMF with N4mb Session Modification Response acknowledging the MB-SMF request. See clause 4.4 of TS 23.502 [6] for more details.

#### 7.2.5.3	MBS session deactivation procedure

Figure 7.2.5.3-1: MBS session deactivation procedure.

In this procedure, steps 3 to 4 and steps 5 to 9 are executed in parallel.

1.	The procedure may be triggered by the following events:

-	When MB-UPF detects there is no data receives for the MBS Session, MB-UPF sends MB-N4 Notification (N4 Session ID) to the MB-SMF for deactivating the MBS session.

-	AF sends MBS Deactivation request (TMGI) to the MB-SMF directly or via NEF.

2.	The MB-SMF send N4mb Session Modification Request (TMGI, Buffered Downlink Traffic detection) to the MB-UPF. See clause 4.4 of TS 23.502 [6] for more details. The Buffered Downlink Traffic detection is requested by MB-SMF for next time MBS session activation. If the MBS session is to be activated via the AF request directly, this indication is not needed. The MB-SMF also indicates the MB-UPF to remove the shared tunnel(s) that are used for Individual MBS traffic delivery over N19mb interface.

MB-UPF to MB-SMF: N4mb Session Modification Response acknowledging the MB-SMF request.

3.	The MB-SMF sends Nmbsmf_MBSSsession ContextStatusNotify request (subscription correlation ID) to the SMFs.

Based on the received subscription correlation ID, the SMF sets the indicated multicast MBS session state to "Inactive" state.

‐	If the SMF finds out there are UE(s) that joined the indicated multicast MBS session and use 5GC Individual MBS traffic delivery, step 4 is performed for those UE(s).

‐	If SMF find there are no UE(s) that joined the indicated MBS session and use 5GC Individual MBS traffic delivery, no further operation for SMF is required.

4.	[Conditional] For those UE(s) that the 5GC individual delivery is used, step 3b and steps 4-8 in clause 4.3.3.2 of TS 23.502 [6] are performed to remove the associated QoS flow(s) related to the multicast MBS session.

NOTE 1:	Whether the associated QoS Flow(s) are removed from UE, NG-RAN, or only resource in NG-RAN is removed is up to implementation.

5.	If the MB-SMF finds out there are shared tunnel established over N3mb interface, the MB-SMF sends Namf_MBSCommunication_N2MessageTransfer Request (TMGI, N2 SM information (Deactivation, TMGI)) to the AMFs.

6.	The AMF sends NGAP deactivation request message (N2 SM information ()) to the NG-RAN nodes.

7.	The NG-RAN node keeps the multicast MBS session context and N3mb shared tunnel for the multicast MBS session.

If the MBS session context indicates no UE for the multicast MBS session (e.g., due to UE becomes CM-IDLE state), the NG-RAN triggers release of the shared delivery as described in clause 7.2.2.4.

8.	NG-RAN acknowledges the NGAP deactivation Response message.

9.	The AMF invokes Namf_MBSCommunication_N2MessageTransfer Response to acknowledge the service for MB-SMF.

When the MBS session is in "Inactive" state and handover procedure is triggered, it is defined in clause 7.2.3.6.

NOTE 2: 	There’s no explicit "deactivation" indication to the UE, how the UE is changed to IDLE state is defined in TS38.300 [9]

# * * * * 3rd change * * * *

### 7.2.8	Service request procedure

If the MBS session is inactive state, the UE can go to CM-IDLE state or the user plane of the associated PDU Session may be deactivated even when the UE is in CM-CONNECTED state.  When user plane of the associated PDU session is activated again, the SMF sends the MBS session information to NG-RAN during Service Request procedure as specified in TS 23.502. The MBS session information indicates that this MBS session is "Inactive" state, and:

-	For the MBS supporting NG-RAN node, the NG-RAN establish the shared tunnel with the MB-UPF as usual. However, as the MBS session is inactive state, the NG-RAN node will not allocate related radio resource.

-	For the non-MBS supporting NG-RAN node, the unicast QoS flow associated with the MBS session are not established.

# * * * * End of change * * * *