3GPP TSG-SA WG2 Meeting #148E e-meeting	S2-2108628r03

# Elbonia, November 15 – 22, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0039 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications and corrections on UAV Re-authentication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. In clause 5.2.4.1, 5.2.4.2 and 5.2.4.3, “PDU Session IP address” suggests to be replaced with “PDU Session ID” in the related figures to align with the text in step 3b of 5.2.4.2. Although, PDU Session IP address can also be used to identify the PDU session, it is more straightforward to use the PDU session ID for SMF. UAS NF can map the IP address into the PDU session ID based on the received information from Nsmf_EventExposure_Notify. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDU Session IP address is replaced with “PDU Session ID” in the related Figures of clause 5.2.4.1(step 3b), 5.2.4.2 (step 3) and 5.2.4.3(step 3b). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear to be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 5.2.4.1, 5.2.4.2, 5.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Next change (new text) *****

5.2.4.1	UAV Re-authentication procedure in 5GS

Figure 5.2.4.1-1: UAV Re-authentication procedure in 5GS

UAS NF stores the UE UUAA context after successful UUAA procedure as explained in clause 5.2.2.2 for UUAA-MM and in clause 5.2.3 for UUAA-SM procedure. The UUAA context may be stored in the UDSF or may be stored locally in the UAS NF depending on deployments.

1.	The USS sends a Naf_Auth_notification request to UAS NF for re-authentication of the UAV. The USS includes GPSI, CAA-Level UAV ID, PDU Session IP address if available in the re-authentication request and an authentication message to be transparently delivered to the UAV.

2.	UAS NF retrieves the UE stored UUAA context. From the stored UUAA context the UAS NF determines the target AMF or SMF for sending the notification.

3a or 3b.	The UAS NF sends Nnef_Auth_Notification request to notify the target NF, i.e. either the AMF or the SMF, to initiate re-authentication of the UAV.

4.	The UAS NF responds back to the USS indicating that re-authentication request has been successfully initiated

5.	If UE is in CM_Idle state, the target NF (i.e. either the AMF or the SMF) initiates the Network Triggered Service Request procedures as described in clause 4.2.3.3 of TS 23.502 [3].

6a.	If UUAA-MM was performed, the AMF initiates re-authentication of the UAV as described in steps 4c to 9 of UUAA-MM procedure, clause 5.2.2.2.

6b.	If UUAA-SM was performed, the SMF then initiates re-authentication of the UAV as described in steps 3c to 6 of the UUAA-SM procedure, clause 5.2.3.2.

*** Next change (new text) *****

5.2.4.2	UAV Re-authentication procedure in EPS

Figure 5.2.4.2-1: UAV Re-authentication procedure in EPS

UAS NF stores the UE UUAA context after successful UUAA-SM procedure as explained in clause 5.2.3. The UUAA context may be stored in the UDSF or may be stored locally in the UAS NF depending on deployments.

1.	The USS sends a request to UAS NF for re-authentication of the UAV. The USS includes GPSI, CAA-Level UAV ID, UE IP address in the re-authentication request and an authentication message to be transparently delivered to the UAV.

2.	UAS NF retrieves the UE stored UUAA context. From the stored UUAA context the UAS NF determines the target SMF+PGW-C for sending the notification.

3.	The UAS NF sends Nnef_Auth_Notification request to notify the SMF+PGW-C, to initiate re-authentication of the UAV.

4.	The UAS NF responds back to the USS indicating that re-authentication request has been successfully initiated

5.	The SMF+PGW-C then initiates re-authentication of the UAV as in steps 5b to 8 in Figure 5.2.3.3-1: UUAA during PDN connection establishment at Attach procedure in EPS.

5.2.4.3	USS initiated UAV Re-authorization procedure in 5GS

Figure 5.2.4.3-1: UAV Re-authorization procedure in 5GS

UAS NF stores the UE UUAA context after successful UUAA procedure as explained in clause 5.2.2.2 for UUAA-MM and in clause 5.2.3 for UUAA-SM procedure. The UUAA context may be stored in the UDSF or may be stored locally in the UAS NF depending on deployments.

1.	The USS sends a request to UAS NF for re-authorization of the UAV. The USS includes GPSI, CAA-Level UAV ID, an authorization message to be transparently delivered to the UAV. The CAA-Level UAV ID may be a new CAA-Level UAV ID. The authorization message may e.g. include a UUAA Authorization Payload, a C2 Authorization Result and/or (C2) Security Information.

2.	Based on the received GPSI the UAS NF retrieves the correspondingly stored UUAA context which includes information on whether a UUAA-MM or a UUAA-SM has been performed and in case of UUAA-SM the identity of the SMF and the IP address of the PDU session that is serving the UUAA.

3a	If UUAA-MM was performed, the UAS-NF/NEF retrieves the AMF ID that serves the UE from the UDM based on the GPSI. and then sends a Nnef_Auth_Notification request including the CAA-Level UAV ID and the authorization message to the serving AMF.

3b	If UUAA-SM was performed, the UAS-NF/NEF sends a Nnef_Auth_Notification request to the SMF serving the UUAA which includes the corresponding PDU session identity, CAA-Level UAV ID and the authorization message.

4.	The UAS NF responds back to the USS indicating that re-authorization request has been successfully initiated.

5a.	In the case of UUAA-MM:

If the UE is in CM_Idle state, the AMF initiates the Network Triggered Service Request procedures as described in clause 4.2.3.3 of TS 23.502 [3].

The AMF initiates re-authentication of the UAV as described in steps 4c to 9 of UUAA-MM procedure, clause 5.2.2.2. during which the CAA-Level UAV ID and the authorization message is sent to the UE.

5b	In the case of UUAA-SM:

The SMF identifies, based on the received information, the PDU Session that is serving the UUAA-SM and invokes the Network Requested PDU Session Modification procedure (figure 4.3.3.2-1 of TS 23.502 [3] triggering event SMF Requested modification) by sending Namf_Communication_N1N2MessageTransfer, including the CAA-Level UAV ID and the authorization message in the N1_SM_Container (step 3b in figure 4.3.3.2-1 of TS 23.502 [3]) .

The Network Triggered service request procedure is invoked by AMF to forward the CAA-Level UAV ID and the authorization message included in the N1_SM_container to the UE (from step 3a in figure 4.2.3.3-1 of TS 23.502 [3]).

6.	The UE receives the CAA-Level UAV ID and the authorization message (which may e.g. include a UUAA Authorization Payload, a C2 Authorization Result and/or (C2) Security Information) and act on it accordingly (outside the scope of 3GPP).

*** End of changes ***