3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108644

# Elbonia, November 15 – 19, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3411 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NSAC for priority services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is proposed to have following corrections: It is agreed that Subject to operator policy and national/regional regulations, the AMF and SMF may exempt from NSAC the network slices that the AMF has determined to include Emergency, Critical and Priority services (e.g. MCS, MPS). The AMF will know the S-NSSAI is associated with Emergency, Critical and Priority services based on Registration Type, Emergency Configuration Data or the establishment cause. However, for Invocation-related Priority Mechanisms, the AMF cannot know the PDU Session is for MPS/MCS, as only the PCF will get the indication from the AF. When the NSACF has changed during mobility (e.g. out of serving area), since the (new) NSACF does not know the UE has registered and counted before, the S-NSSAI for the MPS/MCS might be rejected due to NSAC as the AMF does not know it. Besides, to avoid no Allowed NSSAI for a UE due to NSAC, it is recommended at least one of the Subscribed S-NSSAIs is marked as the default S-NSSAI which is not subject to Network Slice Admission Control |  |  |  |  |  |  |  |  |
| Summary of change: |  | When there is MPS related PDU Session with Invocation-related Priority Mechanisms, the SMF indicates this to the AMF based on the information from the PCF. Besides, it is clarified If the use case requires the UE to remain reachable at all times with at least one slice, it is recommended that at least one of the Subscribed S-NSSAIs is marked as the default S-NSSAI which is not subject to Network Slice Admission Control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The S-NSSAI for the MPS/MCS might be rejected due to NSAC as the AMF does not know it in the Invocation-related Priority Mechanisms. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.0, 5.22.3, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is needed when there is no common solution for SC on interworking and multiple NSACFs scenarios. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.15.11	Network Slice Admission Control

5.15.11.0	General

The Network Slice Admission Control Function (NSACF) monitors and controls the number of registered UEs per network slice and the number of PDU Sessions per network slice for the network slices that are subject to Network Slice Admission Control (NSAC). The NSACF is also configured with the information indicating which access type is specified for the S-NSSAI subject to NSAC (i.e. 3GPP Access Type, Non-3GPP Access Type, or both).

The NSACF also provides event based Network Slice status notifications and reports to the consumer NF (e.g. AF).

Multiple NSACFs may be deployed in a network. One NSACF may be responsible for one or more S-NSSAIs in a service area. Within a service area, one S-NSSAI is handled by one NSACF, which can be one NSACF instance or one NSACF Set. A PLMN may have one or more service areas.

Each NSACF is configured with the maximum number of UEs per network slice and the maximum number of PDU Sessions per network slice which are allowed to be served by each network slice that is subject to NSAC.

Subject to operator policy and national/regional regulations, the AMF may exempt from NSAC the network slices that the AMF has determined to include Emergency, Critical and Priority services (e.g. MCS, MPS). When the Registration Type indicates Emergency Registration or the establishment cause is associated with priority services (e.g. MPS, MCS), for the S-NSSAI to be subject to NSAC, or has been included in the Allowed NSSAI and is subject to NSAC, if the S-NSSAI is included in the Emergency Configuration Data, the AMF decides that the S-NSSAI may be exempt from NSAC.When the emergency PDU Session is requested to be established or a priority header is received from the AMF during PDU Session establishment, if the S-NSSAI is subject to NSAC, the SMF decides that NSAC may be skipped for the S-NSSAI. When NSAC is exempted for the S-NSSAI, the AMF and the SMF skip the corresponding NSAC procedure for the S-NSSAI, i.e. this UE (respectively PDU Session) is not counted towards the maximum number of UEs (respectively PDU Sessions). The SMF may exempt from NSAC the network slices When the PCF indicates one of the PDU Sessions is associated with priority services (e.g. MPS, MCX);

NOTE x: The SMF can identify that the S-NSSAI can be used by this PDU Session for MPS purposes based on the indication from the PCF, so that the SMF will not execute the NSAC procedure in the interworking (e.g. when EPS counting is not required) to avoid reject the S-NSSAI used for priority services (e.g. MPS, MCX) with Invocation-related Priority Mechanisms.

NOTE y: If the use case requires the UE to remain reachable at all times with at least one slice, it is recommended that at least one of the Subscribed S-NSSAIs is marked as the default S-NSSAI which is not subject to Network Slice Admission Control.

# * * * * Next change * * * *

5.22.3	Invocation-related Priority Mechanisms

The generic mechanisms used based on invocation-related Priority Mechanisms for prioritised services are based an interaction with an Application Function and between the Application Function and the PCF over Rx/N5 interface.

These mechanisms apply to mobile originated as well as mobile terminated SIP call/sessions (clause 5.21 of TS 23.228 [15]) and Priority PDU connectivity services including MPS for Data Transport Service.

NOTE 1:	Clause 5.21 of TS 23.228 [15] is applicable to 5GS, with the understanding that the term PCRF corresponds to PCF in the 5GS.

Invocation-related mechanisms for Mobile Originations e.g. via SIP/IMS:

-	PCF: When an indication for a session arrives over the Rx/N5 Interface and the UE does not have priority for the signalling QoS Flow, the PCF derives the ARP and 5QI parameters, plus associated QoS characteristics as appropriate, of the QoS Flow for Signalling as per Service Provider policy as specified in clause 6.1.3.11 of TS 23.503 [45]. The PCF may further provide priority indication for the PDU Session to SMF to exempt the NSAC as described in clause 5.15.11.

-	PCF: For sessions such as MPS, when establishing or modifying a QoS Flow for media as part of the session origination procedure, the PCF selects the ARP and 5QI parameters, plus associated QoS characteristics as appropriate, to provide priority treatment to the QoS Flow(s).

-	PCF: When all active sessions to a particular DN are released, and the UE is not configured for priority treatment to that particular PDU Session for a DN, the PCF will downgrade the IMS Signalling QoS Flows from appropriate settings of the ARP and 5QI parameters, plus associated QoS characteristics as appropriate, to those entitled by the UE based on subscription.

Invocation-related mechanisms for Mobile Terminations e.g. via SIP/IMS:

-	PCF: When an indication for a session arrives over the Rx/N5 Interface, mechanisms as described above for Mobile Originations are applied.

-	UPF: If an IP packet arrives at the UPF for a UE that is CM-IDLE, the UPF sends a "Data Notification" including the information to identify the QoS Flow for the DL data packet to the SMF, as specified in clause 4.2.3.3 of TS 23.502 [3].

-	SMF: If a " Data Notification" message arrives at the SMF for a QoS Flow associated with an ARP priority level value that is entitled for priority use, delivery of priority indication during the Paging procedure is provided by inclusion of the ARP in the N11 interface "N1N2MessageTransfer" message, as specified in clause 4.2.3.3 of TS 23.502 [3].

-	AMF: If an "N1N2MessageTransfer" message arrives at the AMF containing an ARP priority level value that is entitled for priority use, the AMF handles the request with priority and includes the "Paging Priority" IE in the N2 "Paging" message set to a value assigned to indicate that there is an IP packet at the UPF entitled to priority treatment, as specified in clause 4.2.3.3 of TS 23.502 [3].

-	SMF: For a UE that is not configured for priority treatment, upon receiving the "N7 Session Management Policy Modification" message from the PCF with an ARP priority level that is entitled for priority use, the SMF sends an "N1N2MessageTransfer" to update the ARP for the Signalling QoS Flows, as specified in clause 4.3.3.2 of TS 23.502 [3].

-	AMF: Upon receiving the "N1N2MessageTransfer" message from the SMF with an ARP priority level that is entitled for priority use, the AMF updates the ARP for the Signalling QoS Flows, as specified in clause 4.3.3.2 of TS 23.502 [3].

-	(R)AN: Inclusion of the "Paging Priority" in the N2 "Paging" message triggers priority handling of paging in times of congestion at the (R)AN as specified in clause 4.2.3.3 of TS 23.502 [3].

Invocation-related mechanisms for the Priority PDU connectivity services:

-	PCF: If the state of the Priority PDU connectivity services is modified from disabled to enabled, the QoS Flow(s) controlled by the Priority PDU connectivity services are established/modified to have the service appropriate settings of the ARP and 5QI parameters, plus associated QoS characteristics as appropriate, using the PDU Session Modification procedure as specified in clause 4.3.3 of TS 23.502 [3]. The PCF may further provide priority indication for the PDU Session to SMF to exempt the NSAC as described in clause 5.15.11.

-	PCF: If the state of Priority PDU connectivity services is modified from enabled to disabled, the QoS Flow(s) controlled by the Priority PDU connectivity services are modified from Priority PDU connectivity service appropriate settings of the ARP and 5QI parameters, plus associated QoS characteristics as appropriate, to those entitled by the UE as per subscription, using the PDU Session Modification procedure as specified in clause 4.3.3 of TS 23.502 [3].

Invocation-related mechanisms for MPS for Data Transport Service:

-	MPS for Data Transport Service follows the same steps as those for Priority PDU connectivity services. The QoS Flows that will be subject to MPS for Data Transport Service are based on operator policy and regulations by means of local PCF configuration.

NOTE 2:	If no configuration is provided, MPS for Data Transport Service applies to the QoS Flow associated with the default QoS rule.

# * * * * End of changes * * * *