3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108672

# Elbonia, November 15 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0066 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Annex A for transport mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current spec, we demonstrate the several configuration options for MBS the deployment in Annex A. - For Configuration Option 1, it is assumed to be used for "Transport Only Mode, when the Multicast service or Broadcast service does not require service layer interworking with LTE MBMS". - For Configuration Option 2, it is assumed to be used for "Service Mode, or when interworking with LTE MBMS is required". It is proposed to further clarify those aspects since: - Current definition of "Transport Only Mode" and "Service Mode" are not clear, suggest to add some explanations after the related modes; - It is not clear that for configuration option 2, when interworking with LTE MBMS in service layer is required, whether the same mode (i.e., transport only mode or service mode) will be used in MBMS or 5MBS. From the AF implementation perspective, using the same mode can simplify the AF realization and the interaction between AF and PLMN network. And from the service experience perspective, using different modes will cause degraded service experience. Therefore it is suggested to clarify such aspect in current specification. - Nmb8 and Nmb10 are provided by MBSF/MBSTF, however for IWK and when transport only mode is used, there would be nothing more than N6mb and N33. It is proposed to clarify these aspect for the sake of public safety usage. It is also noted that in section 5.1, we have N29mb and N30 for the case of NEF is involved, while for supporting the case without NEF, we only have Nmb13, this issue needs to be fixed in this meeting as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the Transport only mode and service mode mentioned in Annex A - Clarify the same mode will be used for service layer interworking. - Clarify that if transport only mode is used and when Multicast service or Broadcast service requires service layer interworking with LTE MBMS, Nmb8 and Nmb10 provides similar functionalities to N6mb and N33, respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear current specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex A (normative):
Configuration options at Service and/or Application for MBS

Figure A-1 provides the reference architecture with all configuration variants for Application Function interaction with 5G Core Network, usage of NEF or MBSF in the control plane, and usage of N6, MB2-U or xMB-U in user plane.

Figure A-1: Configuration options at Service and/or Application

The following characteristics describe each of the Configuration options:

-	Configuration Option 1: No MBSF:

-	This configuration is used for Transport Only Mode (i.e., MBS provides only transport of MBS data in a transparent manner without modifying the data), when the Multicast service or Broadcast service does not require service layer interworking with LTE MBMS.

-	The control plane entry point for the Application Function outside the trusted domain towards 5GC to request establishment of an MBS session is the NEF via N33.

-	An application function within the trusted domain can directly use the N30 and N29mb service based interfaces. In this case some NEF functionality related to PCF and MB-SMF interaction is incorporated in AF.

NOTE 1:	Application function within the trusted domain selects MB-SMF based on e.g. its local configuration, or query NRF based on location, etc.

-	The user plane entry point for the Application Function towards 5GC is the MB-UPF via N6mb.

-	Configuration Option 2: MBSF, N33 towards AF:

-	This configuration may be used for Service Mode (i.e., MBS provides service layer capability, allows to modify the data), or when service layer interworking with LTE MBMS is required.

-	When interworking with LTE MBMS for Transport Only Mode services, N33/Nmb5 and Nmb8 provide functionalities as N33/N13mb and N6mb respectively.

-	The control plane entry point from the Application Function to request establishment of an MBS session is the NEF via N33.

-	The user plane entry point for the Application Function towards 5GC is the MBSTF via MB2-U, xMB-U or Nmb8.

-	The NEF and MBSF may be collocated.

-	If the MBSF is not collocated with the NEF, the reference point between the NEF and MBSF is Nmb5.

-	An application function within the trusted domain may be collocated with MBSF.

-	Configuration Option 3: MBSF, MB2-C/xMB-C/Nmb10 towards AF:

-	This configuration may be used for Service Mode (xMB-C or Nmb10), or when interworking with LTE MBMS is required (MB2-C, xMB-C or Nmb10).

-	When interworking with LTE MBMS for Transport Only Mode services, Nmb10 and Nmb8 provide functionalities as Nmb13 and N6mb respectively.

-	The control plane entry point from the Application Function to request establishment of an MBS session is the MBSF via MB2-C, xMB-C or Nmb10.

-	The user plane entry point for the Application Function is the MBSTF via MB2-U, xMB-U or Nmb8.

For service mode, MBSF shall be used, i.e. either Configuration 2 or Configuration 3 shall be used. In this case, the MBSF instructs the MBSTF to modify the MBS data (e.g. including the FEC or MBS data transcoding).

For Configuration Option 2 and Option 3, when interworking with LTE MBMS is used, LTE MBMS and MBS use the same mode (i.e., both use Transport Only Mode or Service Mode).

For Transport Only mode:

-	If interworking with LTE MBMS at 5GC is required for the service, MBSF and MBSTF shall be used, i.e. either Configuration 2 or Configuration 3 shall be used.

NOTE 2:	Interworking providing by AF is out of scope of this specification.

-	If interworking with LTE MBMS at 5GC is not required for the service, MBSF and MBSTF are optional.

NOTE 2:	Interworking providing by AF is out of scope of this specification.

MBSTF shall be used when MBSF is used.

Any particular deployment may support any combination of these configurations.

# * * * * End of changes * * * *