SA WG2 Meeting #148E	S2-2108675r02

# Nov 15 - 22, 2021, Electronic

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0042 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Local NEF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-11- 8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clarify how the NEF can discover a local NEF if the AF request for local QoS monitoring exposure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify how the NEF can discover a local NEF if the AF request for local QoS monitoring exposure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how AF can find the local NEF to get result of local QoS monitoring. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

6.4.2.2	Local NEF Discovery

As specified in TS 23.501 [2] clause 6.2.5.0, the NRF may be used by the AF to discover the L-NEF. To become discoverable, the L-NEF registers with an NRF deployed within the operator's domain where the AF resides.

The AF uses existing procedures as described in TS 23.502 [3], clause 4.17.4 to discover the L-NEF.

If the AF only knows the NEF and AF request for local QoS monitoring exposure, the AF should invoke Nnef_AFsessionWithQoS_Create with providing an indication for local QoS monitoring exposure, may also include DNAI or Potential Locations of Applications.If initial NEF gets the indication for local QoS monitoring exposure, as well as DNAI or Potential Locations of Applications, this NEF may map the Potential Locations of Applications into AF location (e.g. geographical location, data centre) based on local configuration. The initial NEF can find the target local NEF by querying NRF with the DNAI or AF location.

If AF cannot provide DNAI or Potential Locations of Applications, the NEF may invoke the Npcf_PolicyAuthorization service to the PCF and SMF(clause 4.3.6.4 in TS 23.502) to get the local PSA UPF location. Then the NEF can find the local NEF through NRF with the local PSA UPF location.

The initial NEF may response with local NEF address/FQDN to the AF. Then the AF may send the local QoS monitoring exposure to local NEF directly.

NOTE: the AF request can be redirected to local NEF through http redirection which has been defined in TS 29.500.

* * * End of changes* * *