SA WG2 Meeting #148E	S2-2108675r07

# Nov 15 - 22, 2021, Electronic

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0042 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Local NEF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-11- 8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clarify how the NEF can discover a local NEF if the AF request for local QoS monitoring exposure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify how the NEF can discover a local NEF if the AF request for local QoS monitoring exposure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how AF can find the local NEF to get result of local QoS monitoring. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

#### 6.4.2.2	Local NEF Discovery

As specified in TS 23.501 [2] clause 6.2.5.0, the NRF may be used by the AF to discover the L-NEF. To become discoverable, the L-NEF registers with an NRF deployed within the operator's domain where the AF resides.

The AF uses existing procedures as described in TS 23.502 [3], clause 4.17.4 to discover the L-NEF.

If the AF only knows the NEF and it initiates a service operation for local QoS monitoring exposure towards this NEF, e.g. a Nnef_AFSessionWithQoS_Create/Update_request procedure with an indication of local event notification as described in clause 6.4.2.1 and clause 6.1.3.21 of TS 23.503[4], the NEF may decide that itself is not suitable for local exposure (and re-direct the request to an L-NEF as described in TS 29.500 [9]. NEF may use NRF to find a suitable L-NEF for the re-direction.

For an internal AF, It is assumed the NEF can know the DNAI of the AF when the AF initiates a service operation for local QoS monitoring exposure towards the NEF. The DNAI can be used for local NEF selection.

For an external AF, the initial NEF first determines the target (DNN, S-NSSAI) and based on the UE address and the target (DNN, S-NSSAI) gets the SMF ID of the SMF serving the PDU Session. Then the initial NEF initiates Nsmf_EventExposure_Subscribe to this SMF to get the locality and the instance ID of the Local PSA (L-PSA) and the DNAI

NOTE: The initial NEF may cache results per DNAI to be used for later L-NEF discoveries.

* * * End of changes* * *