SA WG2 Meeting #148E (e-meeting)	S2-2108710

# 15-22 November, 2021, Electronic, Elbonia							            (revision of S2-2108101)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3274 | rev | 2 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNPN corrections to clarify use of DNN and S-NSSAI for onboarding, AUSF discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson, Samsung, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Current wording in specifications “DNN and S-NSSAI used for onboarding” is unclear as it does not exclude using this DNN and S-NSSAI also for other purposes or use an existing DNN and S-NSSAI also for onboarding. In case existing DNN and S-NSSAI are used for onboarding in a SNPN, SMF and PCF are not able to identify whether a PDU session is meant to be used for remote provisioning or for other services and cannnot apply onboarding configuration data. - Current spec states that the Routing ID can used along with HNI to discover and select AUSF. It needs to be clear that the section 5.30.2.3 is referenced for completeness – otherwise, there is a risk one can use Routing ID even when the assumptions in section 5.30.2.3 are not true. - When Onboarding Network is a PLMN, a default subscription is used for onboarding. To avoid that the network provides PVS addresses to the UE within PCO for all PDU sessions, the UE should request PVS address information from the network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Onboarding network is a SNPN: Introduce an onboarding indication which the AMF may provide to the SMF together with DNN/S-NSSAI and the SMF to the PCF, when a PDU Session for remote provisioning is established. - Refer to 5.30.2.3 for UE configuration assumption. - Onboarding network is a PLMN: Clarify that UE may request PVS address information via PCO for the subscribed DNN/S-NSSAI used for onboarding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear stage 2 specification as specification text is misleading for implementation and deployment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.9.2, 5.30.2.10.1, 5.30.2.10.2.6, 5.30.2.10.4.3, 5.30.2.10.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2: Introduce onboarding indication from AMF to SMF and from SMF to PCF when a PDU Session for remote provisioning is established. Clarify that UE may request PVS address information via PCO for the subscribed DNN/S-NSSAI used for onboarding when Onboarding Network is a PLMN. |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

5.30.2.9.2	Credentials Holder using AAA Server for primary authentication and authorization

The AUSF in SNPN may support primary authentication and authorization of UEs that use credentials from an AAA Server in a Credentials Holder (CH).

-	If the UDM decides that the primary authentication is performed by AAA Server in CH based on UE subscription data and UE's SUPI, which is de-concealed by UDM from the SUCI received from AUSF, then the UDM instructs AUSF that primary authentication by a AAA server in a CH is required, the AUSF shall discover and select the NSSAAF, and then forward EAP messages to the NSSAAF. The NSSAAF selects AAA Server based on the domain name corresponds to the realm part of the SUPI, relays EAP messages between AUSF and AAA Server (or AAA proxy) and performs related protocol conversion. The AAA server acts as the EAP Server for the purpose of primary authentication.

-	The SUPI is used to identify the UE during primary authentication and authorization towards the AAA sever. SUPI privacy is achieved according to existing methods in Annex I.5 of TS 33.501 [29].

-	The AMF discovers and selects the AUSF as described in clause 6.3.4 using the Home Network Identifier (realm part) and Routing Indicator present in the SUCI provided by a UE configured as described in clause 5.30.2.3.

-	The AMF and SMF shall retrieve the UE subscription data from UDM using SUPI.

Figure 5.30.2.9.2-1 depicts the 5G System architecture for SNPN with Credentials Holder using AAA Server for primary authentication and authorization.

Figure 5.30.2.9.2-1: 5G System architecture with access to SNPN using credentials from Credentials Holder using AAA Server

* * * * Next change * * * *

5.30.2.10.1	General

Onboarding of UEs for SNPNs allows the UE to access an Onboarding Network (ONN) based on Default UE credentials for the purpose of provisioning the UE with SNPN credentials for primary authentication and other information to enable access to a desired SNPN, i.e. (re-)select and (re-)register with SNPN.

To provision SNPN credentials in a UE that is configured with Default UE credentials (see clause 5.30.2.10.2.4), the UE selects an SNPN as ONN and establishes a secure connection with that SNPN referred to as Onboarding SNPN (ON-SNPN), see more details in clause 5.30.2.10.2.

NOTE:	If the UE is already provisioned with a set of CH credentials and needs to be provisioned with an additional set of SNPN credentials, the UE can request PVS address information and leverage the User Plane connection enabled by the available set of CH credentials to get access to a PVS.

To provision SNPN credentials in a UE that is equipped with a USIM configured with PLMN credentials, the UE selects a PLMN as ONN and establishes a secure connection with that PLMN, see more details in clause 5.30.2.10.3.

After the secure connection is established, the UE is provisioned with SNPN credentials and possibly other data to enable discovery, (re-)selection and (re-)registration for a desired SNPN, see more details in clause 5.30.2.10.4.

ON-SNPN and SO-SNPN can be roles taken by either an SNPN or different SNPNs. It is possible for the same network to be in both roles with respect to a specific UE.

* * * * Next change * * * *

5.30.2.10.2.6	Registration for UE onboarding

When the user or UE has selected an ON-SNPN according to clause 5.30.2.10.2.5, the UE establishes an RRC connection towards the NG-RAN node of the ON-SNPN. The UE provides an indication in RRC Connection Establishment that the RRC connection is for onboarding as defined in TS 38.331 [28]. This indication allows the NG-RAN node to select an appropriate AMF that supports the UE onboarding procedures. The UE indicates the ON-SNPN as the selected network, and the NG-RAN node shall indicate the selected PLMN ID and NID of the ON-SNPN to the AMF.

NOTE:	As the configuration information in the UE does not include any S-NSSAI and DNN used for onboarding, the UE does not include S-NSSAI and DNN in RRC when it registers for UE onboarding purposes to the ONN.

The UE shall initiate the NAS registration procedure by sending a NAS Registration Request message with the following characteristics:

-	The UE shall set the 5GS Registration Type to the value "SNPN Onboarding" indicating that the registration request is for onboarding.

-	The UE shall provide a SUCI derived from a SUPI as specified in TS 23.003 [19] and TS 33.501 [29]. The SUPI shall uniquely identify the UE and shall be derived from the Default UE Credentials. The ON-SNPN may determine the corresponding DCS identity or address/domain, based on the SUCI (i.e. based on the Home Network Identifier of the SUCI).

The UE does not include a Requested NSSAI in NAS signalling when it registers for UE onboarding purposes to the ONN.

The AMF supporting UE onboarding is configured with AMF Onboarding Configuration Data that may include e.g.:

-	S-NSSAI and DNN to be used for UE onboarding or a configured SMF for the DNN and S-NSSAI used for UE onboarding.

-	Information to enable User Plane Remote Provisioning of UEs in SNPNs, see clause 5.30.2.10.4.

-	Information to use a local AUSF(s) within the ON-SNPN for onboarding of UEs with a SUCI for a DCS with AAA server.

When the AMF receives a NAS Registration Request with a 5GS Registration Type set to "SNPN Onboarding", the AMF:

-	starts an authentication procedure towards the AUSF, the authentication procedure is specified in TS 33.501 [29]. The AMF selects an appropriate AUSF as described in clause 6.3.4 based on the Home Network Identifier of the SUCI used during onboarding.

-	applies the AMF Onboarding Configuration Data e.g. used to restrict UE network usage to only onboarding for user plane remote provisioning of UE as described in clause 5.30.2.10.4.3.

-	stores in the UE context in AMF an indication that the UE is registered for SNPN onboarding.

Upon successful authentication from AUSF, the AMF informs the UE about the result of the registration. If the UE is not successfully authenticated, the AMF shall reject the registration procedure for onboarding, and the UE may select a different ON-SNPN to attempt to register.

* * * * Next change * * * *

5.30.2.10.4.3	User Plane Remote Provisioning of UEs when Onboarding Network is an ON-SNPN

If Onboarding Services are provided using a restricted PDU Session for remote provisioning of UE via User Plane, the AMF selects an SMF used for Onboarding Services using the SMF discovery and selection functionality as described in clause 6.3.2. The AMF Onboarding Configuration Data may contain an S-NSSAI and a DNN used for Onboarding to select an SMF used for Onboarding Services or may contain a configured SMF for the DNN used for Onboarding. The AMF provides Onboarding Indication to SMF via Nsmf_PDUSession_CreateSMContext request message when a PDU Session for remote provisioning of UE via User Plane is established. If PCF is used for UP Remote Provisioning, SMF provides Onboarding Indication to PCF when requesting an SM Policy Association.

When a UPF is selected for Onboarding Services, the UPF selection function described in clause 6.3.3 for normal services is applied considering the DNN used for Onboarding.

The SMF or the PCF may store S-NSSAI and DNN information used for Onboarding. Onboarding Configuration Data available to PCF (for details see TS 23.503 [45]) and/or SMF may include PVS FQDN and PVS IP address(es). The SMF and the PCF may use Onboarding Indication and DNN/S-NSSAI used for Onboarding to access Onboarding Configuration Data.

When the UE registered for Onboarding successfully completes the user plane remote provisioning of SNPN credentials via the Onboarding Network, then the UE should deregister from the Onboarding Network.

Initial QoS parameters used for establishing Onboarding Services are configured in the SMF when dynamic PCC is not used.

Dynamic PCC may be used for a PDU session that is established for Onboarding Services as described in TS 23.503 [45]).

The QoS Flows of a PDU Session associated with the restricted DNN shall be dedicated to Onboarding Services. The SMF may configure PDR and FAR including PVS and DNS server IP addresses for the UPF to block any traffic that is not from or to PVS and DNS server addresses.

If the UE is registered for Onboarding, the network should apply S-NSSAI and DNN in the Onboarding Configuration Data for the PDU Session Establishment request from the UE.

* * * * Next change * * * *

5.30.2.10.4.4	User Plane Remote Provisioning of UEs when Onboarding Network is a PLMN

Onboarding Services for a UE may be provided via PLMN using PDU Session for DNN(s) and S-NSSAI(s) used for onboarding. Subscription data of such a UE shall contain the DNN and S-NSSAI used for onboarding.

The AMF selects an SMF used for remote provisioning using the SMF discovery and selection functionality as described in clause 6.3.2, considering the DNN and S-NSSAI used for onboarding provided by the UE or the default DNN and S-NSSAI provided by UDM.

The UPF selection function described in clause 6.3.3 is applied, considering the DNN and S-NSSAI used for onboarding.

The SMF may be configured with one or more PVS FQDN and/or PVS IP address(es) per DNN and S-NSSAI used for onboarding. When the UE requests a PDU Session for DNN and S-NSSAI used for onboarding, the SMF sends the PVS FQDN and/or PVS IP address(es) associated to the DNN and S-NSSAI of the PDU Session to the UE as part of Protocol Configuration Options (PCO) in the PDU Session Establishment Response if the following conditions are met:

-	the UE subscription data contains the DNN and S-NSSAI used for onboarding, and

-	the SMF has obtained the PVS FQDN and/or PVS IP address(es) from local configuration or from the AMF, and

-	the UE has indicated in the PDU Session Establishment Request that it requests PVS address information.

* * * * End of changes * * * *