3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2108717r02

# Elbonia, November 15 – 19, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3277 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | I-SMF removal triggered by removal of target DNAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current TS has supported 2 scenarios for PCF triggered I-SMF change: - SMF determines target DNAI based on received DNAI(s) from PCF and the SMF can’t serve the target DNAI, then an I-SMF will be inserted. - An I-SMF is used in the PDU Session. SMF determines target DNAI based on received DNAI(s) from PCF and the SMF can serve the target DNAI by itself, then the existing I-SMF can be removed. There can be another case that I-SMF should be removed which was not considered in the specification yet: Assuming an I-SMF is used in the PDU Session triggered by DNAI(s) from a PCC rule, then the PCF generates a new PCC rule with the DNAI removed (e.g. due to an application server was removed from the Local DN). In this case, the I-SMF (and corresponding ULCL/BP) should also be removed. This proposal proposes changes to address scenario3: - When SMF receives updated PCC rule(s) from PCF which removes the provided DNAI(s), the SMF determines no target DNAI should be used. If the existing I-SMF is not needed, the SMF sends the target DNAI information without including target DNAI to the AMF to trigger I-SMF removal. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To support I-SMF removal triggered by removal of DNAI in the PCC rules: - SMF decides no target DNAI should be used based on received PCC rule(s) from PCF. - SMF sends target DNAI information without including target DNAI to AMF. - If the AMF receive target DNAI information without including target DNAI, the AMF may remove the existing I-SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | I-SMF can not be removed even in case the target DNAI is not available anymore after the PCC rules are updated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.23.5.4	SMF triggered I-SMF selection or removal

Figure 4.23.5.4-1: SMF triggered I-SMF selection or removal

1.	The PCF sends to the SMF PCC rule(s) which may include DNAI(s) for the PDU sessions by invoking Npcf_SMPolicyControl_UpdateNotify service operation.

Based on the received DNAI(s), the SMF may subscribe to UE mobility event notification from the AMF (e.g. UE moves into or out of Area of Interest corresponding to the received DNAI(s)).

The SMF determines the target DNAI(s) which are applicable to the current UE location. Then the SMF may decide the target DNAI finally.

SMF may decide no target DNAI should be used based on PCC rule(s), e.g. when the updated PCC rules removes previously provided DNAI(s).

2.	The SMF invokes a Nsmf_PDUSession_SMContextStatusNotify service operation (or Nsmf_PDUSession_StatusNotify) in the following cases:

-	if the SMF (or the associated old I-SMF) cannot serve the target DNAI, or

-	if an I-SMF is used for the PDU Session, and the SMF decides no target DNAI should be used for the PDU Session anymore hence the existing I-SMF is not needed, or

- 	if an I-SMF is used for the PDU Session, and the SMF decides that the SMF itself can serve the target DNAI hence the existing I-SMF is not needed.

The content of the message includes the target DNAI information. The target DNAI information  indicates that the I-SMF selection is expected, and may or may not includes a target DNAI. This is to trigger the AMF to (re)select a suitable I-SMF, or remove the existing I-SMF (if the AMF decides that the SMF can serve the Target DNAI, or the AMF receive a target DNAI information without including target DNAI) for the PDU Session. The target DNAI is used for selecting (I-)SMF, which controls UPF connecting to that DNAI.

If there is an I-SMF serving the PDU session, the SMF invokes Nsmf_PDUSession_StatusNotify and then the I-SMF invokes Nsmf_PDUSession_SMContextStatusNotify message to send the target DNAI information for existing PDU session to AMF.

3.	If the I-SMF selection is expected, the AMF selects a new I-SMF which can serve the target DNAI or remove the existing I-SMF (if the AMF decides that the SMF can serve the Target DNAI, or if the AMF receive a target DNAI information without including target DNAI) for the existing PDU Session as described in clause 5.34.3 of TS 23.501 [2].

4.	The AMF sends a Nsmf_PDUSession_CreateSMContext Request to the new I-SMF as described in step 3 of clause 4.23.4.3, or to the SMF as described in step 10 of clause 4.23.4.3, with the following enhancement:

The AMF includes target DNAI received from SMF in the message. When the UE is in CONNECTED state the AMF also include indication of no NG-RAN change.

5.	The procedure described in clauses 4.23.4.3 (case: I-SMF insertion or I-SMF change) starting from step 4 takes place with the following difference:

In step 4a, the (target) new I-SMF sends the indication of no NG-RAN change to the old I-SMF or SMF as it is received from AMF.

In step 4b, when the old I-SMF or SMF receives indication of no NG-RAN change it include the additional Downlink Tunnel Info of NG-RAN in the SM context of the PDU Session.

In step 5, the I-SMF selects the I-UPF based on target DNAI.

In step 6, the target I-SMF should reuse the downlink Tunnel Info of serving RAN node if received from old I-SMF/SMF as described in clause 4.23.4.3.

In step 9, if the new I-SMF receives the Downlink Tunnel Info of NG-RAN, the N2 SM information includes PDU Session Resource Modification message.

The procedure described in clause 4.23.4.3 (case: I-SMF removal) starting from step 11 takes place with the following difference:

In step 11a, the SMF sends an indication of no NG-RAN change to the old I-SMF as it received from AMF.

In step 11b, when the old I-SMF receives indication of no NG-RAN change it include the additional Downlink Tunnel Info of NG-RAN in the SM context of the PDU Session.

In step 12, the SMF selects a new I-UPF based on target DNAI.

In step 16, if the SMF receives the Downlink Tunnel Info of NG-RAN, the N2 SM information includes PDU Session Resource Modification message.

If the UE is in IDLE state the step 17-21 are skipped. Steps 17a and 17b are still performed to release the old I-SMF.

6.	[Conditional] For the case of I-SMF insertion or I-SMF change, the PSA and UL CL/BP controlled by I-SMF is inserted as described from step 2 to 11 in figure 4.23.9.1-1 with the following difference:

In step 2, the I-SMF selects a new PDU Session Anchor (PSA2) based on the target DNAI received in step 4.

For the case of I-SMF removal, the PSA and UL CL/BP controlled by SMF is inserted as described from step 2 to 8 in figure 4.3.5.4-1 with the following difference:

In step 2, the SMF selects a new PDU Session Anchor (PSA2) based on the target DNAI if received in step 4.

# * * * * End of changes * * * *