3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108721r02

# Elbonia, November 15 – 19, 2021	(revision of S2-21XXXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3278 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for network slice notification procedure for untrusted AF case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In SA2#147E, a CR (S2-2107114) was discussed and one aspect to check is whether the AF considered to be untrusted by the operator (untrusted AF) can be assumed to be able to provide the S-NSSAI when the untrusted AF wants to subscribe or unsubscribe for the number of UEs or the number of PDU Sessions per network slice notification with the NEF. Considering the security aspect, in case of 3rd party AF or untrusted AF, it is normal that such AF should not know about the S-NSSAI. But it is supposed that such AF will provide an external parameter, which would then be translated or mapped into the internal parameter. For example, in TS 33.501, the untrusted AF sends the External Network Slice Information (ENSI) to the NEF, and the NEF translates it further to S-NSSAI. Alternatively, the AF-Service-Identifier may also be used for such translation/mapping to the S-NSSAI as specified in clause 6.2.5.0 in TS23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies that external parameter (e.g., AF-Service-Identifier or ENSI) should be used for translation/mapping to the S-NSSAI, when the untrusted AF wants to subscribe or unsubscribe for the number of UEs or the number of PDU Sessions per network slice notification with the NEF. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear on how the untrusted AF would communicate with NEF for subscribing for the number of UEs or the number of PDU Sessions per network slice notification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

##### 4.15.3.2.10	Number of UEs and PDU Sessions per network slice notification procedure

Figure 4.15.3.2.10-1: Number of UEs and PDU Sessions per network slice notification procedure

1.	To subscribe or unsubscribe for the number of UEs or the number of PDU Sessions per network slice notification with the NSACF, the AF sends Nnef_EventExposure_Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information) message to the NEF. The Event ID parameter defines the subscribed event ID, i.e. Number of Registered UEs or Number of Established PDU Sessions. The Event Filter parameter defines the S-NSSAI for which reporting is required. If the AF is considered not to be trusted by the network operator, the AF may include in the Event Filter parameter the AF-Service-Identifier as defined in TS 29.522. The AF-Service-Identifier is used by the NEF to translate it to the S-NSSAI. The Event Reporting information parameter defines the mode of reporting, i.e. threshold based reporting with included a threshold value or periodic reporting with included periodicity time interval.

Notifications related to the threshold based subscriptions behave as follows:

-	A single notification is sent only when the number of registered UEs or the number of established PDU Sessions reaches the threshold. A single notification is sent every time there is a change from being below the threshold to reach the threshold.

-	A single notification is sent only once when the number of registered UEs or the number of established PDU Sessions go below the threshold after reaching it. A single notification is sent every time there is a change from reaching the threshold to coming down below the threshold.

2.	The NEF checks whether the AF is authorised for the requested subscription. If authorised, the NEF may query the NRF to find the NSACF responsible for the requested S-NSSAI. The NEF forwards the request to the NSACF with Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information).

Editor's note:	It is FFS the NSACF responsible for an S-NSSAI to invoke the current number of UEs or PDU Sessions from all NSACFs which are currently handling the NSAC of the S-NSSAI if the NSAC is controlled by several different NSACFs.

3.	The NSACF confirms with Nnsacf_SliceEventExposure_Subscribe/Usubscribe Response message to the NEF.

4.	The NEF forwards the response from NSACF via the Nnef_EventExposure_Subscribe/Unsibscribe Response message to the AF.

5.	When the reporting condition for a subscribed event is fulfilled, the NSACF triggers a notification towards the AF.

6.	The NSACF sends the Nnsacf_SliceEvent Exposure_Notify (Event ID, Event Filter, Event Reporting information) message to the NEF. If the subscription is for event based notification (e.g. based on the monitored event reaching a threshold value), the Event Reporting information parameter contains confirmation for the event fulfilment. If the subscription is for periodic notification, the Event Reporting information parameter provides information for the current number of UEs registered with a network slice (e.g. represented in percentage of the maximum number of the UEs registered with the network slice), information for the current number of PDU Sessions on a network slice (e.g. represented in percentage of the maximum number of the UEs established on the network slice) or both.

7.	The NEF forwards the message to the AF in the Nnef_EventExposure_Notify (Event ID, Event Filter, Event Reporting information) message.

A notification can relate either to the "Number of Registered UEs" event or the "Number of Established PDU Sessions" event.

##### 4

# * * * * End of changes * * * *