3GPP TSG-SA2 Meeting # 148-E	S2-2108847

# Elbonia, 15 November 2021 - 19 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3438 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple PTP instances supported in 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIOT |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the TS, there are a lot of functional description on the PTP instance, but the functional description is only on the NW-TT/DS-TT instead of on the PTP instance level. For supporting time synchronization service, the 5GS can be configured to operate in one or multiple PTP instances and to operate in different mode or different types(i.e. PTP relay instance, Boundary Clock, end-to-end Transparent Clock, peer-to-peer Transparent Clock) for each PTP instance in clause 5.27.1.1 and 5.27.1.4. The time aware system includes PTP relay instance and PTP end instance. Since only Boundary Clock instead of Ordinary Clock is together used with the time aware system, the time aware system needs to be replaced with PTP relay instance in clause 5.27 and its subclauses, annex H and K. For the gPTP sync and Follow_Up messages, it is supported only by the PTP relay instance and not supported by the Boundary Clock or the Transparent Clock, so it needs explicitly to be described in clause 5.27.1.2.2.1. similar in clause 5.27.1.2.2.2 , it is explicitly described the PTP instance is Boundary Clock or Transparent Clock. The UPF/NW-TT uses the ingress port number and the PTP domain to identify and to match a PTP instance and discards the messages if no matching PTP instance is identified as described in 5.27.1.2.2.2. but in clause 5.27.1.3, it is described that the (g)PTP event messages of all domains are forwarded without any matching procedure. It is corrected only for the matching domains. In clause 5.27.1.3, the 5GS supports multiple (g)PTP domains, also the 5GS shall support multiple PTP instances. And more PTP instances can be supported for the same (g)PTP domain, and the PTP instance can be differented by the ingress port number with the same (g)PTP domain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The 5GS can be configured to operate in one or multiple PTP instances to operate in different mode or different types(i.e. PTP relay instance, Boundary Clock, end-to-end Transparent Clock, peer-to-peer Transparent Clock) for each PTP instance in clause 5.27.1.1 and 5.27.1.4. The time aware system is replaced with PTP relay instance in the clause 5.27 and its subclauses, annex H and K. For the gPTP sync and Follow_Up messages, it is supported only by the PTP relay instance in clause 5.27.1.2.2.1. In clause 5.27.1.2.2.2 , it is explicitly described the PTP instance is Boundary Clock or Transparent Clock. In clause 5.27.1.3, the (g)PTP event messages with the matching domains are forwarded. In clause 5.27.1.3, the 5GS supports multiple (g)PTP domains, also the 5GS shall support multiple PTP instances. And more PTP instances can be supported for the same (g)PTP domain, and the PTP instance can be differented by the ingress port number with the same (g)PTP domain. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PTP relay instance and multiple PTP instances can not be supported by the 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.1, 5.27.1.2.2.1, 5.27.1.2.2.2, 5.27.1.3, 5.27.1.4, 5.27.1.5, 5.27.1.6, 5.27.1.7, H.4, K.2.2.2, K.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE

### 5.27.1	Time Synchronization

#### 5.27.1.1	General

For supporting time synchronization service, the 5GS is configured to operate as one or multiple PTP instances and to operate with the following PTP instance type (if supported) configured for each PTP instance:

1)	as PTP relay instance as described in IEEE Std 802.1AS [104],

2)	as Boundary Clock as described in IEEE Std 1588 [126], provisioned by the profiles supported by this 3GPP specification including SMPTE Profile for Use of IEEE Std 1588 [126] Precision Time Protocol in Professional Broadcast Applications ST 2059-2:2015 [127];

NOTE 1:	Via proper configuration of the IEEE Std 1588 [126] data set members, the 5G internal system clock can become the time source for the PTP grandmaster function for the connected networks in the case of mode 1 and mode 2.

NOTE 2:	In some cases where the 5G internal system clock is the time source for the PTP grandmaster function for the connected networks, it might not be required for the UE to receive gPTP or PTP messages over user plane. The UE and DS-TT uses the 5G timing information and generates the necessary gPTP or PTP message for the end station if needed (this is implementation specific).

3)	as peer-to-peer Transparent Clock as described in IEEE Std 1588 [126], provisioned by the profiles supported by this 3GPP specification including SMPTE Profile for Use of IEEE Std 1588 Precision Time Protocol in Professional Broadcast Applications ST 2059-2:2015 [127]; or

4)	as end-to-end Transparent Clock as described in IEEE Std 1588 [126], provisioned by the profiles supported by this 3GPP specification including SMPTE Profile for Use of IEEE Std 1588 Precision Time Protocol in Professional Broadcast Applications ST 2059-2:2015 [127].

NOTE 3:	When the GM is external, the operation of 5GS as Boundary Clock assumes that profiles that are supported by the 5GS allows the exemption specified in clauses 9.5.9 and 9.5.10 of IEEE Std 1588 [126] where the originTimestamp (or preciseOriginTimestamp in case of two-step operation) is not required to be updated with the syncEventEgressTimestamp (and a Local PTP Clock locked to the external GM). As described in clause 5.27.1.2.2, only correctionField is updated with the5GS residence time and link delay, in a similar operation as specified by IEEE Std 802.1AS [104].

The configuration of the time synchronization service in 5GS for option 1 by TSN AF and CNC is described in clause 5.28.3, and for options 1-4 by AF and NEF or TSCTSF in clause 5.27.1.8 and clause 5.28.3.

The 5GS shall be modelled as an IEEE Std 802.1AS [104] or IEEE Std 1588 [126] compliant entity based on the above configuration.

The DS-TT and NW-TT at the edges of the 5G system may support the IEEE Std 802.1AS [104] or other IEEE Std 1588 [126] profiles' operations respective to the configured mode of operation. The UE, gNB, UPF, NW-TT and DS- TTs are synchronized with the 5G GM (i.e. the 5G internal system clock) which shall serve to keep these network elements synchronized. The TTs located at the edge of 5G system fulfil some functions related to IEEE Std 802.1AS [104] and may fulfil some functions related to IEEE Std 1588 [126], e.g. (g)PTP support and timestamping. Figure 5.27.1-1 illustrates the 5G and PTP grandmaster (GM) clock distribution model via 5GS.

Figure 5.27.1-1: 5G system is modelled as PTP instance for supporting time synchronization

Figure 5.27.1-1 depicts the two synchronizations systems considered: the 5GS synchronization and the (g)PTP domain synchronization.

-	5G Clock synchronization: Used for NG RAN synchronization and also distributed to the UE. 5G Clock synchronization over the radio interface towards the UE is specified in TS 38.331 [28].

-	(g)PTP domain synchronization: Provides synchronization service to (g)PTP network. This process follows IEEE Std 802.1AS [104] or IEEE Std 1588 [126].

The two synchronization processes can be considered independent from each other and the gNB only needs to be synchronized to the 5G GM clock.

The 5GS supports two methods for determining the grandmaster PTP Instance and the time-synchronization spanning tree.

-	Method a), BMCA procedure.

-	Method b), local configuration.

This is further described in clause 5.27.1.6.

END OF CHANGES