SA WG2 Meeting #148e	S2-2108857

November 15th – 22nd, 2021; Elbonia               	(revision of)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2895 | rev | 1 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | L-PSA and I-SMF release during mobiity procedure with I-SMF change or removal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At SA2 Meeting #146E , when discussing CR 2812, how to keep the same EAS for the traffic of applications that do not support mobility (as already described in Figure 4.3.5.7-1 for the non-ETSUN case) has not been considered and it was asked more time to think about this case |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Source I-SMF releases any resource associated with a PDU Session when both following events have taken place: - (if indirect data forwarding applies) the expiry of the data forwarding timer, - the completion of all the N4 release procedures initiated by the SMF to release the local UPF(s) the Source I-SMF is controlling. NOTE1: The SMF remains responsible of any traffic forwarding (involving source I-UPF(s)) related with keeping the same EAS for the traffic of applications that do not support mobility (see Figure 4.3.5.7-1) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | keeping the same EAS for the traffic of applications that do not support mobility (see Figure 4.3.5.7-1) is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.9.0 ; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 4.23.9.0	Overview

The procedures in this clause describe the Addition, Removal and Change of PDU Session Anchor (PSA2), Branching Point or UL CL controlled by I-SMF. They all rely on following principles:

1.	When a (new) I-SMF is inserted (e.g. as described in clause 4.23.7 or clause 4.23.11), the I-SMF provides the DNAI list it supports to the SMF. This list is assumed to remain constant during the N16a association between the I-SMF and the SMF for a PDU Session.

2.	Based on the DNAI list information received from I-SMF, the SMF may then at any time provide or update the list of DNAI(s) of interest for this PDU Session to I-SMF. This may take place e.g. when the I-SMF provides the DNAI list it supports or when new or updated or removed PCC rule(s) is/are received by the SMF as defined in clause 4.23.6. This list of DNAI(s) of interest for this PDU Session indicates to the I-SMF the list of DNAI(s) candidate for local traffic steering within the PDU Session.

An indication of whether Multi-homing is possible is also provided to the I-SMF, and the I-SMF uses this information to decide whether multi-homing can be used for the PDU Session.

3.	Whenever the I-SMF has inserted or removed or changed a local offload capability the I-SMF invokes a Nsmf_PDUSession_Update Request to indicate to the SMF the list of corresponding DNAI(s). Based on this indication the SMF invokes a Nsmf_PDUSession_Update Request to send the corresponding N4 information to the I-SMF.

4.	Then SMF may then at any time invoke a Nsmf_PDUSession_Update Request to send N4 information to the I-SMF.

5.	The I-SMF may at any time send a Nsmf_PDUSession_Update Request to forward to the SMF N4 events received from a local UPF; this may e.g. correspond to traffic reporting.

6.	When source I-SMF is to be removed from a PDU Session (e.g. at I-SMF change or removal), the SMF issues the N4 information targeting the UL CL/BP (s) and L-PSA(s) controlled by this I-SMF, including requests to release the corresponding N4 Sessions.

When the Source I-SMF receives a Nsmf_PDUSession_ReleaseSMContext Request from AMF, it initiates a data forwarding timer (if indirect data forwarding applies).When the Source I-SMF has received N4 release from SMF, it releases the corresponding UL CL/BP (s) and L-PSA(s) resources either directly if no data forwarding timer is started, or at the expiry of the data forwarding timer.

The Source I-SMF releases any resource associated with a PDU Session when both following events have taken place:

-	(if indirect data forwarding applies) the expiry of the data forwarding timer,

-	the completion of all the N4 release procedures initiated by the SMF to release the local UPF(s) the Source I-SMF is controlling.

NOTE1: 	The SMF remains responsible of ensuring traffic forwarding (involving source I-UPF(s) controlled by the I-SMF) related with keeping the same EAS for the traffic of applications that do not support mobility (see Figure 4.3.5.7-1)

When IPv6 multi-homing is used and an I-SMF is removed, the SMF re-configures the UE to not use the corresponding IP prefix @L-PSA(s).

# * * * * End of change * * * *