3GPP TSG-WG SA2 Meeting #148E 	S2-2108941

# Electronic, November 15 – 19, 2021											(revision of S2-2107102)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3237 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MUSIM capabilities in Emergency Registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MUSIM |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 23.502 clause states that the UE is not expected to perform SR with Release Request: NOTE 1: It is not expected that UE in MUSIM mode will execute UE triggered Service Request procedure with Release Request indication if regulatory prioritized services (e.g. emergency service, emergency callback waiting) are ongoing. This text was agreed before the CRs on MUSIM capability exchange. A more appropriate solution would be to mandate the UE to not indicate MUSIM capabilities when performing Emergency Registration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies that the UE shall not indicate MUSIM capabilities when performing Emergency Registration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current text is not normative, leaving the door open for UE implementations that indicate MUSIM capabilities in Emergency Registration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.38.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** FIRST CHANGE *****

## 5.38	Support for Multi-USIM UE

### 5.38.1	General

A network and a UE may support one or more of the following features for Multi-USIM UE operation:

-	Connection Release as described in clause 5.38.2;

-	Paging Cause Indication for Voice Service, as described in clause 5.38.3;

-	Reject Paging Request, as described in clause 5.38.4;

-	Paging Restriction, as described in clause 5.38.5.

In the Registration procedure (as specified in clause 4.2.2.2.2), when a Multi-USIM UE has more than one USIM active, supports and intends to use one or more Multi-USIM specific features, it indicates to the AMF the corresponding Multi-USIM feature(s) are supported. Based on the received indication of the supported Multi-USIM features from the UE, the AMF shall indicate to the UE the support of the Multi-USIM features based on the Multi-USIM features supported by network and any preference policy by the network, if available. When a UE returns to having only one USIM active from a Multi-USIM UE that previously indicated to the network it supported Multi-USIM feature(s), the UE shall indicate all the Multi-USIM features are not supported to the network for that USIM. The AMF shall only indicate the support of Paging Restriction feature together with the support of either Connection Release feature or Reject Paging Request feature.

The Multi-USIM UE includes the support of individual features for Connection Release, Paging Cause Indication for Voice Service, Reject Paging Request and Paging Restriction as specified in clause 5.4.4a.

The network shall not indicate support for any Multi-USIM feature as part of the Emergency Registration procedure.

A Multi-USIM UE shall use a separate PEI for each USIM when it registers to the network.

***** END OF CHANGES *****