SA WG2 Meeting #S2-148E	S2-2108955r02

15 November – 19 November 2021, Electronic, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0073 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the input of PCF/NEF/MB-SMF service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Nnef_ MBSSession related service operation and Nmbsmf_MBSSession ContextStatusSubscribe service operation, it should be MBS session ID, rather than multicast session ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace Multicast session ID with MBS session ID in pcf and NEF service opeartion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The input parameters for NEF/MB-SMF service operation is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.3, 9.4.3.5, 9.4.3.6, 9.4.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 9.1.3.3	Nmbsmf_ MBSSession_ContextStatusSubscribe service operation

Service operation name: Nmbsmf_ MBSSession_ContextStatusSubscribe

Description: Service Consumer NF can use this service operation to request information (e.g. QoS information) about a multicast session and to subscribe to notification of events about the multicast session context.

Inputs, Required: MBS Session ID, notification target address, Events ID(s).

Inputs, Optional: immediate reporting indication.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: If immediate reporting indication is set: Event information (e.g. QoS information for multicast session, multicast session status (activated, deactivated), multicast session service area for local multicast service), Start time of multicast session, if consumer is SMF: indication that the MBS session allows any UE to join.

Next CHANGE

#### 9.4.3.5	Nnef_ MBSSession_StatusNotify service operation

Service operation name: Nnef_ MBSSession_StatusNotify

Description: This service is used by the MB-SMF to notify the NF service consumers of the subscribed events.

Inputs, Required: Status Information, MBS Session ID.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

#### 9.4.3.6	Nnef_ MBSSession_StatusSubscribe service operation

Service operation name: Nnef_ MBSSession_StatusSubscribe

Description: This service is used by the NF service consumers to subscribe to the MBS Session status information.

Inputs, Required: Status Information, MBS Session ID.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

#### 9.4.3.7	Nnef_ MBSSession_StatusUnsubscribe service operation

Service operation name: Nnef_ MBSSession_StatusUnsubscribe

Description: This service is used by the NF service consumers to unsubscribe to the MBS Session status information.

Inputs, Required: Status Information, MBS Session ID.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

Next CHANGE

END OF CHANGES