SA WG2 Meeting #148E e-meeting	S2-2108961

# 15 – 22 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3375 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.501: TSCTSF Discovery and Selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell, Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Similar Reason for change as in S2-2107418 If more than one TSCTSF intances are deployed for a given DNN, S-NSSAI in the network and the TSCTSF is discovered via NRF, then the selected TSCTSF shall store the assocation of (TSCTSF NF ID, DNN, S-NSSAI) in the UDR. Nevertheless, this is only applicable when AF request is received before PDU Session establishment. It doesn’t work when AF request is received after PDU Session establishment or even when there is no AF request for the PDU Session. Furthermore, it states that such assocation is deleted after all the PDU Session(s) for the given DNN/S-NSSAI are released, however, TSCTSF has no concept of “PDU Session” and TSCTSF is still in use when there is active AF request targeting to the given DNN/S-NSSAI for time sync. or TSC QoS service. Notably, this assocation is used to select the same TSCTSF for a given DNN and S-NSSAI, it should be independent of the time sync. application data, i.e. even when there is no time sync. data stored in the UDR, this association should also work. Crux of the problem is that the same UDR cannot be discovered for a given DNN/S-NSSAI thus it is not always possible to ensure we have selected the right UDR to retrieve TSCTSF NF ID. As a matter of fact, only one of the TSCTSF instances can be used for the given DNN, S-NSSAI even multipleTSCTSF intances for a given DNN, S-NSSAI are deployed. Additionly, support of multipleTSCTSF intances for a given DNN, S-NSSAI brings extra network complexilities and impacts, so it is hard to see the benefits in practice. In order to make TSCTSF discovery and selection workable, it is proposed to simply specify that only one TSCTSF instance for a given DNN, S-NSSAI. This avoids the impacts to UDR/BSF services and the extra complexilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that a single TSCTSF instance is associated to a given DNN/S-NSSAI. - TSC TSF discovery section – to clarify that the GPSI, SUPI, Internal Group ID can also be used for discovery and selection. - Add GPSI – in case the NF consumer uses GPSI for TSC TSF discovery and selection. - Add SUPI or Internal Group ID – in case the NF consumer is only aware of the SUPI or Internal Group ID, it uses SUPI or Internal Group ID for TSC TSF discovery and selection. These parameters are in principle available already with NRF framework thus it should be straight forward to use them also for TSC TSF discovery selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to discover the right TSC TSF when certain UE IDs are available and TSCTSF discovery relies on UDR discovery but that is not discoverable when only DNN/S-NSSAI is available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

### 6.3.24	TSCTSF Discovery

The NF consumers  may utilize the NRF to discover TSCTSF instance(s) unless TSCTSF information is available by other means, e.g. locally configured in NF consumers.

The following factors may be considered for TSCTSF discovery and selection:

-	DNN and S-NSSAI: When the NF consumer or PCF discovers the TSCTSF for a DNN/S-NSSAI, the NRF provides NF profile(s) of TSCTSF instance(s) to the NF consumers belonging to single TSCTSF Set for a given DNN/S-NSSAI.

-	GPSI or External Group Identifier.

-	SUPI or Internal Group ID.

If the TSCTSF is locally configured in NF consumers, it shall be ensured that the same TSCTSF Set is configured in all NF consumers (e.g. NEF, AF) for the given DNN and S-NSSAI.

The same TSCTSF Set shall be configured also in the PCF serving PDU Sessions for this DNN and S-NSSAI to notify the TSCTSF for a PDU Session.

NOTE: Thus, it is assumed that here is only one TSCTSF Set for a given DNN/S-NSSAI in this release of the specification.

## ---End of Changes---