3GPP TSG-SA2 Meeting #148e 	S2-2108965

# Elbonia, Elbonia, Nov 15-19, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0074 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Broadcast Frequency selection Identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA2 received an LS from RAN2 requesting to introduce a new identifier for reducing the volume broadcasted in SIB (to be used instead of TMGIs). SA2 agreed a reply LS in S2-2108175 announcing the intention to introduce such an identifier |  |  |  |  |  |  |  |  |
| Summary of change: |  | A Broadcast frequency selection identifier (BFSID) is introdced. The BFSID is used for broadcast MBS session to announce the availability of MBS sessions in signalling between RAN nodes and the UE to guide the frequency selection of the UE. BFSIDs and their mapping to frequencies are provided to RAN nodes and made known to the MB-SMF or NEF via OAM. Based on this configuration, RAN nodes announce in SIBs over the radio interface information about the BFSIDs supported by cells and frequencies. When a broadcast session is created, BFSID(s) are selected for that broadcast session/TMGI by MB-SMF, NEF, or AF (assuming that the AF has related knowledge via business agreements). The BFSIDs assigned to a broadcast session are communicated in the service announcement towards the UE. The UE compares those BFSIDs with the BFSID(s) in SIBS for frequency selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN2 request is not satisfied leading to restrictions of the number of broadcast sessions that a RAN node can support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 6.5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.5.X	Broadcast frequency selection identifier

### editor´s note: The name Broadcast Frequency Selection Identifier is FFS.

The Broadcast frequency selection identifier (BFSID) is used for broadcast MBS session to announce the availability of MBS sessions in signalling between RAN nodes and the UE to guide the frequency selection of the UE.

BFSIDs and their mapping to frequencies are provided to RAN nodes and made known to the MB-SMF or NEF via OAM. Based on this configuration, RAN nodes announce in SIBs over the radio interface information about the BFSIDs supported by cells and frequencies.

When a broadcast session is created, BFSID(s) are selected for that broadcast session/TMGI by MB-SMF, or AF (assuming that the AF has related knowledge via business agreements).

NOTE 1:	One BFSID can be assigned to several TMGIs. One TMGI can be assigned to several BFSIDs. Depending on operator policy, BFSIDs can be used similar to 4G SIDs, or simply to group TMGIs, and also allow usage of diffrent frequencies for different broadcast services.

The BFSIDs assigned to a broadcast session are communicated in the service announcement towards the UE. The UE compares those BFSIDs with the BFSID(s) in SIBs for frequency selection.

NOTE 2:	A BFSID in a SIB does not guarantee that the broadcast service is available in a cell at the given time.

The BFSID(s) assigned to a broadcast session are signaled towards the RAN during broadcast session creation. The RAN selects a BFSID it is configured with out of the signalled ones to select a frequency for the broadcast session.

editor´s note: It is ffs whether to modify the encoding of MBS service areas by including the BFSIDs.