3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2109017r03

Elbonia, November 15 – 22, 2021	(revision of S2-210xxxx)

Title:	[DRAFT] Reply LS on Response LS on Positioning Reference Units (PRUs) for enhancing positioning performance

Response to:	LS (S2-2108999/R2-2111488) on Response LS on Positioning Reference Units (PRUs) for enhancing positioning performance

Release:	Release 17

Work Item:	5G_eLCS_Ph2 (SA2), NR_pos_enh (RAN)

Source:	[Huawei to be] SA2

To:	RAN2

Cc:	RAN1, RAN3

Contact Person:

Name:	Runze Zhou

Tel. Number:

E-mail Address:	zhourunze  AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

1. Overall Description:

SA2 thanks RAN2 for providing the progress on the PRU discussion in RAN WG (S2-2108999/R2-2111488). Regarding the questions listed in the LS:

RAN2 kindly asks SA2 whether the MT-LR or MO-LR location procedures as currently specified in TS 23.273 can be used to enable an LMF obtaining location measurements from PRUs (via LPP) and to trigger SRS transmission of PRUs (via NRPPa), or whether an LMF needs to be enabled to instigate location procedures for a PRU (e.g., LPP, NRPPa procedures) without receiving a location request for the PRU from an AMF (i.e., in the absence of an MT-LR or MO-LR for the PRU), and if so, whether support can be provided as part of Release 17.

SA2 would like to provide the following answers.

If a PRU is considered as a normal UE and has registered to the network, then a normal MO-LR, MT-LR, or NI-LR could be used to locate the PRU, where the LMF will initiate the positioning procedure, as defined in TS 23.273, clause 6.11. The procedure will trigger the UE to enter into CM-connected mode, i.e. page the UE if it is currently unreachable. However, the LMF may have no knowledge of which UEs act as PRUs and may therefore be unable to control and manage PRUs or make use of measurements provided by PRUs.

Furthermore, in release 17 and previous releases, SA2 has not specified a procedure for LMF to initiate a positioning procedure for a UE without an external trigger.

SA2 has finished R17 work on 5G_eLCS_Ph2, therefore it is not possible to study this topic in this release. SA2 may study support of PRUs in Release 18 in which case a better solution may be available later.

2. Actions:

To RAN2 group.

ACTION: 	SA2 asks RAN2 to take above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #149E		February 14 – 25, 2022			Elbonia