SA WG2 Meeting #S2-149E	S2-2200056

14 - 25 February, 2022, Electronic meeting

3GPP TSG-CT WG4 Meeting #107-e	C4-216390

# E-Meeting, 15th – 23rd November 2021			was C4-216342

# Title:	Reply LS on GTP-C cause value used for UAS services

# Response to:	C1-216293

# Release:	Rel-17

# Work Item:	ID_UAS

Source:	CT4

To:	CT1

Cc:	SA2

Contact Person:

Name:	Hao Jing

E-mail Address:	hao.jing@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: C4-216391

1. Overall Description:

CT4 would like to thank CT1 for the LS C1-216293. CT4 would like to provide the following response to the question:

CT1 would like to ask CT4 which GTP-C cause could be selected to ensure that the mapping Table C.1 in TS 29.274 enables the MME to use an existing NAS ESM cause for the above PDN connectivity reject handling and which existing NAS ESM cause value it will be mapped to.

CT4 proposes to use GTP-C cause Reject indication #92 "User authentication failed" from the SGW to the MME/S4-SGSN over S11/S4 and the MME could map it to NAS ESM cause #29 "User authentication failed" and send to the UE based on the mapping Table C.1 in TS 29.274. This would be an alignment with 5GS in which 5GSM cause #29 "User authentication or authorization failed" is used for UUAA-SM failure cases.

2. Actions:

To CT1 group.

ACTION: CT4 kindly asks CT1 group to take the CT4 decision into account.

3. Date of Next CT4 Meetings:

3GPP TSG CT4#107e-bis		01/2022	E-Meeting

3GPP TSG CT4#108e			02/2022	E-Meeting