SA WG2 Meeting #S2-149E	S2-2200058

14 - 25 February, 2022, Electronic meeting

3GPP TSG-CT WG4 Meeting #107-e	C4-216607

# E-Meeting, 15th – 23rd November 2021

Title:	LS on Deletion of "ME support of SOR-CMCI" indicator during Nudm_SDM_Get

Response to:

Release:	Rel-17

# Work Item:	     eCPSOR_CON

Source:	CT4

To:	CT1, SA2

cc:

Contact Person:	Varini Gupta

E-mail Address:	Varini.gupta@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

CT4 is carrying out work to support SOR-CMCI in CT4 specifications. CT4 has noticed following in 3GPP TS 23.122 v17.4.0, Annex C.2:

Step #2 & 3a state:

2)	Upon receiving REGISTRATION REQUEST message, the VPLMN AMF executes the registration procedure as defined in clause 4.2.2.2.2 of 3GPP TS 23.502 [63]. As part of the registration procedure:

…

…

then the VPLMN AMF invokes Nudm_SDM_Get service operation message to the HPLMN UDM to retrieve the steering of roaming information (see step 14b in clause 4.2.2.2.2 of 3GPP TS 23.502 [63]);

…

3a)	If the user subscription information indicates to send the steering of roaming information due to initial registration in a VPLMN, then the HPLMN UDM shall provide the steering of roaming information to the UE when the UE performs initial registration in a VPLMN, otherwise the HPLMN UDM may provide the steering of roaming information to the UE, based on operator policy. If the UE is performing initial registration or emergency registration, the HPLMN UDM shall delete the stored "ME support of SOR-CMCI" indicator, if any.

…

…

Question 1: CT4 would like to highlight that Nudm_SDM_Get is a read-only operation on UDM, and should not cause changes to server state. Hence, CT4 would like to request CT1 to explore if the deletion of "ME support of SOR-CMCI" indicator during initial/emergency registrations can be performed via some alternate procedures (e.g. before/after Nudm_SDM_Get) instead, or an alternative that does not require deletion of “ME support of CMCI” indicator upon specific registration types by the UE.

Question 2: CT4 has discussed the possibility of using any information provided in Nudm_UECM_Registration operation to be used for influencing the steering of roaming related information, however was not sure if Nudm_UECM_Registration operation always completes prior to Nudm_SDM_Get operation during UE's initial registration.

2. Actions:

To CT1

ACTION: 	CT4 kindly asks CT1 to provide guidance on Question 1 above.

To SA2

ACTION: 	CT4 kindly asks SA2 to provide response on Question 2 above as to whether Nudm_UECM_Registration operation always completes prior to Nudm_SDM_Get operation during UE's initial registration.

# 3. Dates of next CT4 meetings

The upcoming CT4 meetings can be found in the CT4 Meetings calendar