SA WG2 Meeting #149e	S2-2200101

# February 14th – 25th, 2022, Elbonia

Source: 	China Mobile

Title: 	Key Issue: support UE access EHE in a VPLMN in roaming scenario

Document for: 	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_ EDGE_Ph2 / Rel-18

Abstract of the contribution: Propose the key issue which is related with WT#1

1	Discussion

This paper is to propose a new key issue according to the WT#1 of FS_ EDGE_Ph2.

WT#1:Improvements to roaming, to support access to EHE in a VPLMN

2 Proposal

It is proposed to update TR 23.700-48 on FS_ EDGE_Ph2 as follows.

FIRST CHANGE

5.X 	Key Issue #X: Support UE access EHE in a VPLMN in roaming scenario

5.X.1	Description

Current specifications do not support the UE to access EAS via a Home Routed PDU Session. So, if a roaming UE wants to access EAS in EHE deployed in VPLMN, it has to establish a dedicated LBO PDU Session for the local traffic routing, and another HR PDU Session for other services. One or more dedicated DNN(s) have to be assigned to these applications in URSP rules determined by the HLPMN.

The following aspects should be studied to support UE access EHE in a VPLMN through a HR PDU session:

- How to authorize the HR PDU session with supporting local traffic routing to access EHE in VPLMN;

- Whether and How to support online charging when UE establishments an HR PDU session with supporting local traffic routing to access EHE in VPLMN;

- How to configure the VPLMN ECS address to UE in roaming scenario.

NOTE: In R17 specification, the ECS address is provided by UDM, so it is impossible for the H-UDM having the knowledge of ECS in a VPLMN. This may need some coordination with SA6.

5.X.2	Scenarios

A roaming UE may want to visit the VPLMN EHE but it can only support limited DNN(s) based on the subscription or operator policy. The specific scenario, for example a V2X roaming UE, the low latency communication between UE and V2X server is needed, but the V2X UE has limited DNN(s) and generally, the roaming UE should establish a HR PDU session between VPLMN and HPLMN. So it is needed to help HR PDU session enhancement to support local traffic routing.

End of CHANGE