SA WG2 Meeting #149e	S2-2200116

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	New solution for FS_GMEC WT3

Document for: 	Approval

Agenda Item: 	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract of the contribution: New solution for FS_GMEC WT3

# Discussion

The GMEC SID contains

Work Task 3: Whether additional mechanism or enhancement is needed and how to support group communication allowing UE to simultaneously send data to different groups, where each group has a different QoS policy (requirement regarding 5SEI as indicated in clause 6.13.2 of TS 22.261)

Note the only non 5MBS related requirements in clause 6.13.2 of TS 22.261 are

The 5G system shall allow a UE to request a communication service to simultaneously send data to different groups of UEs at the same time.

The 5G system shall allow different QoS policy for each group the UE communicates with.

# 2 Proposal

It is proposed to update TR 23.700-74 for FS_GMEC as follows

## 6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  |  | WT3 |  |
| Solution #X: reuse of R17 QoS framework |  |  | X |  |

NEXT CHANGE (2)

## 6.X	Solution #X: reuse of R17 QoS framework

### 6.X.1	Description

To support support group communication allowing UE to simultaneously send data to different groups, where each group has a different QoS policy, following R17 mechanisms may be used:

-	usage of a single PDU Session where PCC rules from the PCF can associate different Qos flows to the traffic of the UE within different groups (e.g. traffic to or from different sets of addresses for IP PDU session types, or using different VLAN(s) for Ethernet PDU session types)

-	usage of 5G VN group communications: where each group maps to a DNN (and slice) that may be associated with its own QoS policies.

NOTE:	Broadcast -multicast requirements in TS 22.261 clause 6.13.2 need to be studied in a 5MBS related study

### 6.X.2	Procedures

No changes to existing procedures are needed.

### 6.X.3	Impacts on Existing Nodes and Functionality

None