3GPP TSG-SA2 Meeting #149E (e-meeting)	S2-2200134

# Elbonia, Feb 14 - 25, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0061 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve EN for Mobility Restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-01-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following EN on Non-Allowed Area remains unresolved: Editor's note: It is FFS on mobility restriction when the 5G ProSe UE-to-Network Relay and 5G ProSe Remote UE locate in Non-allowed area. For Layer-3 Remote UE, Service Area Restriction is not applicable as there is not direct signalling connection between Layer-3 Remote UE and the network. For Layer-3 Relay, Layer-2 Relay or Layer-2 Remote UE, the same principles of Non-Allowed Area as defined in clause of TS 23.501 should be applied. For UE-to-Network Relay Discovery and Communication, the 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE shall follow the principles of a UE in limited-service state which is already defined for Forbidden Area. Core Network type restriction and Closed Access Group are not applied to 5G ProSe Remote UE or 5G ProSe UE-to-Network Relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following aspects are clarified: 1. Servcie Area Restriction is not applicable to 5G ProSe Layer-3 Remote UE. 2. In a Non-Allowed Area, the 5G ProSe Layer-2 Remote UE, 5G ProSe Layer-2 UE-to-Network Relay or 5G ProSe Layer-3 UE-to-Network Relay follows the same principles as specified in clause 5.3.4.1 of TS 23.501 [4] for communication with the network. 3. In a Non-Allowed Area, for UE-to-Network Relay Discovery and Communication, the 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE shall follow the principles of a UE in limited-service state. 4. Core Network type restriction is not applicable to 5G ProSe Remote UE or 5G ProSe UE-to-Network Relay. 5. Closed Access Group information is not applicable to 5G ProSe Remote UE or 5G ProSe UE-to-Network Relay. The EN is removed with the above clarifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN remains unresolved and lack of clarifications on mobility restrictions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** FIRST CHANGE ***************

5.4.3	Mobility Restrictions for 5G ProSe UE-to-Network Relay

The handling of Mobility Restrictions for 5G ProSe UE-to-Network Relay case follows the principles as specified in clause 5.3.4.1 of TS 23.501 [4] with the following additions:

-	Forbidden Area:

In a Forbidden Area, for UE-to-Network Relay Discovery and Communication, a 5G ProSe UE-to-Network Relay and a 5G ProSe Remote UE shall follow the principles of a UE in limited-service state as specified in clause 5.9.

-	Servcie Area Restriction:

Servcie Area Restriction is not applicable to 5G ProSe Layer-3 Remote UE.

-	Non-Allowed Area:

In a Non-Allowed Area, the 5G ProSe Layer-2 Remote UE, 5G ProSe Layer-2 UE-to-Network Relay or 5G ProSe Layer-3 UE-to-Network Relay follows the same principles as specified in clause 5.3.4.1 of TS 23.501 [4] for communication with the network. The 5G ProSe Layer-2 UE-to-Network Relay is not allowed to relay 5G ProSe Layer-2 Remote UE traffic.

In a Non-Allowed Area, for UE-to-Network Relay Discovery and Communication, the 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE shall follow the principles of a UE in limited-service state as specified in clause 5.9.

-	Core Network type restriction:

Core Network type restriction is not applicable to 5G ProSe Remote UE or 5G ProSe UE-to-Network Relay.

-	Closed Access Group information:

Closed Access Group information is not applicable to 5G ProSe Remote UE or 5G ProSe UE-to-Network Relay.

****************** END OF CHANGES ***************