3GPP TSG-SA2 Meeting # 149E (e-meeting)	S2-2200143

# Feb 14 – 25, 2022		  										                             (revision of S2-22xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3467 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the MUSIM Connection Release feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MUSIM |  |  |  |  |  | Date: |  |  | 2022-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In existing definition, when the Registration Request is sent due to mobility outside the Registration Area and before detecting whether the network supports the feature in the new Tracking Area, the UE assumes that the network has already indicated support for Connection Release feature in the current stored Registration Area. However, this description “provided that the network has already indicated support for Connection Release feature in the current stored Registration Area” conflicts with the description for emergency registration “The network shall not indicate support for any Multi-USIM feature to the UE as part of the Emergency Registration procedure.” Hence, this description “provided that the network has already indicated support for Connection Release feature in the current stored Registration Area” is only valid when the registration is not emergency registration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the Registration Request is sent due to mobility outside the Registration Area and before detecting whether the network supports the feature in the new Tracking Area, - If the UE registration is not emergency registration, the UE shall assume that the network has already indicated Connection Release feature is supported in the new Registration Area. - If the UE registration is emergency registration, the UE shall assume that the network has already indicated Connection Release feature is not supported in the new Registration Area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MUSIM Connection Release feature is not correct for emegency registration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.38.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** FIRST CHANGE *****

5.38	Support for Multi-USIM UE

5.38.2	Connection Release

A Multi-USIM UE may request the network to release the UE from RRC-CONNECTED state in 3GPP access for a USIM due to activity on another USIM in 3GPP access, if both UE and network indicate the Connection Release feature is supported to each other.

In the case of NAS connection release procedure, the UE indicates that it requests to be released from RRC-CONNECTED state, by initiating either a Service Request procedure over 3GPP access or a Registration procedure over 3GPP access (if case the UE needs to perform Registration Update at the same time with this network. When the Registration Request is sent due to mobility outside the Registration Area and the UE registration is not emergency registration, before explicitly detecting whether the network supports the feature in the new Tracking Area, the UE should treat the network has already indicated support for Connection Release feature in the current stored Registration Area) and may include a Release Request Indication in the Registration Request. When UE registration is emergency registration, before explicitly detecting whether the network supports the feature in the new Tracking Area, the UE shall treat the network has already indicated the Connection Release feature is not supported in the current Registration Area and shall not include a Release Request Indication in the Registration Request. If supported by the UE and network, the UE may also provide, only together with the Release Request Indication, Paging Restriction Information, as specified in clause 5.38.5, which requests the network to restrict paging.

For NR/5G access, an AS method for the UE to request the network to release the UE from RRC-CONNECTED state is specified in TS 38.300 [27]. This mechanism does not allow the UE to indicate Paging Restrictions.

NOTE 1:	When both the access stratum and NAS based approaches for requesting the connection release are supported by the UE and the network, it depends on the UE implementation which of the two to use (for example: based on the preferred end state (RRC-INACTIVE or IDLE) and whether Paging Restriction Information is to be provided).

NOTE 2:	When there is no PLMN-wide support for the Connection Release feature, it can occur that upon Mobility Registration Update with Release Request indication the UE is not released by the network. The UE behaviour, when it detects that the network does not support the feature in a new RA, is outside the scope of this specification.

***** END OF CHANGES *****