3GPP TSG-SA WG2 Meeting #149E	S2-2200247

# 14 - 25 February, 2022, Electronic meeting     						 (revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3475 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT |  |  |  |  |  | Date: |  |  | 2022-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the curret SA2 specification, there are some parts which are not algined with CT1 specification. From TS 23.122 (based on C1-217213) The indication of whether disaster roaming is enabled in the UE, the disaster roaming wait range and the disaster return wait range provisioned by the network are stored in the non-volatile memory of the ME, as specified in 3GPP TS 24.501 [64] annex C. In addition, the MS can also be pre-configured with an indication of whether disaster roaming is enabled in the UE, a disaster roaming wait range and a disaster return wait range stored in the USIM (see 3GPP TS 31.102 [40]). |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to clarify UE configuration related sentences based on CT1 specification.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with Stage 3 specification. Unclear specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.40.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.40.2	UE configuration and provisioning for Disaster Roaming

A UE supporting Disaster Roaming is configured with the following information:

-	Activation of Disaster Roaming;

-	Optionally, list of PLMN(s) to be used in Disaster Condition.

-	Optionally, Disaster Roaming wait range information and/or Disaster Return wait range information

-	Optionally, an indication 'applicability of "lists of PLMN(s) to be used in disaster condition" provided by a VPLMN'

The Activation of Disaster Roaming is performed by the HPLMN using the UE Parameters Update Procedure as defined in TS 23.502 [3]. The UE shall not perform disaster roaming if HPLMN has not configured the UE with Activation of Disaster Roaming. The indication on Activation of Disaster Roaming may be pre-configured in USIM.

The optional 'list of PLMN(s) to be used in Disaster Condition' may be provided by the HPLMN during and after a successful registration procedure over 3GPP access or non-3GPP access via Registration Request procedure or UE Configuration Update procedure as defined in TS 23.502 [3]. The 'list of PLMN(s) to be used in Disaster Condition' may be configured over non-3GPP access before disaster condition has occurred.

While roaming (i.e. not in the country of the HPLMN), the Registered PLMN may provide the 'list of PLMN(s) to be used in Disaster Condition' during and after a successful registration procedure to the UE via Registration Request procedure or UE Configuration Update procedure as specified in TS 23.502 [3]. This list shall not alter any list provided by the HPLMN.

The optional indication of 'applicability of "lists of PLMN(s) to be used in disaster condition" provided by a VPLMN', may be provided by the HPLMN.

The details of the UE behaviour regarding the usage of this list are described in TS 23.122 [17] and TS 24.501 [47]. If the UE is not configured with 'list of PLMN(s) to be used in Disaster Condition' follows the procedure described in TS 23.122 [17] to select PLMN to be used in Disaster Condition.

* * * * End of Changes * * * *