3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200257

Elbonia, February 14th – 25th, 2022

Source:	Huawei, HiSilicon

Title:	New key issue on the service continuity in case of Network Slice instance resource shortage

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: it is proposed to define a new key issue on the service continuity in case of Network Slice instance resource shortage.

1. Introduction

The WT#1 of FS_eNS_Ph3 proposes to study the service continuity in case of the Network Slice or the Network Slice instance cannot serve the PDU session, or if the existing Network Slice instance cannot meet the performance requirements of the applications. Therefore, there are two scenarios related to the service continuity mentioned in the WT#1.

- Scenario#1: the Network Slice cannot serve the PDU session;

- Scenario#2: the Network Slice instance cannot serve the PDU session or the Network Slice instance cannot meet the performance requirements of the applications

As discussed in the S2-2200258, Scenario#1 results from the misconfiguration. Only Scenario#2 needs to be considered for the WT#1. The existing mechanism on SMF Discovery and Selection, i.e. AMF querying the SMF within the same selected Network Slice instance from the NRF, is not flexible. It will cause the resource shortage for the selected Network Slice instance. It is proposed to focus on Scenario#2 and define new key issue on the service continuity in case of Network Slice instance resource shortage.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-41.

# * * * * First change * * * *

5.X	Key Issue #X: Service continuity in case of Network Slice instance resource shortage

5.X.1	General description

Network Slice instance is a set of Network Function instances and the required resources (e.g. compute, storage and networking resources) which forms a deployed Network Slice, as defined in the TS 23.501[x]. Based on the operator's operational or deployment needs, an S-NSSAI can be associated with one or more Network Slice instances. During the PDU Session Establishment for a given S-NSSAI, the NFs/services within the corresponding Network Slice instance shall be selected. According to the existing mechanism as specified in the TS 23.501[x] and TS 23.502[x], when the AMF is aware of the appropriate NRF to be used to select NFs/services within the corresponding Network Slice instance, the AMF always queries this NRF associated with for SMF selection for a given S-NSSAI. In addition, the AMF stores the selected NSI ID of the S-NSSAI for the PDU Session in the UE context so that this NSI ID can be reused for the SMF selection for other PDU Sessions for the same S-NSSAI. In other words, the AMF is restricted to discover the SMF within the same Network Slice instance for the same S-NSSAI during PDU session establishment procedure.

Therefore, it is possible that the overload or resource shortage for this selected Network Slice instance happens if the UE establishes multiple PDU Sessions for the same S-NSSAI, which will result in the selected Network Slice instance cannot serve the subsequent new PDU session or cannot meet the performance requirements of the applications.

This key issue will study how to enhance the existing mechanism to support service continuity in case of Network Slice instance resource shortage.

The following aspects shall be studied:

- How to allow the AMF to select an SMF that is in a suitable Network Slice instance, instead of restricting the selection of SMF within the same Network Slice instance for the same S-NSSAI.

- Whether and how to trigger the PDU Session re-establishment when the current Network Slice instance serving the existing PDU session is overloaded.

NOTE: The current mechanism on service continuity by re-establishing PDU Sessions (i.e. SSC mode#2 or SSC mode#3) is recommended to be reused as much as possible.

# * * * * Second change * * * *

Annex A:

A.X	Scenario of Network Slice instance resource shortage

Since R15, there are two options (early binding vs late binding) to select the Network Slice instance, in case of multiple Network Slice instances associated with the same S-NSSAI.

- Early binding: The NSSF returns the NRF(s) to be used to select NFs/services within the selected Network Slice instance(s), optional NSI ID(s) to be associated to the Network Slice instance(s) corresponding to certain S-NSSAIs. The AMF maintains the relationship between the S-NSSAI, NRF ID and (optional) NSI ID in the UE context. During PDU session establishment, the AMF queries the NRF to discover the SMF within the Network Slice instance for a given S-NSSAI. The related procedures are described in clause 4.2.2.2.3 and clause 4.3.2.2.3.2 of TS 23.502[y].

- Late binding: During PDU session establishment procedure, the NSSF determines and returns the appropriate NRF to be used to select NFs/services within the selected Network Slice instance queried by the AMF and optionally returns an NSI ID to be used to select NFs within the selected Network Slice instance to use for a given S-NSSAI. The related procedure is described in clause 4.3.2.2.3.2 of TS 23.502[y].

No matter whether early binding or late binding is used, when the AMF is aware of the appropriate NRF to be used to select NFs/services within the corresponding Network Slice instance, the AMF always queries this NRF associated with for SMF selection for a given S-NSSAI. In addition, the AMF stores the selected NSI ID of the S-NSSAI for the PDU Session in the UE context so that this NSI ID can be reused for the SMF selection for other PDU Sessions for the same S-NSSAI. In other words, the AMF is restricted to discover the SMF within the same Network Slice instance for the same S-NSSAI during PDU session establishment procedure.

Figure A.X-1: NF selection within same Network Slice instance for the same S-NSSAI

Therefore, the overload or resource shortage for this selected Network Slice instance may happens if the UE establishes multiple PDU Sessions for the same S-NSSAI., Then, the selected Network Slice instance cannot serve the subsequent new PDU session or cannot meet the performance requirements of the applications.

# * * * * End of changes * * * *