3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200259

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	New KI for WT#3 on VPLMN Selection

Document for:	Approval

Agenda Item:	8.19

Work Item / Release:	FS_eNS_ph3/ Rel-18

Abstract:  To add New KI for WT#3.1 on VPLMN Selection

1. Introduction/Discussion

As described in WT#4 in FS_eNS_ph3,

Study whether and how to support the following stage one Rel-18 EASNS requirements related to roaming specified in TS22.261 clause 6.1.2.1, i.e. Requirement on enhancement the information available to the UE in roaming scenarios regarding the availability of network slices in VPLMNs available in the roaming country, in order to allow the UE to select and obtain services from the VPLMN supporting the network slices which UE may wish to use.

The use case is in roaming scenario, the UE will select a VPLMN to access without considering the preferred slices as shown in figure 1. It should be noted that the UE may not have the Configured NSSAI for a VPLMN.

Figure 1

The UE prefers to use S-NSSAI 3 and S-NSSAI 4 during roaming scenario. But in the roaming country, the VPLMN has a higher priority for this UE, which only supports S-NSSAI1 and S-NSSAI2. If the UE has not stored the Configured NSSAI for VPLMN2, the UE will not know that VPLMN2 could support S-NSSAI-3 and S-NSSAI4. The UE has to access to VPLMN1 but ignores VPLMN2 (which is more sufficient) based on current mechanism.

As described in TS 22.261, For a roaming UE activating a service/application requiring a network slice not offered by the serving network but available in the area from other network(s), the HPLMN shall be able to provide the UE with prioritization information of the VPLMNs with which the UE may register for the network slice.

So, it is proposed to study how to select a VPLMN in roaming case in order to make sure the UE can request the sufficient S-NSSAIs.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41.

# * * * * First change * * * *all new

5	Key Issues

5.x	Key Issue #x: Assistance on VPLMN Selection based on S-NSSAI

5.1.1	Description

In roaming scenario, the UE may not be able to select the preferred S-NSSAI VPLMN to access with considering the preferred slices. For example, there could be no Configured NSSAI for the VPLMNs in a location. The UE will just select a VPLMN base on the SoR information from the HPLMN. The KI will address whether and how to enhance the current mechanism to assist the UE to select a VPLMN in roaming case in order that the UE can request the sufficient S-NSSAIs when the UE is not aware of the available S-NSSAIs of the VPLMNs in the area.

NOTE 1: The EASNS requirement on enhancement the information available to the UE in roaming scenarios regarding the availability of network slices in VPLMNs available in the roaming country is specified in TS 22.261 [x] clause 6.1.2.1. The VPLMN selection related information should be sent by the HPLMN.

NOTE 2: The detail of PLMN Selection should be discussed in CT1.

# * * * * Second change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[x]	3GPP TS 22.261:” Service requirements for the 5G system; Stage 1”.

# * * * * End of changes * * * *