3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200276

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	New KI on Detection of Improved Correctness of an Analytics (WT1.2)

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract: Propose a new key issue to study the detection of improved correctness of NWDAF Analytics related to WT1.2.

1. Introduction/Discussion

This contribution is related to WT#1.2:

WT#1.2:	Study possible mechanisms for improved correctness of NWDAF analytics.

The Network Data Analytics Function (NWDAF) provides various analytics that can be requested by several Network Functions (e.g., SMF, PCF, AMF) and AFs to support them on some of their decisions. The quality of NWDAF analytics can be affected by various factors e.g., the quality or amount of collected data to train the model, configuration of machine learning model etc. Consequently, the quality of an NWDAF machine learning model may affect the NF performance or status, according to the usage of an NWDAF analytics. A not reliable (or not well trained) analytics ID may not help a NF to make a precise or correct decision. Hence, it is important to detect and address when an analytics ID requires improvement.

NWDAF Training needs to become fast aware of problems in analytics IDs to support the update of an analytics ID output leading to unstable network status. It is proposed to study how to quickly detect a low performance Analytics ID and to trigger the process to improve NWDAF analytics (e.g., re-training).

In addition, the time to detect and update an analytics ID leading to unstable network status is a variable that depends on various factors. It can take seconds (e.g., if it is a model reselection) or it can take days (e.g., needing data collection with enough amount of data, or having human intervention). In that case one or more NFs may continue phasing the same performance issues, by continue consuming the specific analytics ID (that requires improvement). It is important to investigate whether and how to continue consuming analytics in a reliable manner, while the retraining process for an analytics ID has been triggered by NWDAF.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-81.

# * * * * First change * * * * All new

5	Key Issues

5.x	Key Issue # - Detection of Improved Correctness of an Analytics

5.x.1	Description

Various NFs use NWDAF analytics to support some of their decisions. A correct and high performance analytics ID (e.g., in term of accuracy) can also help the NFs to make more accurate decisions and thus improve their behaviour. For that reason, it is needed to monitor the performance of NWDAF analytics and to detect those analytics that improvement of their model is needed. In addition, it is important to understand if a NF can continue consuming a specific analytics ID that the need for improvement has been detected. This key issue includes the following aspects:

- Study how to detect an Analytics ID that correctness is needed and to trigger a process to improve NWDAF analytics (e.g., re-training).

- Study whether and how a NF can continue consuming an analytics ID that the need for improvement has been detected.

# * * * * End of changes * * * *