3GPP TSG-WG SA2 Meeting #149E 	S2-2200300

# Electronic Meeting, February 14–25, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3339 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying the 5G AS time distribution parameter delivery to NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2022-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 4.15.9.4 states that 5G Access Stratum time distribution parameters (for one UE, group of UEs, or any UE using the DNN and S-NSSAI) are delivered to NG-RAN, yet, it remains unspecified how and when these parameters are send to an NG-RAN node and which procedures/messages are used/affected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying in step 10 that the AMF provides the 5G AS time distribution parameters to an NG-RAN node during the NGAP procedures (e.g., UE Context Setup/Modification) specified in TS 38.413 [10]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading proceduring, which may lead to incorrect implementations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

#### 4.15.9.4	Procedures for management of 5G access stratum time distribution

The AF can use the procedure to activate, update or delete the 5G access stratum time distribution for one UE, group of UEs or any UE using the DNN and S-NSSAI.

The AF may query the status of the 5G access stratum time distribution using Nnef_TimeSynchronization_ASTIGet service operation. The Nnef_TimeSynchronization_ASTICreate and Nnef_TimeSynchronization_ASTIUpdate request may contain the parameters as described in Table 4.15.9.4-1.

Table 4.15.9.4-1: Description of 5G access stratum time distribution parameters

| Parameter | Description |
| --- | --- |
| 5G access stratum time distribution indication (enable, disable) | Indicates that the access stratum time distribution via Uu reference point should be activated or deactivated for the associated UE identities. |
| Time synchronization error budget | Indicates the time synchronization error budget for the time synchronization service (as described in clause 5.27.1.8 of TS 23.501 [2]). [optional] |
| Temporal Validity Condition | Indicates start-time and stop-time attributes that describe the time period when the time synchronization service is active. [optional] |

Figure 4.15.9.4: Management of 5G access stratum time information

1.	AM Policy Association establishment as described in clause 4.16.1.

2.	The PCF subscribes to policy data related to time synchronization (Data Set = Application Data; Data Subset = Time-Sync data, Data Key = S-NSSAI and DNN and/or Internal Group Identifier or SUPI).

NOTE 1:	The subscription by the PCF for a UE provides only the target UE(s) as data key (contrary to the subscription by the PCF for a PDU Session, which provides target UE(s), DNN, and S-NSSAI as data keys).

3.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	To create a new request, the AF provides access stratum time distribution parameters to the NEF using the Nnef_TimeSynchronization_ASTICreate service operation (together with the AF identifier and potentially further inputs as specified in table 4.15.9.4-1), including a target (one UE identified by SUPI or GPSI, a group of UEs identified by an External Group Identifier, or all UEs connected to a combination of DNN/S-NSSAI). The request may also contain a temporal validity condition. The NEF maps the External Group Identifier to an Internal Group Identifier and any GPSI to a SUPI.

-	To update or remove an existing request, the AF invokes an Nnef_TimeSynchronization_ASTIUpdate or Nnef_TimeSynchronization_ASTIDelete service operation providing the corresponding AF Transaction Id.

The AF that is part of operator's trust domain may invoke the services directly with TSCTSF.

NOTE 2:	Steps 1, 2, and 3 can occur in any order.

4.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_TimeSynchronization_ASTICreate service operation with the corresponding TSCTSF.

5.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	The TSCTSF determines whether the targeted UE is part of a PTP instance in 5GS, if so the TSCTSF rejects the request (steps 6-10 .are skipped). The TSCTSF associates the 5G access stratum time distribution parameters with a SUPI without DNN, S-NSSAI when storing them to the UDR.

(When the procedure is triggered by PTP instance activation, modification, or deactivation in the TSCTSF):

-	If time synchronization error budget is provided by the AF, the TSCTSF may use the PTP port state of each DS-TT to determine an Uu time synchronization error budget for corresponding SUPIs that are part of the PTP instance.

-	If time synchronization error budget is provided by the AF, the TSCTSF calculates the Uu time synchronization error budget as described in clause 5.27.1.9 of TS 23.501 [2]. If the TSCTSF receives multiple time synchronization error budgets for a given UE, then the TSCTSF picks the most stringent budget.

Then, TSCTSF stores the 5G access stratum time distribution parameters in the UDR (Data Set = Application Data; Data Subset = Time-Sync data, Data Key = Internal Group Identifier or SUPI). The Time-Sync data contains 5G access stratum time distribution indication (enable, disable) and optionally the calculated Uu time synchronization error budget.

6.	The UDR informs the TSCTSF about the result of the operation of step 5.

7.	The UDR notifies the PCF(s) that have a matching subscription (from step 2) about the data stored, updated, or removed in step 5. If matching entries already existed in the UDR when step 5 is performed, this shall be immediately reported to the PCF.

8.	If the 5G access stratum time distribution parameters in UDR are associated with a DNN/S-NSSAI for the SUPI (i.e. the parameters are valid only for a DNN/S-NSSAI), the PCF for the UE may discover the PCF for the PDU Session using Nbsf_Management_Subscribe with SUPI and (DNN, S-NSSAI) as parameters.

9.	Upon PDU Session establishment, if the PCF of the UE has subscribed for discovery of the PCF for the PDU Session from the BSF, the BSF provides to the PCF of the UE the identity of the PCF for the PDU Session and the UE address for the requested SUPI and (DNN, S-NSSAI) combination via an Nbsf_Management_Notify operation. If a matching entry already exists in the BSF when step 8 is performed, this shall be immediately reported to the PCF for the UE.

10.	The PCF takes a policy decision and then the PCF may initiate an AM Policy Association Modification procedure for the UE  as described in clause 4.16.2.2 to provide AMF the 5G access stratum time distribution parameters. As part of this, the AMF shall, if supported, send the 5G access stratum time distribution indication (enable, disable) and the Uu time synchronization error budget, when they are available, to an NG-RAN node using NGAP procedures (e.g., UE Context Setup/Modification) specified in TS 38.413 [10]. The NG-RAN node shall, if supported, store the information in the UE Context. Based on this information, the NG-RAN node provides the 5GS access stratum time to the UE according to the Uu time synchronization error budget as provided by the TSCTSF (if supported by UE and NG-RAN).

NOTE 3:	This release of the specification assumes that deployments ensure that the targeted UEs and the NG-RAN nodes serving those UEs support Rel-17 propagation delay compensation as defined in TS 38.300 [30].

11.	The TSCTSF responds the AF with the Ntsctsf_TimeSynchronization_ASTI operation response, including a reference to the time synchronization service configuration.

12.	The NEF informs the AF about the result of the Nnef_TimeSynchronization_ASTI operation performed in step 3.

>>>> End of Changes <<<<