SA WG2 Meeting #S2-149E	S2-2200408

# February 14th – 25th, 2022; Electronic meeting

# Title:	[DRAFT] Reply to LS on UE location during initial access in NTN

# Response to:	S2-2200079/R2-2201881

# Release:	Rel-17

# Work Item:	5GSAT_ARCH

Source:	Thales [to be changed to SA2]

To:	RAN2, SA3

Cc:	SA3-LI, RAN3, CT1

Contact Person:

Name:	Jean Yves Fine

E-mail Address:	jean-yves.fine@thalesgroup.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 would like to thank RAN2 for their new LS on UE location during initial access in NTN (S2-2200079/R2-2201881).

In this LS, RAN2 informs SA2 and RAN3 that RAN2 is discussing, after several liaisons, with SA2, SA3 and RAN3 (see R2-2200145/S2-2109337, R2-2200149/S3-214360, R2-2202542/S3i200056) whether or not, the UE may report to the NG-RAN its coarse GNSS coordinates during initial access (before AS security is activated).

RAN2 mentions the possibility that this location can be determined by the AMF by invoking UE location procedure (LCS) in connected mode (once AS security is activated) and provided to the NG-RAN.

RAN2 would then like to ask SA2/RAN3 if it's acceptable that no UE location information is reported at the NG-RAN in a NTN network during initial access.

Answer from SA2:

SA2 has agreed that under certain conditions, see clause §6.3.5 “AMF discovery and selection” in TS 23.501 here under:

“4)	When the UE attempts to establish a signalling connection, and the following conditions are met:

-	the 5G-AN knows in what country the UE is located; and

-	the 5G-AN is connected to AMFs serving different PLMNs of different countries; and

-	the UE provides a 5G-S-TMSI or GUAMI, which indicates an AMF serving a different country to where the UE is currently located; and

-	the 5G-AN is configured to enforce selection of the AMF based on the country the UE is currently located.

Then the 5G-AN shall select an AMF serving a PLMN corresponding to the UE's current location.”

Therefore SA2 assumed that 5G-AN has the knowledge of the UE location and has not considered the option for AMF to provide the UE location to NG-RAN in R17 nor plans to work on this option.

Consequently SA2 kindly request RAN2 to consider a solution enabling NG-RAN to “know in what country the UE is located” in order to select the appropriate core network.

For example, RAN2 could explore together with SA3 ways to address the possible privacy issues associated with the reporting UE location before AS security is activated.

2. Actions:

To RAN2 and SA3 groups.

ACTION: 	SA2 kindly asks RAN2 and SA3 to take these answers into account in their work.

3. Date of Next SA2 Meetings:

3GPP TSG SA2 #150 (E)	             6th – 12th Apr. 2022

3GPP TSG SA2 #151 (E)	16th – 20th May 2022