3GPP TSG-SA2 Meeting #149E(e-meeting)	S2-2200467

# Elbonia, Feburary 14 - 25, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0079 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing UE from Multicast MBS Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | #1 Clause 7.2.2.3 intends to specify the procedure of removing UE from the MBS session due to MBS session release or due to SMF decision without MBS Session release (see below): 1. When the MB-SMF decides to release an MBS Session: - based on a request from the AF (directly or via the NEF/MBSF); 2. When the SMF decides to remove a UE from an MBS session: - based on a request from the UDM (subscription change); or - due to local and location dependent MBS service is described in clause 7.2.4; or - due to network internal reasons. However, in step 3, “MBS session release” is indicated to the UE in the N1 SM container (aiming to cover the 1st scenario above), while in step 5, “Leave Request” is indicated to the UE (without clarifying the 1st scenario above). It is proposed to clarify the above two scenarios in the steps so that the UE can take actions accordingly (e.g. the UE should not re-attempt to join if the MBS Session is released). #2. In addition, for scenarios that SMF decides to remove a UE from the MBS session (without MBS session release), they are not reflected in Figure 7.2.2.3-1. #3. For MBS Session release, it needs to be clarified that SMF removes all joined UEs from the MBS Session one by one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #1. Align the description in step 3 and step 5 by indicating “UE removed from MBS session” in the N1 SM container with appropriate cause. #2. Add missing step in Figure 7.2.2.3-1 for SMF decision to remove UE from the MBS session without MBS session release. #3. Clarify that for MBS Session release, the SMF removed all joined UE from MBS Session one by one. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | #1. Unclear specification, and lack of appropriate cause information for UE to take proper action. #2. Incomplete procedure. #3. Unclear actions in SMF when MBS session is released. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

#### 7.2.2.3	Multicast session leave requested by the network or MBS session release

This procedure applies to the following scenarios:

1.	When the MB-SMF decides to release an MBS Session:

-	based on a request from the AF (directly or via the NEF/MBSF);

In this scenario, the MB-SMF notifies the SMF of multicast session release, and the SMF initiates procedures to remove all joined UEs from the MBS session.

2.	When the SMF decides to remove a UE from an MBS session:

-	based on a request from the UDM (subscription change); or

-	due to local and location dependent MBS service is described in clause 7.2.4; or

-	due to network internal reasons.

When the SMF receives the notification indicating multicast session release from the MB-SMF, the SMF initiates procedures to remove the joined UEs from the MBS session.

For the active MBS session, to release radio resources as early as possible, the MB-SMF may trigger Multicast Session Deactivation towards the NG-RAN as specified in steps 5-9 of clause 7.2.5.3, prior to or in parallel with triggering MBS Session Release to the SMF.

Figure 7.2.2.3-1: MBS Session Release or Multicast session leave requested by the network

1a.	For MB-SMF triggered MBS session release, the SMF receives Nmbsmf_MBSSession_ContextStatusNotify (MBS Session ID, multicast session release) from the MB-SMF with MBS Session ID. The SMF checks all joined UEs and perform step 2 to step 9 for each UE.

1b.	The SMF decides to remove a UE from the MBS session without MBS session release (e.g., due to UE moving out of MBS service area for local or location dependent MBS service as described in clause 7.2.4).

2.	For UEs without activated UP, the SMF may perform the same procedure as defined in step 3-7 in clause 7.2.5.2.

Alternatively, for UEs without activated UP, the SMF does not trigger message to the AMF, instead the SMF marks that the UE is to be informed of the MBS Session release. In this case, the SMF initiates PDU Session Modification to inform the UE of the MBS Session release at next UP activation of the associated PDU Session, if needed.

3.	For the joined UEs with UP activated, the SMF invokes Namf_Communicate_N1N2MessageTransfer to the AMF. The N1 SM container indicates UE removed from MBS session with appropriate cause (e.g., MBS session release, out of MBS service area, etc.). In N2 SM information, the SMF informs the NG-RAN to remove the UE from the MBS session. If there are associated QoS Flow(s) for individual delivery, the SMF also releases those QoS Flow(s) as specified in TS 23.502 [6] clause 4.3.3.2.

4.	The AMF sends N2 Request to the NG-RAN.

5.	The NG-RAN transports the N1 SM container (PDU Session Modification Command (MBS Session ID, UE removed from MBS session with appropriate cause)) to the UE.

6.	The NG-RAN performs radio resource modification. If there are no joined UEs in the MBS session, the NG-RAN releases the radio resources.

7.	If there are no joined UEs in the MBS session, for unicast transport of N3mb, the NG-RAN initiates the DL tunnel release towards MB-UPF via AMF and MB-SMF. For multicast transportation of N3mb, the NG-RAN performs IGMP/MLD Leave for the MBS session. See clause 7.2.2.4 for details.

8.	The NG-RAN sends N2 Response to the AMF. If there are no joined UEs in the MBS session, the MBS Session context is removed from the NG-RAN.

9.	The AMF transfers the N2 message received in step 8 to the SMF via the Nsmf_PDUSession_UpdateSMContext service operation. The SMF removes the UE from the MBS Session.

****************** END CHANGES ***************