3GPP TSG-SA2 Meeting # 149E	 S2-2200475

# Elbonia,  Februrary 14 - 25, 2022										 (Revision of )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3355 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Nnef_AMInfluence_Create |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_DCAMP |  |  |  |  |  | Date: |  |  | 2022-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | #1 SUPI should not be provided by the AF to NEF SUPI is 5GC internal info and should not be provided by the AF in Nnef_AMInfluence_Create in step 3a of clause 4.15.6.9.3 and in clause 5.2.6.23.2 (see below): 4.15.6.9.3 Processing AF requests to influence AM policies … 3a. To create a new request, the AF provides "AM influence information" data to the NEF using the Nnef_AMInfluence_Create service operation (together with the AF identifier and potentially further inputs as specified in clause 5.2.6.23.2), including a target (one UE identified by SUPI or GPSI, a group of UEs identified by an External Group Identifier, or all UEs, noting that the "all UEs" option is applicable only if an External Application Identifier is also provided) … 3b. The NEF stores, updates, or removes the policy data of step 3a in the UDR, having translated any External Group Identifier to an Internal Group Identifier and any GPSI to a SUPI. ##### 5.2.6.23.2 Nnef_AMInfluence_Create operation Service operation name: Nnef_AMInfluence_Create … Inputs, Optional: SUPI, GPSI, DNN, S-NSSAI, External Group Identifier, throughput requirements, service coverage requirements, policy duration, List of External Application Identifiers, subscribed event(s). As specified in step 3b of clause 4.15.6.9.3 (see above), the NEF may get SUPI translated from GPSI. #2 Parameter for “all UEs” is specified for Nnef_AMInfluence_Create service operation in step 3a of 4.15.6.9.3 but missing in 5.2.6.23.2. #3 The input parameter (GPSI, External Group Identifier) in 5.2.6.23.2 is to identify the target for AM policy influencing. It is proposed to group them in the same way as in step 3a of 4.15.6.9.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 4.15.6.9.3, remove SUPI from step 3a Clause 5.2.6.23.2, remove SUPI, and add “indication that any UE is targeted” for “all UEs” Editorial update to group the parameters related to UE IDs (as in step 3a of clause 4.15.6.9.3) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect/inconsistent specification, Security risk of allowing AF to provide SUPI to NEF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.9.3, 5.2.6.23.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

##### 4.15.6.9.3	Processing AF requests to influence AM policies

With this procedure, the AF can provide its AM Policy related request (for one or multiple UEs) at any time.

Figure 4.15.6.9.3: Handling an AF request to influence AM Policy

This procedure concerns only non-roaming scenarios, i.e. to cases where the involved entities serving the UE (i.e. AF, NEF, PCF, BSF, UDR, AMF) belong to the home PLMN, or the AF belongs to a third party with which the home PLMN has an agreement.

The PCF for the UE and the PCF for the PDU Session can be the same entity, then steps 6a, 6b, 7a, and 7c are not performed, while step 7b is performed by that entity.

1.	AM Policy Association establishment as described in clause 4.16.1.

2. The PCF for the UE may subscribe to policy data related to AM influence (Data Set = Application Data; Data Subset = AM influence information, Data Key = S-NSSAI and DNN and/or Internal Group Identifier or SUPI).

3a.	To create a new request, the AF provides "AM influence information" data to the NEF using the Nnef_AMInfluence_Create service operation (together with the AF identifier and potentially further inputs as specified in clause 5.2.6.23.2), including a target (one UE identified by GPSI, a group of UEs identified by an External Group Identifier, or all UEs, noting that the "all UEs" option is applicable only if an External Application Identifier is also provided), an optional indication of target application traffic (a list of (DNN, S-NSSAI)(s), and optionally a list of External Application Identifier(s)), and requirements related to AM policy (e.g. service coverage requirements, throughput requirements). The request contains also an AF Transaction Id and may contain a timer on how long this policy shall last, in which case the system behaviour upon expiration of this timer is as specified in TS 23.503. If with this request the AF subscribes to events related with access and mobility management policies, the AF indicates also where it desires to receive the corresponding notifications. To update or remove an existing request, the AF invokes an Nnef_AMInfluence_Update or Nnef_AMInfluence_Delete service operation providing the corresponding AF Transaction Id.

3b.	The NEF stores, updates, or removes the policy data of step 3a in the UDR, having translated any External Group Identifier to an Internal Group Identifier and any GPSI to a SUPI.

3c.	The UDR informs the NEF about the result of the operation of step 3b.

3d.	The NEF informs the AF about the result of the Nnef_AMInfluence operation performed in step 3a.

NOTE 1:	Steps 1, 2, and 3 can occur in any order.

4.	The UDR notifies the PCF(s) that have a matching subscription (from step 2) about the data stored, updated, or removed in step 3. If matching entries already existed in the UDR when step 3b is performed, this shall be immediately reported to the PCF. The PCF may check that an SM Policy association is established for the SUPI, DNN, S-NSSAI then subscribe to the SMF to Policy Control Request Trigger to detect the application traffic that triggers the allocation of a service area coverage or an allocation of RFSP index value, then steps 8 follows.

5.	Steps 2 to 11 in Figure 4.16.14.2-1 applies.

NOTE 2:	The PCF for the UE may subscribe to the "start/stop of application traffic detection" events for multiple applications with different application identifiers in the same Npcf_PolicyAuthorization_Subscribe request. When PCF receives the notifications for multiple applications, the PCF for the UE can determine which AM policy to apply based on local configuration and operator policy.

****************** NEXT CHANGE ***************

##### 5.2.6.23.2	Nnef_AMInfluence_Create operation

Service operation name: Nnef_AMInfluence_Create

Description: Authorize the request for AM policy influence and store the request data in the UDR, potentially translating GSPI to SUPI and External Group Identifier to Internal Group Identifier.

Inputs, Required: AF Transaction Id.

The AF Transaction Id refers to the request.

Inputs, Optional: DNN, S-NSSAI, Target (GPSI, or External Group Identifier(s) and optionally indication of all UEs), throughput requirements, service coverage requirements, policy duration, List of External Application Identifiers, subscribed event(s).

The subscribed event(s) includes Event ID(s) as specified in Nnef_AMInfluence_Notify service operation, Event Reporting Information defined in Table 4.15.1-1 (only the Event Reporting mode and the immediate reporting flag when applicable), Notification Target Address.

When a list of External Application Identifiers is provided, the throughput requirement, service coverage requirements, policy duration and subscribed event(s) are assigned the same value for all External Application Identifiers.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

****************** END OF CHANGE ***************