SA WG2 Meeting #S2-149E	S2-2200488

14 - 25 February, 2022, E-meeting

Source:	Ericsson

Title:	FS_eNPN_Ph2: Scope

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_ph2/Rel-18

Abstract of the contribution: This paper introduces the TR scope for the Study on enhanced support of Non-Public Networks; Phase 2.

# Discussion

The FS_eNPN_Ph2 SID in SP-211656 includes the following objectives

1.	Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection.

2.	Support for non-3GPP access for SNPN.

3.	Void

4.	Void

5.	Address new SA1 requirements (e.g. TS 22.261 requirements from PALS work) related to NPN e.g.:

a.	Support of enabling Localized Services via a local hosting NPN e.g. by configuration of the local NPN via interaction with localized service provider.

b.	Support for enabling UE to discover, select, access the local hosting NPN and the localized services via the hosting NPN with proper authorization.

-	Support for both UE configuration for discovery and selection, and user manual selection.

c.	Support for what, how and when services are accessed via a specific network e.g. via home network or via other network (e.g. local hosting NPN).

NOTE:	The TS 22.261 requirements for Providing Access to Local Services related to Multicast/Broadcast are not part of this objective.

The scope of the TR 23.700-08 is to address the above objectives. It is proposed to list the above objectives as part of the TR scope while for objective 5 it is proposed to limit it to support for Localized Services.

# Proposal

It is proposed to add the following scope to the TR 23.700-08 "Study on enhanced support of Non-Public Networks; Phase 2"

*** Start of changes ***

# 1	Scope

The scope of this Technical Report is to study further enhancements of the 5GS to support Non-Public Networks including the following aspects:

1.	Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection.

2.	Support for non-3GPP access for SNPN.

3.	Address SA1 requirements in TS 22.261 [2] related to support for Localized Services.

NOTE:	The TS 22.261 [2] requirements for Providing Access to Local Services related to Multicast/Broadcast are not part of the scope.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

*** End of changes ***