3GPP TSG-SA2 Meeting #149e	S2-2200526

Feb 14 – 25, 2022, Electronic meeting

Title:	[Draft] Reply LS on MBS Service Area Identity and start procedure for broadcast service

Response to:	LS on MBS Service Area Identity and start procedure for broadcast service (S2-2200415/R3-221302)

Release:	Release 17

Work Item:	5MBS

Source:	[CATT to be] SA2

To:	RAN3, RAN2

Cc:	SA4, SA5, SA6

Contact Person:

#### Name:	Xiaoyan Duan

Tel. Number:

E-mail Address:	duanxiaoyan@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TBD

1. Overall Description:

SA2 thanks RAN3 for their LS on MBS Service Area Identity and start procedure for broadcast service. SA2 discussed the LS and would like to provide the following feedback:

RAN3 has discussed the issue on MBS broadcast service continuity and identification, and would like to inform SA2 about our progress as follows: - To use the term “MBS Service Area ID” to replace the “MBS XXX ID” in the interim until SA2 comes up with final name; - It is configured via OAM at NG-RAN;

SA2 response

SA2 agrees to replace the "MBS XXX ID" with "MBS Service Area ID", same as the term used in RAN3. And SA2 agrees that MBS Service Area ID and their mapping to frequencies are provided to RAN nodes via OAM. Please see the attached CR.

Regarding to the length of MBMS service area ID, RAN3 did not reach an agreement and would like to wait for the conclusion in SA2.

SA2 response

SA2 suggests using 2 bytes for the MBMS service area ID, to align with MBMS SAI defined in 3GPP TS 23.002 and TS 36.331.

In the meanwhile, RAN3 also discussed on session start procedure for broadcast service and the view of majority is that indication of session start success or failure with cell accuracy is not needed. Instead, RAN3 is of the opinion that a success/failure on a per gNB basis may be sufficient and would like to know the views of SA2.

SA2 response

SA2 agree that indication of session start success or failure is on a per gNB basis. Please see the attached CR.

2. Actions:

To RAN WG3 group.

ACTION: SA2 kindly requests RAN3 to take the above information into consideration.

To RAN WG2 group.

ACTION: SA2 kindly requests RAN2 to confirm the length of MBMS service area ID suggested by SA2.

3. Date of Next TSG-SA WG2 Meetings:

TSG-SA2 Meeting #150E	April 6–12, 2022	E-meeting