3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200576

Elbonia, February 14th – 25th, 2022

Source:	vivo

Title:	Key Issue: 5GC architecture enhancements to support PIN

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this paper proposes a key issue on.

1. Introduction

According to the SP-211643, the study on Personal IoT Networks.

Following work tasks related to the study of architecture enhancements:

- To study the potential architectural enhancements for supporting management of PIN, access of PIN via PIN Element with Gateway Capability (PEGC), and communication of PIN (e.g. PIN Element communicates with other PIN Elements directly or via 5GS or via PEGC and 5GS). - To study the potential architecture enhancements for supporting identifying PIN and the PIN Elements in the PIN.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in 23.501 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] or in 23.501 [2].

PEGC	PIN Element with Gateway Capability

PEMC	PIN Element with Management Capability

PIN	Personal IoT Networks

PINE	PIN Element

# * * * * Second change * * * *

5	Key Issues

5.X	Key Issue #X: 5GC architecture enhancements to support PIN

5.X.1	Description

It is required that at least one PEGC is in a PIN, which is able to relay the traffic between 5GS and PINEs that are behind the PEGC. A PINE may be a non-3GPP device, or can be a UE. There’re one or more PEMCs for a PIN, which are able to control the PIN, e.g., create/delete a PIN, add/remove a PINE for the PIN, etc.

Considering the PIN feature can be deployed for wearable scenario, smart home scenario, and smart office scenario, the PEGC could be, e.g., smart phone, or, gateway for smart home.

The following aspects will be studied:

-	Whether additional function(s) and/or interface(s) are needed for supporting management of PIN, access of PIN via PEGC, communication of PIN, as well as identifying PIN and the PIN Elements in the PIN;

NOTE:	If new function(s) or new interface(s) are introduced in solution proposals addressed to other key issues, the architecture proposal needs to be addressed in this key issue, and those solutions needs to indicate the architecture proposal addressed to this key issue.

# * * * * End of changes * * * *