3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200583

Elbonia, February 14 – 25, 2022	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on who should check the user consent

Document for:	Discussion

Agenda Item:	8.1

Work Item / Release:	eNA_Ph2 / Rel-17

Abstract: This contribution proposes to discussion on who should check the user consent.

1. Introduction/Discussion

During the SA2#148E meeting, the S2-2109092 was approved that the DCCF (as a data collector) checks if user consent is granted or not for data collection.

However, based on the conclusion of SA3 in clause 8.2/8.3, TR 33.867, only the NWDAF (when only acted as a data consumer) will check the user consent from the UDM. Therefore, there is a misalignment between the SA2 and SA3.

8.2	Conclusion on KI #2: User consent for UE data collection

In the use case of UE related analytics in NWDAF, the following specific aspects are concluded for normative work:

-	The NWDAF determines whether to allow the NF service consumer to request for analysis using data subject to user consent based on user consent parameters.

-	The NWDAF retrieves user consent parameters in NWDAF's UE context or invokes UDM service to retrieve user consent parameters.

NOTE: 	This architecture only works when NWDAF and data provider are operated by the same entity.

8.3	Conclusion for Key Issue #3: Modification or revocation of user consent

The following specific aspects for use case#1 (NWDAF) are concluded for normative work:

-	NWDAF subscribes the service for notification of revocation of user consent if the NWDAF processes data subject to user consent, and is notified if user consent is changed.

-	If user consent is revoked, the NWDAF halts analysing and collecting of data subject to the user consent. Depending on circumstances/regulations outside the scope of 3GPP, the data may have to be deleted, or quarantined, or temporarily retained.

-	If user consent is revoked, the NWDAF notifies NF service consumers to halt processing of data subject to the user consent.

NOTE 1:	 This architecture only works when NWDAF and data provider are operated by the same entity.

2. Proposal

Proposal: It is proposed to approve the CR S2-2200584 to align the SA3’s conclusion i.e. only the NWDAF takes the responsibility of the user consent information checking.