3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200588

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	New KI: how to minimize UPF performance degradation due to UPF data reporting

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract: It proposes a new key issue of how to minimize UPF performance degradation due to UPF data reporting.

1. Introduction/Discussion

This contribution is related to WT#3.3, highlighted in cyan as follows:

WT#3.3: UPF data report to NWDAF to support UPF data report for analytics as specified in R16/R17 and additional UPF data identified in R18.

- NOTE: Coordination with FS_ UPEAS / Study on UPF enhancement for Exposure and SBA is needed.

The eNA R16 and R17 specifications already specified the support of various types and granularities (such as per QoS flow level and per NF) of UPF data collection per different Analytics ID. Table 1 summaries the type of UPF data that needs to be collected per Analytics ID, which are currently defined in eNA R16 and R17. It also provides the analytics about the granularity of the UPF data in the last column.

Table 1: Example for different type and granularity of UPF data per Analytics ID

| Analytics ID | UPF Data | Description | Granularity |
| --- | --- | --- | --- |
| Service Experience (6.4, TS 23.288) | QoS flow Bit Rate | The observed bit rate for UL/DL direction | Per QoS Flow |
|  | QoS flow Packet Delay | The observed Packet delay for UL/DL direction | Per QoS Flow |
|  | Packet transmission | The observed number of packet transmission. | Per QoS Flow |
|  | Packet retransmission | The observed number of packet retransmission. | Per QoS Flow |
| NF load (6.5, TS 23.288) | Aggregated UP Info | Report of user plane traffic in the UPF for the accumulated usage of network resources | Per NF |
| UE Communication (6.7, TS 23.288) | UE communication (1..max) |  |  |
|  | >Communication start | The time stamp that this communication starts | Per QoS Flow |
|  | >Communication stop | The time stamp that this communication stops | Per QoS Flow |
|  | >UL data rate | UL data rate of this communication | Per QoS Flow |
|  | >DL data rate | DL data rate of this communication | Per QoS Flow |
|  | >Traffic volume | Traffic volume of this communication | Per QoS Flow |

However, it is known that the UPF performance will be degraded if the UPF has to continuously report the required data per QoS flow to the NWDAF all the time, especially for some real-time and fresh data e.g., QoS flow Bit Rate and QoS flow Packet Delay and etc.

Therefore, the following aspect is needed to be studied for UPF data reporting:

-	How to minimize UPF performance degradation due to UPF data reporting

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-81.

# * * * * First change (all new text) * * * *

## 5.X	Key Issue #X: How to minimize UPF performance degradation due to UPF data reporting

### 5.X.1	General description

The eNA R16 and R17 specifications already specified the support of various types and granularities (such as per QoS flow level and per NF) of UPF data collection per different Analytics ID.

However, it is known that the UPF performance will be degraded if the UPF has to continuously report the required data per QoS flow to the NWDAF all the time, especially for some real-time and fresh data e.g., QoS flow Bit Rate and QoS flow Packet Delay and etc.

Therefore, the following aspect is needed to be studied for UPF data reporting:

-	How to minimize UPF performance degradation due to UPF data reporting.

# * * * * End of changes * * * *